Micro Simulator Clean Build Setup Guide

nRF9151 DK + Onomondo SoftSIM + Serial-Controlled TCP Simulator

Version: Validated clean-build notes for the working v8 simulator package

Audience: Developer with an existing Onomondo SoftSIM workspace and nRF Connect SDK environment.

# 1. Outcome

At the end of this procedure, the Micro simulator firmware is built and flashed onto the nRF9151 DK. The board uses the already configured Onomondo SoftSIM stack, attaches to LTE-M, exposes a serial command prompt, and can send simulated Micro heartbeat/location packets to the TCP server.

# 2. Assumptions

The Onomondo SoftSIM workspace already exists at C:\Users\INSERTUSER\onomondo-softsim-test.

The Onomondo SoftSIM module exists at modules\lib\onomondo-softsim.

The SoftSIM CLI/provisioning folder exists at C:\ss. Do not delete it and do not commit it to Git.

The board target is nrf9151dk/nrf9151/ns.

The developer can run west from an nRF Connect SDK terminal or equivalent PowerShell environment.

The simulator code is received from GitHub, a ZIP package, or an emailed code package. This guide does not require redoing the Onomondo SoftSIM setup.

# 3. Clean-slate rule

For a clean application rebuild, delete only the simulator application folder and the short build directory. Do not delete the SDK workspace, SoftSIM module, west metadata, or provisioning folder.

| Allowed to delete/recreate | Do not delete |
| --- | --- |
| C:\Users\INSERTUSER\onomondo-softsim-test\applications\micro_simulator | C:\Users\INSERTUSER\onomondo-softsim-test\.west |
| C:\b\micro_sim_clean | C:\Users\INSERTUSER\onomondo-softsim-test\nrf |
|  | C:\Users\INSERTUSER\onomondo-softsim-test\zephyr |
|  | C:\Users\INSERTUSER\onomondo-softsim-test\modules |
|  | C:\ss |

# 4. Required folder layout

C:\Users\INSERTUSER\onomondo-softsim-test\applications\micro_simulator
|   CMakeLists.txt
|   prj.conf
|   sysbuild.conf
|
+---src
|       main.c
|
+---docs
        build_setup_guide.md
        serial_commands.md

# 5. Required project files

The current working package contains four required application files. The three small configuration files are shown below. The source file src\main.c should be copied from the validated repository/package, not retyped and not replaced by documentation text.

## 5.1 CMakeLists.txt

cmake_minimum_required(VERSION 3.20.0)

# Include the Onomondo SoftSIM overlay from the workspace.
# This path assumes the app lives at:
#   <workspace>/applications/micro_simulator
# and the Onomondo SoftSIM module lives at:
#   <workspace>/modules/lib/onomondo-softsim
list(APPEND OVERLAY_CONFIG
    "${CMAKE_CURRENT_LIST_DIR}/../../modules/lib/onomondo-softsim/overlay-softsim.conf"
)

find_package(Zephyr REQUIRED HINTS $ENV{ZEPHYR_BASE})

project(micro_simulator)

target_sources(app PRIVATE
    src/main.c
)

## 5.2 prj.conf

# Micro simulator application configuration for nRF9151 DK + Onomondo SoftSIM

# Logging and console
CONFIG_LOG=y
CONFIG_LOG_DEFAULT_LEVEL=2
CONFIG_PRINTK=y
CONFIG_SERIAL=y
CONFIG_UART_CONSOLE=y
CONFIG_CONSOLE=y

# Stack/heap
CONFIG_MAIN_STACK_SIZE=8192
CONFIG_HEAP_MEM_POOL_SIZE=30000

# C library support
CONFIG_NEWLIB_LIBC=y

# Nordic modem and LTE link control
CONFIG_NRF_MODEM_LIB=y
CONFIG_LTE_LINK_CONTROL=y
CONFIG_LTE_NETWORK_MODE_LTE_M=y

# Networking and sockets through the nRF modem offload path
CONFIG_NETWORKING=y
CONFIG_NET_NATIVE=n
CONFIG_NET_SOCKETS=y
CONFIG_NET_SOCKETS_OFFLOAD=y
CONFIG_NET_IPV4=y

# Keep image size reasonable for nRF91 + TF-M builds
CONFIG_DEBUG=n
CONFIG_REBOOT=y

# Required for UART IRQ callback input used by SoftSIM provisioning and CLI
CONFIG_UART_INTERRUPT_DRIVEN=y

# Keep SoftSIM/UICC internals quiet. The simulator still logs its own INFO messages.
# If deep SoftSIM debugging is needed later, temporarily set these to y.
CONFIG_SOFTSIM_NRF_DEBUG_LOGS=n
CONFIG_SOFTSIM_LIBS_DEBUG_LOGS=n
CONFIG_LOG_BUFFER_SIZE=4096

## 5.3 sysbuild.conf

# Bundle the Onomondo SoftSIM template flash data with the sysbuild image.
SB_CONFIG_SOFTSIM_BUNDLE_TEMPLATE_HEX=y

## 5.4 main.c sanity requirements

src\main.c must start with C source code, normally a C comment followed by #include lines.

src\main.c must not start with Markdown such as # Micro Simulator, ## Purpose, or ```text.

The current working source uses UART interrupt-driven input for reliable SoftSIM profile paste and CLI input.

The source uses Zephyr socket APIs such as zsock_socket, zsock_connect, zsock_send, zsock_recv, and zsock_poll.

The source intentionally does not use LTE_LC_EVT_PSM_UPDATE or evt->psm_cfg for SDK-version compatibility.

Get-Content .\applications\micro_simulator\src\main.c -TotalCount 10
Select-String -Path .\applications\micro_simulator\src\main.c -Pattern "zsock_socket","K_MSGQ_DEFINE","LTE_LC_EVT_PSM_UPDATE","psm_cfg"

# 6. Get the code from GitHub or email

Download the repository ZIP, GitHub release ZIP, or emailed ZIP package.

Extract it outside the Onomondo workspace, preferably to C:\tmp\micro_simulator_clean_build.

Additionally, copy the application folder in the zip file to C:\Users\INSERTUSER\onomondo-softsim-test

Do not extract the package directly into C:\Users\INSERTUSER\onomondo-softsim-test.

If Windows blocks scripts from a downloaded ZIP, run Unblock-File on the setup script or use PowerShell execution-policy bypass for that command only.

mkdir C:\tmp
# Extract the ZIP so that this exists:
C:\tmp\micro_simulator_clean_build\micro_simulator_clean_build\scripts\create_clean_micro_simulator.ps1

# 7. Setup option A: use the package script

The package script deletes only the simulator app folder and build folder, then copies the clean application into the existing Onomondo SoftSIM workspace.

cd C:\tmp\micro_simulator_clean_build\micro_simulator_clean_build\scripts
powershell -ExecutionPolicy Bypass -File .\create_clean_micro_simulator.ps1

# 8. Setup option B: manual copy

cd C:\Users\INSERTUSER\onomondo-softsim-test
Remove-Item Env:ZEPHYR_BASE -ErrorAction SilentlyContinue
Remove-Item -Recurse -Force .\applications\micro_simulator -ErrorAction SilentlyContinue
Remove-Item -Recurse -Force C:\b\micro_sim_clean -ErrorAction SilentlyContinue
New-Item -ItemType Directory -Force .\applications\micro_simulator\src
New-Item -ItemType Directory -Force .\applications\micro_simulator\docs

# Then copy these files from the downloaded package:
# CMakeLists.txt  -> applications\micro_simulator\CMakeLists.txt
# prj.conf        -> applications\micro_simulator\prj.conf
# sysbuild.conf   -> applications\micro_simulator\sysbuild.conf
# main.c          -> applications\micro_simulator\src\main.c
# docs\*          -> applications\micro_simulator\docs directory

# 9. Verify before building

cd C:\Users\INSERTUSER\onomondo-softsim-test
Get-Content .\applications\micro_simulator\src\main.c -TotalCount 10
Select-String -Path .\applications\micro_simulator\prj.conf -Pattern "CONFIG_REBOOT","CONFIG_UART_INTERRUPT_DRIVEN","CONFIG_SOFTSIM_LIBS_DEBUG_LOGS"

Expected checks: main.c starts as C code; CONFIG_REBOOT=y is present; CONFIG_UART_INTERRUPT_DRIVEN=y is present; CONFIG_SOFTSIM_LIBS_DEBUG_LOGS=n is present.

# 10. Build and flash

Use a short build path to avoid Windows path-length problems.

cd C:\Users\INSERTUSER\onomondo-softsim-test
Remove-Item Env:ZEPHYR_BASE -ErrorAction SilentlyContinue
west build --sysbuild -b nrf9151dk/nrf9151/ns -s applications\micro_simulator -d C:\b\micro_sim_clean --pristine=always
west flash -d C:\b\micro_sim_clean

# 11. First boot after flash

If the board does not see a provisioned profile, it asks for the SoftSIM HEX profile. Use the generated profile from C:\ss, not the API key and not the encrypted profile file.

cd C:\ss
$profile = (Get-Content .\profile_001.hex -Raw).Trim()
"Profile length: $($profile.Length)"
Set-Clipboard -Value $profile

Paste the profile into the nRF serial terminal. The current simulator ignores whitespace and line breaks and automatically provisions after paste activity stops. After successful provisioning, the board reboots and attaches to LTE. If this does not occur after 10 seconds, reset the board.

Micro serial simulator started
Transfer SoftSIM profile using serial COM port.
Profile received: 258 hex characters in total
SoftSIM provisioned

# After reboot:
Micro serial simulator started
Waiting for LTE connect event
Network registration status: Connected - roaming
LTE connected
Serial CLI ready. Type: help
Recommended first command: connect 137.184.163.176 5000

# 12. Troubleshooting

| Symptom | Likely cause | Fix |
| --- | --- | --- |
| invalid preprocessing directive #Micro / stray ## / stray backticks | Markdown documentation was copied into src\main.c. | Replace src\main.c with the C source file from the package. Keep docs in docs\. |
| undefined reference to sys_reboot | CONFIG_REBOOT is missing. | Add CONFIG_REBOOT=y to prj.conf and rebuild pristine. |
| Failed to set UART callback: -134 | UART interrupt-driven API is not enabled. | Add CONFIG_UART_INTERRUPT_DRIVEN=y to prj.conf. |
| LTE_LC_EVT_PSM_UPDATE undeclared | SDK version does not define that event. | Use the current source, which does not reference that event. |
| Huge softsim_uicc logs and messages dropped | SoftSIM internal debug logs are too verbose for UART. | Keep CONFIG_SOFTSIM_NRF_DEBUG_LOGS=n and CONFIG_SOFTSIM_LIBS_DEBUG_LOGS=n. |
| Profile received is much shorter than the PowerShell profile length | Serial paste was truncated or wrong text was pasted. | Copy profile_001.hex with Set-Clipboard and paste once. Wait for automatic provisioning. |
| Build path failures on Windows | Build path too long. | Use -d C:\b\micro_sim_clean. |

# 13. What not to include in Git or shared docs

Onomondo SoftSIM API keys.

RSA private key files, especially C:\ss\softsim_key.

Generated SoftSIM HEX profiles such as profile_001.hex.

DigitalOcean root passwords or private server credentials.