SoftSIM, Simulator Build, and Windows Relay Setup Guide

nRF9151 DK + Onomondo SoftSIM + Direct LTE or Windows Wi-Fi Relay | Version 5

OUTCOME: A developer starting from a new Windows computer can install the required Nordic tools, prepare the source workspace, build and flash the dual-transport simulator, and then use either direct LTE or a Windows Wi-Fi relay. SoftSIM account/profile generation is optional unless direct cellular operation is required.

Project standard: nRF Connect SDK 3.4.x with the matching 3.4.x toolchain, installed through the nRF Connect extension in Visual Studio Code.

# How to use this guide

Read Section 1 first and choose the path that matches the board and test location.

Steps marked REQUIRED FOR EVERYONE must be completed unless the exact item is already installed or already exists.

Steps marked REQUIRED FOR CELLULAR are skipped for Windows Wi-Fi relay-only testing.

Steps marked OPTIONAL are only needed in the stated situation.

A board can be flashed and used in relay mode without a provisioned SoftSIM profile.

# 1. Choose the correct setup path

| Situation | Required path | Steps to skip |
| --- | --- | --- |
| Windows Wi-Fi relay only; no LTE-M coverage | Install Nordic tools, create the Onomondo source workspace, install/build/flash the simulator, and configure the Windows relay. | Skip the C:\ss folder, SoftSIM CLI, RSA/API-key work, HEX-profile generation, profile provisioning, and LTE connection steps. |
| Board already has a provisioned SoftSIM profile | Install/build/flash the simulator. Use direct LTE where LTE-M is available, or choose relay mode as a backup. | Skip all profile generation and profile provisioning steps. |
| Cellular required; Amir provides the board-specific HEX profile (Email me!) | Install/build/flash the simulator, provision the supplied profile, then connect to LTE. | Skip C:\ss, SoftSIM CLI download, RSA key generation, API-key creation, profile fetch, and profile conversion. |
| Cellular required; developer must generate a new profile | Complete every section, including the optional C:\ss/SoftSIM CLI workflow. | No cellular-related sections are skipped. |

# 2. Terminal glossary

| Name used in this guide | How to open it | Used for |
| --- | --- | --- |
| nRF Connect SDK 3.4 terminal | VS Code -> nRF Connect extension -> Welcome -> select SDK/toolchain 3.4.x -> Open Terminal. | west, git, building, and flashing. This terminal has the Nordic environment. |
| nRF Device Connections | VS Code -> nRF Connect extension -> Connected Devices -> expand nRF9151 DK -> open the serial terminal that shows board logs. | Board output, simulator commands, and SoftSIM profile paste. Close it before the Python relay uses the same COM port. |
| Standard Windows PowerShell | Windows Start menu -> PowerShell. Do not use the Nordic SDK terminal for the Python relay environment. | Package installation, relay virtual environment, serial-port listing, and relay launch. |
| Python relay console | The PowerShell window after micro_serial_wifi_relay.py starts. | Acts as the board terminal and forwards packets/responses over the laptop Wi-Fi connection. |
| Remote Ubuntu shell | ssh root@<server-ip> from PowerShell. | TCP server logs, service status, and server code changes. |

# 3. Install the required Windows and Nordic software

REQUIRED FOR EVERYONE! Complete this section unless the exact software is already installed and working.

1. Install Visual Studio Code.

2. In VS Code, open Extensions and install the nRF Connect for VS Code Extension Pack.

3. Open the nRF Connect view and install nRF Connect SDK 3.4 and the matching 3.4 toolchain through the extension.

4. Connect the nRF9151 DK over USB, power it on, and confirm that it appears under Connected Devices.

5. If the board is not detected, install or update the SEGGER/J-Link software before continuing.

VERSION RULE: Use the SDK 3.4 terminal for the commands in Sections 4, 6, and 7. Do not mix another SDK version with the 3.4 toolchain.

# 4. Create or verify the Onomondo west workspace

REQUIRED FOR EVERYONE! The dual-transport firmware still compiles against the Onomondo SoftSIM source module, even if no profile is provisioned and only relay mode will be used.

Run the following commands in the nRF Connect SDK 3.4 terminal. The workspace path automatically uses the current Windows user account.

$Workspace = Join-Path $env:USERPROFILE "onomondo-softsim-test" New-Item -ItemType Directory -Force $Workspace ¦ Out-Null Set-Location $Workspace Remove-Item Env:ZEPHYR_BASE -ErrorAction SilentlyContinue west init -m https://github.com/onomondo/nrf-softsim.git git -C modules/lib/onomondo-softsim submodule update --init west update

SKIP RULE: If $Workspace already contains .west, nrf, zephyr, and modules\lib\onomondo-softsim, do not run west init again. Instead run the verification commands below.

$Workspace = Join-Path $env:USERPROFILE "onomondo-softsim-test" Set-Location $Workspace Remove-Item Env:ZEPHYR_BASE -ErrorAction SilentlyContinue Test-Path .west Test-Path modules\lib\onomondo-softsim git -C modules/lib/onomondo-softsim submodule update --init west update

# 5. Decide how the SoftSIM profile will be obtained

OPTIONAL FOR RELAY-ONLY USERS: Skip all of Section 5 when the device will connect through the Windows laptop and Wi-Fi only.

REQUIRED FOR DIRECT CELLULAR: A unique SoftSIM HEX profile must be provisioned before the nRF9151 can attach through Onomondo. Use only one profile per physical board.

## 5.1 Amir provides the HEX profile

Email Amir for a board-specific SoftSIM HEX profile through a private channel. Team members sometimes call this a HEX key, but it is technically a SoftSIM profile. Save this for later.

Do not reuse the same profile on another board.

Do not put the profile in Git, a shared chat, a ticket, or general documentation.

Save the file in any private local folder. C:\ss is not required when Amir provides the profile.

## 5.2 Otherwise, Generate a new profile using the Onomondo CLI

OPTIONAL: Complete this subsection only when cellular is required and no valid board-specific profile is available from Amir or from an already-provisioned board.

The separate C:\ss folder, SoftSIM CLI, RSA key pair, Onomondo API key, and profile-generation commands are not required for relay-only use.

Onomondo SoftSIM CLI is the PC-side tool used to fetch encrypted profiles from Onomondo and convert one profile into the HEX format accepted by the nRF91 SoftSIM sample

### 5.2.1 Prepare C:\ss and the CLI

Open the Onomondo SoftSIM CLI GitHub page: https://github.com/onomondo/onomondo-softsim-cli

On the right side of the GitHub page, find Releases.

Open the latest release.

Download the release asset for your operating system. On Windows, download the Windows binary or archive.

Extract the downloaded archive.

Find softsim.exe inside the extracted files.

Move softsim.exe into C:\ss, so the final path is C:\ss\softsim.exe.

- Standard Windows PowerShell or NRF Connect SDK Terminal

Set-Location C:\ss Unblock-File .\softsim.exe -ErrorAction SilentlyContinue .\softsim.exe --help

### 5.2.2 Generate the RSA key pair and API key

Standard Windows PowerShell or NRF Connect SDK Terminal

Set-Location C:\ss ssh-keygen -t rsa -m PEM -b 4096 -f .\softsim_key ssh-keygen -e -m PKCS8 -f .\softsim_key.pub ¦ Set-Content -Encoding ascii .\softsim_key_public.pem Get-Content .\softsim_key_public.pem -Raw ¦ Set-Clipboard

Press Enter twice when ssh-keygen asks for a passphrase. The CLI does not support password-protected keys. Fater running the command the key is copied to your clipboard.

Log into the Onomondo platform website.

Go to the SoftSIM API key creation page. The CLI guide points to https://app.onomondo.com/api-keys/softsim/new.

Paste the public key from your clipboard.

Create the SoftSIM API key.

Copy the generated API key temporarily. You will use it in the next step.

The private key C:\ss\softsim_key must remain private.

### 5.2.3 Fetch and convert one profile

Run in standard Windows PowerShell, one command at a time. Do not screen-share the API key.

$apiKey = Read-Host "Paste your Onomondo SoftSIM API key" .\softsim.exe fetch --api-key $apiKey -n 1 -o .\profiles Remove-Variable apiKey .\softsim.exe next --key .\softsim_key -i .\profiles --format=hex ¦ Set-Content -Encoding ascii .\profile_001.hex $profile = (Get-Content .\profile_001.hex -Raw).Trim() if ($profile -notmatch "^[0-9A-Fa-f]+$") { throw "Profile contains non-HEX characters." } if (($profile.Length % 2) -ne 0) { throw "Profile has an odd number of HEX characters." } Set-Clipboard -Value $profile

# 6. Install the dual-transport simulator package

REQUIRED FOR EVERYONE! This installs the firmware application and the Windows relay tools into the user-specific workspace.

- Go to https://github.com/Alaghai/NRF9151-Simulator-Guides This is where you will download simulator firmware.

- Move the zip folder “Micro_Simulator_Dual_Transport_Complete_v5” to C:\tmp.

- Right click on it and press extract all. Then press extract.

Standard Windows PowerShell. The installer derives the workspace from $env:USERPROFILE, so it works for any Windows username.

$PackageRoot = "C:\tmp\Micro_Simulator_Dual_Transport_Complete_v5\Micro_Simulator_Dual_Transport_Complete_v5" Set-Location (Join-Path $PackageRoot "scripts") powershell -ExecutionPolicy Bypass -File .\install_micro_simulator.ps1

Afterwards the following should be done:

Installed firmware path: $env:USERPROFILE\onomondo-softsim-test\applications\micro_simulator

Installed relay path: $env:USERPROFILE\onomondo-softsim-test\tools\micro_relay

Short build path: C:\b\micro_sim_relay

# 7. Build and flash the simulator

REQUIRED FOR EVERYONE! The same firmware image supports a provisioned SoftSIM, an unprovisioned board, direct LTE, and Windows relay mode.

Run in the nRF Connect SDK 3.4 terminal. The long west build command is one command; it is displayed in a small monospace font to reduce wrapping.

$Workspace = Join-Path $env:USERPROFILE "onomondo-softsim-test" Set-Location $Workspace Remove-Item Env:ZEPHYR_BASE -ErrorAction SilentlyContinue west build --sysbuild -b nrf9151dk/nrf9151/ns -s applications\micro_simulator -d C:\b\micro_sim_relay --pristine=always west flash -d C:\b\micro_sim_relay

If there is an error “Target "partition_manager" not found.”

Run the following in the nRF Connect SDK 3.4 terminal

$Workspace = Join-Path $env:USERPROFILE "onomondo-softsim-test" Set-Location $Workspace @' # Onomondo SoftSIM still requires Partition Manager under NCS 3.4. SB_CONFIG_PARTITION_MANAGER=y # Bundle the SoftSIM template flash data with the sysbuild image. SB_CONFIG_SOFTSIM_BUNDLE_TEMPLATE_HEX=y '@ ¦ Set-Content -Encoding ascii .\applications\micro_simulator\sysbuild.conf

Reset the Board via the connected device or physical button. In a com port you should see:

*** Using Zephyr OS v4.1.99-ff8f0c579eeb ***

[00:00:00.321,990] <inf> micro_simulator: Micro serial simulator started

[00:00:00.323,974] <inf> micro_simulator: SoftSIM is not provisioned

The simulator can run without SoftSIM using the Windows Wi-Fi relay.

To provision SoftSIM later, run: softsim provision

Serial CLI ready. Type: help

Transport: relay

Start the Windows relay script, then run:

connect 137.184.163.176 5000

# 8. First boot and SoftSIM provisioning

REQUIRED FOR EVERYONE! Open the nRF Device Connections briefly after flashing to confirm the board reaches the CLI.

1. Open the nRF Device Connections for the COM port that shows the board logs.

2. Reset the board if needed.

3. Wait for Serial CLI ready. Type: help.

4. Run status and softsim status.

Run in the nRF Device Connections serial COM port

status softsim status

## 8.1 Relay-only Wifi Board

SKIP CELLULAR SETUP: If softsim status reports that no profile is provisioned and only relay mode is required, do not paste a profile. Continue directly to Section 9.

## 8.2 Provision SoftSIM Hex Key

REQUIRED FOR DIRECT CELLULAR ONLY: Use this procedure when the board is not provisioned and cellular operation is needed.

1. In standard Windows PowerShell, copy the complete Hex Key profile to the clipboard.

2. In the nRF Device Connections, run softsim provision. (Shown below)

3. Wait for the board to ask for the SoftSIM HEX profile.

4. Click inside the nRF Device Connections and paste the profile once. Then, press enter. The pasted HEX may not visibly echo.

5. After paste activity stops, wait approximately three seconds. The firmware validates, provisions, and reboots.

6. After reboot, run softsim status again.

softsim provision # Paste the complete HEX profile when prompted. # Wait for automatic provisioning and reboot. softsim status

SUCCESS: The board should report that the profile is provisioned. If the received profile length is much shorter than the PowerShell profile length, copy it again and paste only once.

# 9. Configure the Windows Wi-Fi relay

REQUIRED WHEN LTE-M IS UNAVAILABLE: This is the normal path for countries without LTE-M. It is optional as a backup when cellular is available.

Close the nRF Device Connections, nRF Terminal, and VS code, before running the relay. Only one program can own a COM port at a time.

## 9.1 Create the isolated Python relay environment

Standard Windows PowerShell, not the Nordic SDK terminal. The setup script avoids mixing the Nordic Python environment with the normal Windows Python installation.

$Workspace = Join-Path $env:USERPROFILE "onomondo-softsim-test" $Relay = Join-Path $Workspace "tools\micro_relay" Set-Location $Relay powershell -ExecutionPolicy Bypass -File .\setup_windows_relay.ps1

Download python 3 from the website if you do not have it installed

## 9.2 Start the relay and connect through Wi-Fi

Replace COM5 with the actual nRF9151 DK COM port. Use the VS CODE -> Connected Devices-> COM port to see what the active, talking, COM port is actually named.

The Python relay console now becomes the board terminal.

$Workspace = Join-Path $env:USERPROFILE "onomondo-softsim-test" $Relay = Join-Path $Workspace "tools\micro_relay" Set-Location $Relay powershell -ExecutionPolicy Bypass -File .\run_windows_relay.ps1 -Port COM5

Close the nRF Device Connections, nRF Terminal, and VS code, before running the relay. Only one program can own a COM port at a time.

Try typing  these commands into the Python relay console.

help transport relay connect 137.184.163.176 5000 mode ascii_hex send sample_hb_beacon send sample_hb_gps send sample_location status

BIDIRECTIONAL PATH: The laptop forwards board packets to the TCP server over Wi-Fi, receives the server response, and writes that response back to the board over USB serial.

# 10. Connect directly through LTE

REQUIRED FOR CELLULAR ONLY: Skip this entire section when the test location does not have LTE-M or when relay mode is intentionally being used.

1. Open the nRF Device Connections. The Python relay must be closed if it was using the COM port.

2. Confirm that a SoftSIM profile is provisioned.

3. Select LTE transport and start LTE registration. (Commands below)

4. Wait for lte status to report connected.

5. Connect the firmware TCP socket to the server and send a sample packet.

Run in the nRF Device Connections serial COM port

softsim status transport lte lte connect lte status connect 137.184.163.176 5000 mode ascii_hex send sample_hb_beacon status

NETWORK SUCCESS: Typical success logs include Network registration status: Connected - roaming and LTE connected. Roaming is expected for a global IoT SIM.

# 11. Verify both outgoing packets and returned server responses

The board or relay console should show that the TCP connection was accepted.

The TCP server log should show the received ASCII HEX packet.

The current server normally returns OK.

The board should display the response whether the transport is direct LTE or Windows relay.

Use Document 2 for all simulator commands and Document 3 for TCP server login, logs, and response development.

Run after SSH login in the remote Ubuntu shell. This follows only new server-service output.

journalctl -u micro-tcp-server -f -n 0

# 12. Troubleshooting

| Symptom | Likely cause | Action |
| --- | --- | --- |
| west is not recognized | Wrong terminal. | Open the nRF Connect SDK 3.4 terminal from the VS Code nRF Connect extension. |
| west says the workspace is already initialized elsewhere | ZEPHYR_BASE points to another SDK workspace. | Run Remove-Item Env:ZEPHYR_BASE -ErrorAction SilentlyContinue in the same Nordic terminal. |
| Missing onomondo-uicc files | The git submodule was not initialized. | Run git -C modules/lib/onomondo-softsim submodule update --init, then west update. |
| Build path is too long | Build output is under a long workspace path. | Use C:\b\micro_sim_relay exactly as shown. |
| Board has no SoftSIM but CLI is available | This is expected with the dual-transport firmware. | Use relay mode, or run softsim provision if cellular is needed. |
| Profile does not echo when pasted | nRF Device Connections input is not echoed. | Paste once and wait. Do not paste repeatedly. |
| LTE never connects | No LTE-M coverage, antenna/account/network issue, or unprovisioned profile. | Use relay mode where LTE-M is unavailable; otherwise verify profile, antenna, Onomondo account, and network logs. |
| Python reports python312.dll conflicts with another Python version | Nordic Python and Windows Python were mixed. | Run setup_windows_relay.ps1 from standard Windows PowerShell and use the dedicated .venv. |
| COM port is busy | nRF Device Connections and relay are open at the same time. | Close one program before opening the other. |
| Relay reaches server but board shows no response | Wrong COM port or response framing issue. | Confirm MICRO_RELAY_RX_HEX output in the relay console and retry with the correct board COM port. |

# 13. Security and sharing rules

Do not commit SoftSIM API keys, RSA private keys, or HEX profiles to Git.

Do not include private profiles or server passwords in general documentation.

Use one SoftSIM profile for one physical board.

Ask Amir privately for a board-specific HEX profile and TCP server credentials when authorized.

The C:\ss folder is optional and should exist only on a computer used to generate profiles.

A supplied profile can be stored in another secure local folder; the PowerShell path should point to that actual file.

# Appendix A. Relay-only quick checklist

Install VS Code, nRF Connect extension, SDK/toolchain 3.4.x.

Create/verify the Onomondo west workspace and source module.

Install the simulator package.

Build and flash.

Skip C:\ss, profile generation, profile provisioning, and LTE.

Set up the Windows relay, select transport relay, connect, and send packets.

# Appendix B. Cellular with Amir-provided profile

Install VS Code, nRF Connect extension, SDK/toolchain 3.4.x.

Create/verify the Onomondo west workspace and source module.

Install, build, and flash the simulator.

Skip C:\ss, CLI, RSA, API-key, fetch, and conversion steps.

Copy the supplied board-specific profile to the clipboard.

Run softsim provision, paste once, and wait for reboot.

Select transport lte, run lte connect, and connect to the TCP server.

# Appendix C. Cellular with self-generated profile

Complete all required software/workspace/build steps.

Complete the optional C:\ss SoftSIM CLI workflow because no profile is available.

Provision the generated profile through softsim provision.

Attach to LTE and connect to the TCP server.