DOCUMENT 3 OF 4

Simulator TCP Server and Command Guide

DigitalOcean deployment, Version 7 stream framing, packet validation, timestamp testing, and server-to-device responses

| Version | 7 — Big-endian protocol and validation release |
| --- | --- |
| Protocol | All multi-byte fields big-endian; CRC-16/XMODEM; Unix-millisecond timestamps |
| Primary endpoint | 137.184.163.176:5000 |
| Project standard | Use nRF Connect SDK v3.4.x with the matching v3.4 toolchain |

Compatibility warning: Version 7 is a breaking wire-format change. The nRF simulator, TCP server, decoder, tests, documentation, and the receiving team’s parser must use the same version.

Operational document — do not store passwords, SoftSIM profiles, or private keys in Git or shared documentation.

# Quick-start deployment checklist

| Check | Verification item |
| --- | --- |
| ☐ | Download or clone the Version 7 files from GitHub. |
| ☐ | Upload the server, decoder, and both test files to a staging directory on the DigitalOcean server. |
| ☐ | Back up the currently deployed server and decoder. |
| ☐ | Run Python syntax checks and both automated test suites. |
| ☐ | Stop the existing micro-tcp-server service. |
| ☐ | Install micro_tcp_server.py and micro_packet_decoder.py together in /root. |
| ☐ | Restart the systemd service and confirm port 5000 is listening. |
| ☐ | Send known-good packets in binary mode and confirm validation: VALID in the server log. |
| ☐ | Test dynamic heartbeat, location, timestamp, and response behavior. |

Most important log check: An OK response confirms that the server framed a packet and returned a response. It does not by itself prove that the CRC or packet contents were valid. Confirm validation: VALID in the log.

# 1. Purpose and current endpoint

This guide explains how to update and operate the Version 7 TCP test server, verify packet framing and protocol fields, test direct LTE and Windows relay paths, and confirm server-to-device responses. It replaces the Version 4 server guide for Version 7 integration work.

| Item | Current value |
| --- | --- |
| Public IPv4 | 137.184.163.176 |
| TCP port | 5000 |
| SSH user | root |
| Systemd service | micro-tcp-server |
| Operational server file | /root/micro_tcp_server.py |
| Operational decoder file | /root/micro_packet_decoder.py |
| Packet log | /root/micro_tcp_packets.log |
| Response mode file | /root/micro_response_mode.txt |
| Protocol version | Version 7 — all multi-byte fields big-endian |
| Preferred wire mode | Binary |

Credentials: Ask Amir for current server access details through a private channel. Credentials and SoftSIM HEX profiles must not be placed in Git, shared documents, email threads, or public messages.

# 2. Version 7 change summary

Version 7 corrects the protocol discrepancies identified during integration testing and makes the server validate complete application packets rather than treating each TCP receive operation as a packet.

Endianness: Every numeric field larger than one byte is transmitted big-endian. This includes Length, CRC, Sequence ID, Timestamp, lastUpdate, Latitude, Longitude, Accuracy, and Speed.

Length: The Length field counts the one-byte Command plus the Payload. Total packet bytes are calculated as 8 + Length.

CRC: The placeholder CRC was removed. CRC-16/XMODEM is calculated over the Payload only and placed on the wire as a two-byte big-endian value.

Timestamp: Heartbeat and location timestamps are exactly eight bytes and represent unsigned Unix milliseconds in UTC. A zero timestamp means unavailable.

TCP framing: The server buffers the byte stream, waits for the complete length-defined packet, and separates multiple packets received together.

Wire modes: The server accepts raw binary packets and printable ASCII-HEX packets. Binary is the preferred integration format.

Validation: The server invokes micro_packet_decoder.py to validate length, CRC, command, IMEI, timestamp, coordinates, and other payload fields.

Automated tests: Separate protocol-decoder and TCP-stream tests verify known-good packets, legacy rejection, fragmentation, multiple packets, and ASCII-versus-binary behavior.

## 2.1 Version 7 packet prefix

| Bytes | Field | Encoding | Example |
| --- | --- | --- | --- |
| 0 | Header | Fixed byte | AB |
| 1 | Property | Fixed byte | 10 |
| 2–3 | Length | uint16 big-endian | 00 25 = 37 |
| 4–5 | CRC | uint16 big-endian | 69 FD |
| 6–7 | Sequence ID | uint16 big-endian | 00 01 |
| 8 | Command | 1 byte | 01 heartbeat / 10 location |
| 9+ | Payload | Command-specific | IMEI, timestamp, state, location data |

Total packet bytes = 8 + Length CRC input bytes = Payload only, beginning at byte offset 9

# 3. Transport paths

The server receives the same Micro application packet through either transport. The Windows relay removes its internal serial control markers before forwarding the packet to the TCP server.

| Mode | Path to server | Return path |
| --- | --- | --- |
| Direct LTE | nRF9151 DK → Onomondo SoftSIM/LTE-M → TCP server | Server → LTE TCP socket → board |
| Windows relay | nRF9151 DK → USB serial → Windows relay → Wi-Fi → TCP server | Server → Windows TCP socket → USB serial → board |

Scope: Relay mode validates packet generation, TCP parsing, server responses, and return delivery. It does not validate LTE-M registration, cellular latency, PSM/RRC behavior, roaming, or power consumption.

# 4. Binary versus ASCII-HEX and the leading AB

The text “AB” can represent two different things on a TCP connection. This distinction explains why a logger may appear to see the header while another parser reports that byte 0 is not 0xAB.

| Mode | What the simulator sends | First raw TCP bytes | Parser sees header byte 0xAB? |
| --- | --- | --- | --- |
| Binary | Actual packet bytes | AB 10 00 25 … | Yes |
| ASCII-HEX | Printable text “AB100025…” | 41 42 31 30 30 30 32 35 … | No; 41 is ASCII “A” |

The Version 4 server only printed each recv() chunk. In ASCII-HEX mode, safe_ascii() displayed the characters AB even though the actual bytes were 41 42. The Version 7 server logs both raw_hex and ascii, then converts ASCII-HEX into canonical binary bytes before validation.

Integration recommendation: Use mode binary when testing another team’s real packet parser. In that mode, the first actual TCP byte is 0xAB. Use ASCII-HEX only for readable debugging.

# 5. Required server files

| File | Purpose | Operational requirement |
| --- | --- | --- |
| micro_tcp_server.py | Persistent TCP server, stream buffering, packet framing, logging, responses | Required |
| micro_packet_decoder.py | Protocol field decoding, length and CRC validation | Required for validation; keep beside server file |
| test_micro_tcp_server.py | Offline TCP stream-framing regression tests | Run before deployment |
| test_micro_packet_decoder.py | Offline protocol and decoder regression tests | Run before deployment |

The operational server imports decode_packet from micro_packet_decoder.py. If the decoder is missing or cannot be imported, the server can still frame and log packets, but the log reports validation: decoder unavailable.

# 6. Safe update workflow

## 6.1 Upload from Windows

Windows PowerShell

scp "$env:USERPROFILE\Downloads\micro_simulator_protocol_v7_big_endian_package.zip" root@137.184.163.176:/root/

Windows PowerShell

ssh root@137.184.163.176

## 6.2 Extract to a staging directory

Remote Ubuntu shell

cd /root unzip -o micro_simulator_protocol_v7_big_endian_package.zip cd /root/micro_simulator_protocol_v7_big_endian ls -la

## 6.3 Back up the deployed files

Remote Ubuntu shell

BACKUP="/root/micro_backup_$(date -u +%Y%m%dT%H%M%SZ)" mkdir -p "$BACKUP" cp -a /root/micro_tcp_server.py "$BACKUP/" 2>/dev/null ¦¦ true cp -a /root/micro_packet_decoder.py "$BACKUP/" 2>/dev/null ¦¦ true cp -a /root/micro_tcp_packets.log "$BACKUP/" 2>/dev/null ¦¦ true cp -a /root/micro_response_mode.txt "$BACKUP/" 2>/dev/null ¦¦ true echo "Backup: $BACKUP"

## 6.4 Run syntax checks and automated tests

Remote Ubuntu shell

python3 -m py_compile micro_tcp_server.py micro_packet_decoder.py test_micro_tcp_server.py test_micro_packet_decoder.py python3 -m unittest -v test_micro_packet_decoder.py python3 -m unittest -v test_micro_tcp_server.py

Deployment gate: Do not replace the running server if py_compile fails or either test suite reports FAILED or ERROR.

## 6.5 Stop, install, and restart

Remote Ubuntu shell

systemctl stop micro-tcp-server cp micro_tcp_server.py /root/micro_tcp_server.py cp micro_packet_decoder.py /root/micro_packet_decoder.py python3 -m py_compile /root/micro_tcp_server.py /root/micro_packet_decoder.py systemctl restart micro-tcp-server systemctl status micro-tcp-server --no-pager ss -ltnp ¦ grep ":5000"

## 6.6 Follow new service output

Remote Ubuntu shell

journalctl -u micro-tcp-server -f -n 0

Keep the journal window open while sending packets from the board or a local test client.

# 7. Service, log, network, and firewall commands

## 7.1 Service management

| Command | Use / expected result |
| --- | --- |
| systemctl status micro-tcp-server --no-pager | Show whether the service is running and recent status lines. |
| systemctl start micro-tcp-server | Start the TCP server. |
| systemctl stop micro-tcp-server | Stop the TCP server. |
| systemctl restart micro-tcp-server | Restart after installing or editing Python files. |
| systemctl enable micro-tcp-server | Start automatically after server reboot. |
| systemctl daemon-reload | Reload systemd unit files after changing the service definition. |

## 7.2 Journal and packet logs

| Command | Use / expected result |
| --- | --- |
| journalctl -u micro-tcp-server -f -n 0 | Follow only new service output. Best command during live tests. |
| journalctl -u micro-tcp-server -n 100 --no-pager | Show the last 100 service lines. |
| journalctl -u micro-tcp-server --since "10 minutes ago" --no-pager | Show recent activity. |
| tail -f /root/micro_tcp_packets.log | Follow saved framed-packet records. |
| tail -n 100 /root/micro_tcp_packets.log | Show the last 100 saved records. |
| grep -E "validation:¦wire_mode:¦packet_hex:" /root/micro_tcp_packets.log ¦ tail -n 60 | Show recent framing and validation results. |

## 7.3 Network and firewall checks

| Command | Use / expected result |
| --- | --- |
| ss -ltnp ¦ grep ":5000" | Confirm that a process is listening on TCP port 5000. |
| ufw status verbose | Show firewall rules. |
| ufw allow OpenSSH | Preserve SSH access before enabling UFW. |
| ufw allow 5000/tcp | Allow the Micro test server port. |
| ufw reload | Reload firewall rules. |
| Test-NetConnection 137.184.163.176 -Port 5000 | From Windows, verify that the public endpoint is reachable. |

# 8. Packet decoder and protocol verification

micro_packet_decoder.py is a local inspection tool. It does not connect to the board or server. It accepts a pasted packet HEX string, decodes every field, and reports whether Length and CRC are valid.

## 8.1 Interactive packet decode

Windows or Linux terminal

python3 micro_packet_decoder.py

Paste the complete packet HEX string and press Enter. The decoder prints byte offsets, raw field bytes, interpreted values, errors, warnings, and an Overall result of VALID or INVALID.

## 8.2 Decode a packet supplied on the command line

python3 micro_packet_decoder.py "AB10002569FD0001013836313335323036343035303738370000019F84DE77C0010107FF0101010FAC91003B91"

## 8.3 CRC and timestamp utility commands

python3 micro_packet_decoder.py --crc-self-test python3 micro_packet_decoder.py --timestamp-to-unix-ms 2026-07-21T13:30:00Z python3 micro_packet_decoder.py --unix-ms-to-timestamp 1784640600000

The CRC self-test must produce 0x31C3 for the ASCII text 123456789. The timestamp converter displays the Unix-millisecond value and the exact eight-byte big-endian representation used by the simulator.

## 8.4 Automated protocol tests

python3 -m unittest -v test_micro_packet_decoder.py

The protocol tests verify the CRC check vector, known-good packets, big-endian fields, coordinate decoding, timestamp conversion, zero timestamp behavior, GNSS fix age, corrupted length detection, and rejection of malformed legacy packets.

# 9. TCP stream-parser tests

test_micro_tcp_server.py exercises the parser without opening the public port or changing the running service.

python3 -m unittest -v test_micro_tcp_server.py

One binary packet divided across multiple recv() calls.

One ASCII-HEX packet divided across multiple recv() calls.

Two packets received in one TCP chunk.

Binary and ASCII-HEX packets on the same connection.

Leading non-packet noise before a valid header.

Big-endian Length controlling the total framed packet size.

The difference between binary byte AB and ASCII bytes 41 42.

Expected result: The test output must end with OK. These tests validate stream framing logic, not LTE coverage or the deployed systemd service.

## 9.1 Foreground server test

Stop the systemd service before starting a foreground copy, because only one process can bind port 5000.

Remote Ubuntu shell

systemctl stop micro-tcp-server cd /root python3 -u micro_tcp_server.py

Expected startup text includes “Protocol: Version 7, all multi-byte fields big-endian” and “Accepted wire modes: binary 0xAB... and ASCII-HEX text AB...”. Press Ctrl+C when finished, then restart the service.

## 9.2 Local binary packet test

Second remote Ubuntu shell

python3 - <<'PY' import socket packet = bytes.fromhex( "AB10002569FD000101383631333532303634303530373837" "0000019F84DE77C0010107FF0101010FAC91003B91" ) with socket.create_connection(("127.0.0.1", 5000), timeout=5) as sock: sock.sendall(packet) print(sock.recv(1024)) PY

Expected response: b'OK\n'. The server log should report wire_mode: binary, header_byte: 0xAB, and validation: VALID.

## 9.3 Local ASCII-HEX packet test

python3 - <<'PY' import socket packet_hex = ( "AB10002569FD000101383631333532303634303530373837" "0000019F84DE77C0010107FF0101010FAC91003B91" ) with socket.create_connection(("127.0.0.1", 5000), timeout=5) as sock: sock.sendall(packet_hex.encode("ascii")) print(sock.recv(1024)) PY

The raw receive log begins with 4142 because those are the ASCII characters A and B. The framed packet is converted to binary and should still validate as VALID.

# 10. Systemd deployment

## 10.1 Existing service

systemctl cat micro-tcp-server.service

Confirm that the service runs /root/micro_tcp_server.py and that micro_packet_decoder.py is in the same directory. Unbuffered Python output is recommended so journal messages appear immediately.

## 10.2 Recommended service definition

[Unit] Description=Micro TCP Server Version 7 After=network-online.target Wants=network-online.target [Service] Type=simple User=root WorkingDirectory=/root ExecStart=/usr/bin/python3 -u /root/micro_tcp_server.py Restart=on-failure RestartSec=2 [Install] WantedBy=multi-user.target

systemctl daemon-reload systemctl enable micro-tcp-server systemctl restart micro-tcp-server systemctl status micro-tcp-server --no-pager

# 11. Direct LTE and relay tests

## 11.1 Direct LTE test

nRF terminal

softsim status transport lte lte connect lte status connect 137.184.163.176 5000 mode binary send sample_hb_beacon send sample_hb_gps send sample_location

Expected server results: one accepted TCP connection, one response per complete packet, and validation: VALID for all three known-good packets.

| Packet | Total bytes | Command |
| --- | --- | --- |
| Beacon heartbeat | 45 | 0x01 |
| GPS heartbeat | 51 | 0x01 |
| LTE-M location | 48 | 0x10 |

## 11.2 Windows relay test

Windows PowerShell

$relay = Join-Path $env:USERPROFILE "onomondo-softsim-test\tools\micro_relay" powershell -ExecutionPolicy Bypass -File (Join-Path $relay "run_windows_relay.ps1") -Port COM7

Relay console / nRF terminal

transport relay connect 137.184.163.176 5000 mode binary send sample_hb_beacon send sample_hb_gps send sample_location

Expected return path: server response → Windows TCP socket → MICRO_RELAY_RX_HEX over USB serial → board decode and print.

## 11.3 Dynamic state tests

| Commands | Expected state | Server check |
| --- | --- | --- |
| preset normal send hb | Medium battery, not charging, beacon opcode | Heartbeat validates; beacon address decoded |
| preset low send hb | Low battery | Battery field decodes as low |
| preset charging send hb | Charging | Charging field decodes as charging |
| preset outside send hb send loc | 45.450000, -75.750000, 30.0 m, 1.2 m/s | GPS heartbeat and location both validate |

# 12. Timestamp simulation tests

Version 7 timestamps are uint64 big-endian Unix milliseconds in UTC. The dynamic heartbeat timestamp represents the state-sample time. The dynamic location timestamp represents the GNSS fix time.

| Test | Commands | Expected result |
| --- | --- | --- |
| Fixed time | set timestamp 1784640600000 set time_mode fixed send hb send hb | Both packets decode to 2026-07-21T13:30:00.000Z; Sequence ID increases. |
| Step time | set timestamp 1784640600000 set time_mode step set time_step 5000 send hb three times | Timestamps increase by exactly 5 seconds per dynamic packet. |
| Running time | set timestamp 1784640600000 set time_mode running send hb; wait; send hb | Second timestamp advances approximately by elapsed board uptime. |
| Old GNSS fix | set timestamp 1784640600000 set time_mode fixed set fixage 120 send loc | GNSS fix timestamp decodes to 2026-07-21T13:28:00.000Z. |

Fixed sample commands: send sample_hb_beacon, send sample_hb_gps, and send sample_location remain constant known-good vectors. Timestamp state controls affect send hb and send loc.

# 13. Server response modes

The server reads the response mode from /root/micro_response_mode.txt for each complete framed packet.

| File value | Response | Use |
| --- | --- | --- |
| OK | OK\n | Basic acknowledgement and return-path test. |
| CONFIG_NONE | CONFIG_NONE\n | Indicate that no configuration update is required. |
| ECHO_HEX | RX_HEX:<canonical packet HEX>\n | Prove exactly which packet the server reconstructed. |
| CONFIG_UPDATE_ASCII_HEX:<HEX> | Same readable configuration-update line plus newline | Temporary development response. |
| BINARY_HEX:<HEX> | Raw bytes represented by the supplied HEX | Binary response test. |

echo "OK" > /root/micro_response_mode.txt echo "CONFIG_NONE" > /root/micro_response_mode.txt echo "ECHO_HEX" > /root/micro_response_mode.txt

Validation behavior: The current test server responds even when the decoder marks a framed packet INVALID. This is intentional for diagnostics. Check the packet log; do not treat OK as proof of a valid CRC.

# 14. Reading the Version 7 logs

The service journal records raw TCP chunks, while /root/micro_tcp_packets.log records complete framed packets and decoder results.

Representative valid binary record

recv chunk 45 bytes ... raw_hex=AB100025... ascii=..%... wire_mode: binary packet_length: 45 bytes packet_hex: AB100025... header_byte: 0xAB validation: VALID sequence_id: 1 command: 0x01 (heartbeat) imei: 861352064050787 timestamp_utc: 2026-07-21T13:30:00.000Z

For ASCII-HEX, the raw chunk begins 4142 and the ascii text begins AB. The framed packet_hex still begins AB because the server converts the readable text to binary before decoding it.

An invalid packet includes validation: INVALID followed by one or more validation_error lines, such as a CRC mismatch or Length mismatch.

# 15. Troubleshooting

| Symptom | Check / corrective action |
| --- | --- |
| No new journal output | Confirm service status, port 5000 listener, endpoint, and that a new packet was actually sent. |
| Connection refused | The service is stopped or not listening. Check systemctl status and ss -ltnp. |
| Connection timeout | Check UFW, cloud firewall, address, port, and public reachability. |
| Other server cannot find AB | Confirm wire mode. Binary starts with raw AB; ASCII-HEX starts with raw 41 42. |
| Server logs AB but parser rejects byte 0 | The logger may be displaying ASCII characters. Inspect raw_hex, not only printable ascii. |
| Packet arrives in pieces | Normal TCP behavior. Use a persistent per-connection buffer and Length-based framing. |
| CRC mismatch | Confirm CRC-16/XMODEM, initial 0x0000, payload-only coverage, and big-endian CRC bytes. |
| Length mismatch | Confirm Length is uint16 big-endian and counts Command + Payload. Total bytes are 8 + Length. |
| Timestamp appears unreasonable | Confirm uint64 big-endian Unix milliseconds in UTC, not seconds or nanoseconds. |
| Decoder unavailable | Place micro_packet_decoder.py beside micro_tcp_server.py and restart the service. |
| Address already in use | Stop the existing service or foreground process before starting another server on port 5000. |
| OK received but packet invalid | Expected test-server behavior. Inspect validation: VALID or INVALID in the log. |
| LTE fails but relay works | Likely a cellular coverage, roaming, provisioning, or modem issue. Relay does not validate LTE. |

# 16. Rollback and security

## 16.1 Roll back the server files

systemctl stop micro-tcp-server cp /root/micro_backup_<TIMESTAMP>/micro_tcp_server.py /root/micro_tcp_server.py cp /root/micro_backup_<TIMESTAMP>/micro_packet_decoder.py /root/micro_packet_decoder.py python3 -m py_compile /root/micro_tcp_server.py /root/micro_packet_decoder.py systemctl restart micro-tcp-server

A rollback only restores server software. If the board still sends Version 7 packets, an older parser may remain incompatible. Keep firmware and server protocol versions aligned.

## 16.2 Security boundaries

Do not commit server passwords, API keys, SoftSIM profiles, private keys, or production device credentials.

The current TCP test server is for development. It does not provide application-layer authentication or encryption.

Use restricted firewall rules and a production-grade authenticated protocol before deployment to real users.

Rotate and archive logs according to project data-retention requirements.

# 17. Version 7 acceptance checklist

| Check | Verification item |
| --- | --- |
| ☐ | Both Python files compile without syntax errors. |
| ☐ | Protocol decoder tests complete with OK. |
| ☐ | TCP stream-parser tests complete with OK. |
| ☐ | Systemd service is active and enabled. |
| ☐ | Port 5000 is listening and publicly reachable where intended. |
| ☐ | Known-good beacon heartbeat validates in binary mode. |
| ☐ | Known-good GPS heartbeat validates in binary mode. |
| ☐ | Known-good location packet validates in binary mode. |
| ☐ | ASCII-HEX mode is recognized and converted correctly for debugging. |
| ☐ | Dynamic battery, charging, outside, and location states decode correctly. |
| ☐ | Fixed, running, step, and GNSS-fix-age timestamp tests behave as documented. |
| ☐ | Server responses return through both LTE and Windows relay paths. |
| ☐ | The receiving team has updated its parser to Version 7 big-endian fields. |

Completion criterion: Integration testing is complete only when the sender, receiver, decoder, tests, and documentation all agree on the same Version 7 byte layout and wire mode.