Micro Simulator Protocol Decisions and Usage Guide

nRF9151 DK + Onomondo SoftSIM + Serial-Controlled TCP Simulator

Version: v3 decisions locked for simulator bring-up

Purpose: document the protocol decisions, serial command interface, dynamic packet encoding, server expectations, and copy-paste testing profiles for the Micro device simulator.

Contents

1. Scope and operating model	2

2. Final simulator decisions	2

3. Decisions that correct or override ambiguous protocol text	4

3.1 IMEI fixed at 15 digits	4

3.2 Latitude/longitude use degrees x 1,000,000	4

3.3 Timestamp is 8 bytes	4

3.4 Dynamic packets do not copy inconsistent sample header lengths	4

4. CRC policy	4

5. Header structure for dynamic packets	5

6. Dynamic heartbeat packet	5

7. Dynamic LTE-M location packet	5

8. Startup flow	6

SoftSIM HEX profile	6

9. Default simulator values and available presets	7

Default Values:	7

Available presets:	7

10. Serial commands	8

11. Copy-paste testing profiles	9

Basic bring-up	9

Normal heartbeat	9

Low-battery heartbeat	9

Charging heartbeat	9

Outside safe-zone sequence	9

Beacon detected	9

Trusted device detected	9

Custom location update	9

GPS + LTE heartbeat with custom location	9

Switch to binary transport	9

12. Server expectations and commands	10

13. Source notes	10

# 1. Scope and operating model

This document covers the serial-controlled Micro simulator firmware running on an nRF9151 DK with Onomondo SoftSIM. The simulator is used for backend and firmware bring-up before final device behavior is complete.

The purpose is to let firmware/backend developers simulate Micro device behavior before the final device firmware and hardware are complete.

The simulator can send:

- Heartbeat packet

- LTE-M location update packet

- Exact sample packets from the protocol document

- Raw custom hex packets

- Dynamic packets built from editable fields

# 2. Final simulator decisions

| Topic | Decision | Rationale |
| --- | --- | --- |
| IMEI length | Exactly 15 decimal digits | The sample IMEI is 861352064050787, which is 15 digits. The sample hex contains exactly those 15 ASCII bytes. The simulator rejects anything else. |
| Latitude/longitude scale | Signed int32, degrees x 1,000,000 | The document text says divide by 100,000, but the sample coordinate bytes decode correctly only with divide by 1,000,000. |
| Coordinate byte order | Big-endian signed int32 | FA A2 B5 80 decodes as -90,000,000 only when treated as big-endian two's-complement. |
| Accuracy/speed scale | Signed int16, value x 10 | This follows the protocol text and examples. 25.5 m is encoded as 255 decimal = 00 FF. |
| Timestamp length | 8 bytes uint64 for both heartbeat and LTE-M location | The LTE-M table says 9 bytes while also saying uint64. A uint64 is 8 bytes. Heartbeat already uses 8 bytes. |
| Dynamic timestamp value | Fixed sample timestamp bytes | This keeps simulator output deterministic. Real-time timestamps can be added after clock source and endian policy are finalized. |
| Dynamic header format | AB, property, length LE, CRC LE, sequence LE, command, payload | This follows the documented header structure and avoids copying inconsistent sample-header byte placement into dynamic packets. |
| Length field | uint16 little-endian count of Command + Payload bytes | Best fit to the beacon heartbeat example and clear for backend parser implementation. |
| CRC algorithm | CRC-16/CCITT-FALSE | The LifeAssist CRC divider is unresolved. This gives a deterministic, common CRC for simulator packets. |
| CRC coverage | Sequence ID + Command + Payload | Excludes header, property, length, and CRC field to avoid circular dependency. |
| TCP socket | Persistent socket | The firmware keeps the socket open after connect so developers can send multiple packets in sequence. |
| Default TCP wire format | ASCII hex | Best for bring-up and server logs. Binary mode is still available for parser testing. |
| Production target | Raw binary | Once the parser and protocol stabilize, the backend should be tested in binary mode. |

# 3. Decisions that correct or override ambiguous protocol text

These are simulator decisions, not claims that the protocol source document is already corrected. They should be reflected back into the protocol document when the team updates it.

## 3.1 IMEI fixed at 15 digits

The simulator treats IMEI as exactly 15 decimal digits. The protocol table says 16 bytes, but the sample IMEI value is 861352064050787 and the sample hex is exactly 15 ASCII characters. For safety and consistency with the examples, the simulator rejects 14, 16, non-numeric, or padded IMEIs.

Valid:   set imei 861352064050787
Invalid: set imei 8613520640507870
Invalid: set imei 86135206405078
Invalid: set imei 86135206405078A

## 3.2 Latitude/longitude use degrees x 1,000,000

The source text says divide by 100,000, but the sample bytes prove that 1,000,000 is the consistent scale. Using 100,000 would create impossible coordinates such as -900 degrees latitude.

FA A2 B5 80 = -90,000,000 signed int32
-90,000,000 / 100,000   = -900.00000  (impossible)
-90,000,000 / 1,000,000 = -90.000000  (matches sample)

F5 45 6B 00 = -180,000,000 signed int32
-180,000,000 / 1,000,000 = -180.000000

## 3.3 Timestamp is 8 bytes

The simulator uses 8 bytes for timestamps in both heartbeat and LTE-M location packets. The reason is simple: the document labels the value as uint64, and uint64 is 8 bytes. The heartbeat section also uses 8 bytes. The LTE-M 9-byte line is treated as a document error for simulator v3.

## 3.4 Dynamic packets do not copy inconsistent sample header lengths

The exact sample packet commands remain available for regression tests. Dynamic packet generation uses one internally consistent header policy. This prevents backend developers from having to parse contradictory length and header interpretations during simulator bring-up.

# 4. CRC policy

Dynamic simulator packets use CRC-16/CCITT-FALSE. This is a simulator decision because the original LifeAssist CRC divider is not finalized in the source protocol document.

Polynomial:       0x1021
Initial value:    0xFFFF
Reflected input:  false
Reflected output: false
Final XOR:        0x0000
Coverage:         Sequence ID + Command + Payload
Packet order:     Little-endian CRC field

The firmware writes 00 00 into the CRC bytes while building the packet, then computes CRC from byte offset 6 through the end of the packet and writes the result into bytes 4 and 5.

# 5. Header structure for dynamic packets

Byte(s)  Field        Encoding
0        Header       0xAB
1        Property     0x10
2-3      Length       uint16 little-endian, Command + Payload length
4-5      CRC          uint16 little-endian, CRC-16/CCITT-FALSE
6-7      Sequence ID  uint16 little-endian
8        Command      0x01 heartbeat or 0x10 LTE-M location
9+       Payload      packet-specific payload

# 6. Dynamic heartbeat packet

IMEI                    15 ASCII bytes, exactly 15 decimal digits
Timestamp               8 bytes, fixed simulator timestamp
Battery Level           1 byte: 00 low, 01 medium, 10 high
isCharging              1 byte: 01 not charging, 10 charging
lastUpdate              2 bytes big-endian
Software version         1 byte
Firmware version         1 byte
OPCODE                  1 byte
locationData            6 bytes for beacon/trusted, 12 bytes for GPS-based opcodes

| Opcode | Meaning | locationData |
| --- | --- | --- |
| 0x01 | Beacon detected | 6-byte fixed beacon MAC |
| 0x0A | GPS safe zone | Latitude, longitude, accuracy, speed |
| 0x10 | GPS + LTE | Latitude, longitude, accuracy, speed |
| 0xA0 | Trusted device detected | 6-byte trusted-device address/identity |

# 7. Dynamic LTE-M location packet

IMEI                    15 ASCII bytes, exactly 15 decimal digits
GNSS Fix Timestamp       8 bytes, fixed simulator timestamp
Battery Level           1 byte: 00 low, 01 medium, 10 high
isCharging              1 byte: 01 not charging, 10 charging
lastUpdate              2 bytes big-endian
Latitude                4 bytes signed int32 big-endian, degrees x 1,000,000
Longitude               4 bytes signed int32 big-endian, degrees x 1,000,000
Accuracy                2 bytes signed int16 big-endian, meters x 10
Speed                   2 bytes signed int16 big-endian, meters/second x 10

# 8. Startup flow

After flashing the board:

1. Open the nRF serial terminal.
2. If prompted, paste the Onomondo SoftSIM HEX profile.
3. Wait for LTE connection.
4. Type `help`.
5. Connect to the TCP server.
6. Send sample or dynamic packets.

Expected successful LTE output:

LTE connected
Serial CLI ready. Type: help
Recommended first command (to connect to the DigitalOcean Server) : connect 137.184.163.176 5000

## SoftSIM HEX profile

The SoftSIM profile is the Onomondo HEX profile used to provision the cellular identity.

This is pasted only when the board asks for:

Transfer SoftSIM profile using serial COM port, terminate by newline character (return key)

There will be no indication that it is pasted. Paste it into the serial port and press enter.

# 9. Default simulator values and available presets

## Default Values:

Default server IP:       137.184.163.176
Default server port:     5000
Default mode:            ascii_hex
IMEI:                    861352064050787
Sequence ID:             1
Battery:                 medium
Charging:                not charging
lastUpdate:              2047 minutes
Software version:        1
Firmware version:        1
Heartbeat opcode:        beacon
Latitude:                45.421500
Longitude:               -75.697200
Accuracy:                25.5 m
Speed:                   0.0 m/s
Beacon MAC:              0FAC91003B91
Trusted address:         AABBCCDDEE01

## Available presets:

| Normal profile preset normal Sets: Battery: medium Charging: off Heartbeat opcode: beacon Typical use: preset normal send hb | Low battery profile preset low Sets: Battery: low Charging: off Typical use: preset low send hb This simulates a low-battery heartbeat. | Charging profile preset charging Sets: Battery: medium Charging: on Typical use: preset charging send hb This simulates the device being charged. | Outside safe-zone profile preset outside Sets: Heartbeat opcode: GPS + LTE Latitude: 45.450000 Longitude: -75.750000 Accuracy: 30.0 m Speed: 1.2 m/s Typical use: preset outside send hb send loc This simulates a device outside the default safe-area/beacon context. | Beacon profile preset beacon Sets: Heartbeat opcode: beacon Typical use: preset beacon send hb | Trusted-device profile preset trusted Sets: Heartbeat opcode: trusted device Typical use: preset trusted send hb The protocol separates trusted devices and fixed beacons. Beacons are fixed environmental BLE devices. Trusted devices are user-associated BLE devices such as phones. |
| --- | --- | --- | --- | --- | --- |

# 10. Serial commands

| Category | Commands |
| --- | --- |
| Connection | Help (shows available commands) status (Shows current simulator state) connect (Connect to the default TCP server) connect <PUBLIC_IPV4> <PORT> (Connect to a specific server) disconnect (Disconnect the TCP socket) recv (Manually check for server response) |
| Send mode | mode ascii_hex mode binary |
| Exact sample packets (These commands send exact sample packets from the current Micro protocol document.) | send sample_hb_beacon (Heartbeat: beacon range sample) send sample_hb_gps (Heartbeat: GPS + LTE sample) send sample_location (LTE-M location update sample) |
| Dynamic packets Dynamic packets are built from the simulator’s current state. | send hb (Send dynamic heartbeat) send loc (Send dynamic LTE-M location update) |
| Raw injection Use this when backend developers want to test a new packet string without reflashing firmware. | send raw <HEX_STRING> Example: send raw AB102500D5E623013836 |
| Presets | preset normal preset low preset charging preset outside preset beacon preset trusted |
| Field changes | set imei <15_DIGITS> set battery low¦medium¦high set charging on¦off set opcode beacon¦trusted¦gps_lte¦gps_safezone set pos <latitude> <longitude> <accuracy_m> <speed_mps> set lastupdate <minutes> set versions <software_version> <firmware_version> set beacon <12_HEX_CHARS> set trusted <12_HEX_CHARS> |

# 11. Copy-paste testing profiles

### Basic bring-up

connect 137.184.163.176 5000
mode ascii_hex
send sample_hb_beacon
send sample_hb_gps
send sample_location
status

### Normal heartbeat

preset normal
send hb

### Low-battery heartbeat

preset low
send hb

### Charging heartbeat

preset charging
send hb

### Outside safe-zone sequence

preset outside
send hb
send loc

### Beacon detected

preset beacon
set beacon 0FAC91003B91
send hb

### Trusted device detected

preset trusted
set trusted AABBCCDDEE01
send hb

### Custom location update

set pos 45.4215 -75.6972 25.5 0.0
set battery medium
set charging off
send loc

### GPS + LTE heartbeat with custom location

set opcode gps_lte
set pos 45.4500 -75.7500 30.0 1.2
send hb

### Switch to binary transport

mode binary
send sample_hb_beacon
send hb
send loc

# 12. Server expectations and commands.

FOR MORE INFORMATION SEE 3_Micro_Simulator_TCP_Server_and_Command_Guide.DOCX

After each send, the firmware attempts to read a response from the TCP server.

The current server may return:

OK

Future server responses may include:

CONFIG_NONE
CONFIG_UPDATE_ASCII_HEX:<hex packet>

or eventually a raw binary configuration update packet.

The protocol document says server-to-device configuration updates are sent in response to heartbeat or location packets when the server has newer configuration than the device’s reported lastUpdate.

The Python TCP server should allow long-lived TCP connections. Do not leave a short 20-second idle timeout if the simulator is expected to keep the socket open while a developer types commands.

Recommended timeout:
conn.settimeout(3600)

Recommended default response:
conn.sendall(b"OK\n")

Useful server commands:

systemctl status micro-tcp-server --no-pager
journalctl -u micro-tcp-server -f -n 0
tail -f /root/micro_tcp_packets.log
python3 -m py_compile /root/micro_tcp_server.py
systemctl restart micro-tcp-server
ss -ltnp | grep ':5000'
ufw status verbose

# 13. Source notes

Primary protocol source: Micro Data Packet_updated_ES1.docx. This document defines the packet header, heartbeat, LTE-M location packet, configuration update packet, and TCP transport notes. Simulator v3 deliberately resolves several ambiguities for stable backend testing.

SoftSIM source notes: Onomondo documentation describes the external-profile provisioning flow: build and flash, paste the profile over serial, reboot, then attach using SoftSIM. The simulator preserves this model.