Micro Simulator TCP Server and Serial Command Guide

nRF9151 DK + Onomondo SoftSIM + Persistent TCP Test Server

Purpose. This guide explains how to connect the working Micro serial simulator to the DigitalOcean TCP server, monitor server logs, restart or edit the service safely, and verify simulator commands. It assumes the Onomondo SoftSIM setup and firmware build are already complete.

Current default endpoint Public IPv4: 137.184.163.176 TCP port: 5000 Server service: micro-tcp-server Server file: /root/micro_tcp_server.py Packet log: /root/micro_tcp_packets.log

# 1. Fast path: verify the server and send packets

Use this sequence when the board has booted, LTE is connected, and the serial CLI is ready.

Get TCP server passcode from Amir Laghai if you want to work on that server @Will.

ssh root@137.184.163.176

journalctl -u micro-tcp-server -f -n 0

Then, in the nRF serial terminal:

connect 137.184.163.176 5000
mode ascii_hex
send sample_hb_beacon
send sample_hb_gps
send sample_location
status

Expected result. The server journal should show a new TCP connection and received packet payloads. The board should print a server response such as OK if the server sends one.

# 2. Server command reference

This section avoids repeated explanations and lists the commands by task. The most useful live log command is journalctl -u micro-tcp-server -f -n 0 because it follows only new service output.

## 2.1 SSH and service status

| Command | Use / expected result |
| --- | --- |
| ssh root@137.184.163.176 | Log into the DigitalOcean droplet. |
| systemctl status micro-tcp-server --no-pager | Show whether the TCP server service is running and recent status lines. |
| systemctl start micro-tcp-server | Start the TCP server service. |
| systemctl stop micro-tcp-server | Stop the TCP server service. |
| systemctl restart micro-tcp-server | Restart after editing the Python server file. |
| systemctl enable micro-tcp-server | Start the service automatically after server reboot. |
| systemctl disable micro-tcp-server | Disable automatic startup. Do not normally use this during active testing. |
| systemctl daemon-reload | Reload systemd unit files after editing /etc/systemd/system/micro-tcp-server.service. |

## 2.2 Live service logs with journalctl

| Command | Use / expected result |
| --- | --- |
| journalctl -u micro-tcp-server -f -n 0 | Best command during board testing. Follow only new service logs from this point forward. |
| journalctl -u micro-tcp-server -n 100 --no-pager | Show the last 100 service log lines without entering a pager. |
| journalctl -u micro-tcp-server --since "10 minutes ago" --no-pager | Show logs from a recent time window. |
| journalctl -u micro-tcp-server -b --no-pager | Show service logs since the server last booted. |
| journalctl -u micro-tcp-server -f | Follow logs, including previous tail lines. Use -n 0 when you want only new lines. |

## 2.3 Packet log file commands

| Command | Use / expected result |
| --- | --- |
| tail -f /root/micro_tcp_packets.log | Follow saved packet records as the board sends data. |
| tail -n 100 /root/micro_tcp_packets.log | Show the last 100 saved packet records. |
| grep -i "AB10" /root/micro_tcp_packets.log ¦ tail -n 20 | Show the last likely Micro protocol packets in ASCII hex mode. |
| wc -l /root/micro_tcp_packets.log | Count total saved packet records. |
| ls -lh /root/micro_tcp_packets.log | Check packet log file size. |

## 2.4 Network and firewall checks

| Command | Use / expected result |
| --- | --- |
| ss -ltnp ¦ grep ':5000' | Confirm that something is listening on TCP port 5000. |
| ufw status verbose | Show firewall rules and whether port 5000 is allowed. |
| ufw allow OpenSSH | Allow SSH before enabling UFW. |
| ufw allow 5000/tcp | Allow the Micro TCP test server port. |
| ufw reload | Reload UFW rules after changes. |
| ip -s link show eth0 | Show network byte counters for the main interface. |
| vnstat | Show historical traffic if vnStat is installed. |
| vnstat -l | Live traffic view. Press Ctrl+C to stop. |

## 2.5 Safe edit workflow for the server

Use this workflow every time you edit /root/micro_tcp_server.py.

cp /root/micro_tcp_server.py /root/micro_tcp_server.py.bak.$(date +%Y%m%d_%H%M%S)
nano /root/micro_tcp_server.py
python3 -m py_compile /root/micro_tcp_server.py
systemctl restart micro-tcp-server
systemctl status micro-tcp-server --no-pager
journalctl -u micro-tcp-server -f -n 0

# 3. Recommended server settings

| Command | Use / expected result |
| --- | --- |
| conn.settimeout(3600) | Use a long timeout so the persistent board socket does not close while a developer is typing commands. |
| conn.sendall(b"OK\n") | Simple default response after each received packet. |
| conn.sendall(b"CONFIG_NONE\n") | Optional future response when no config update is needed. |
| conn.sendall(b"CONFIG_UPDATE_ASCII_HEX:AB100100000000\n") | Optional future ASCII config-update test response. |
| datetime.datetime.now(datetime.UTC).isoformat().replace("+00:00", "Z") | Preferred UTC timestamp generation in Python. |
| ExecStart=/usr/bin/python3 -u /root/micro_tcp_server.py | Use unbuffered Python in the systemd service so journal logs appear immediately. |

# 4. PC-side connection check before using the board

From Windows PowerShell, this checks whether the droplet and port are reachable from the PC. It does not prove LTE connectivity, but it quickly catches server or firewall problems.

Test-NetConnection 137.184.163.176 -Port 5000

Expected result. TcpTestSucceeded should be True. If it is False, check server status, firewall, and whether the service is listening on port 5000.

# 5. nRF serial command reference

| Category | Command | Purpose |
| --- | --- | --- |
| Help/status | help | Print available simulator commands. |
| Help/status | status | Show current server, socket state, wire mode, IMEI, battery, charging, lastUpdate, versions, opcode, position, accuracy, and speed. |
| Connection | connect | Connect to the default server endpoint compiled into the firmware. |
| Connection | connect 137.184.163.176 5000 | Connect to the current DigitalOcean TCP server using the public IPv4. |
| Connection | disconnect | Close the current TCP socket. |
| Connection | recv | Manually check for a server response. |
| Wire mode | mode ascii_hex | Send packets as readable ASCII hex. Use this first for server log debugging. |
| Wire mode | mode binary | Convert hex strings to raw bytes before sending. Use after the backend binary parser is ready. |
| Samples | send sample_hb_beacon | Send the exact heartbeat beacon sample packet. |
| Samples | send sample_hb_gps | Send the exact heartbeat GPS + LTE sample packet. |
| Samples | send sample_location | Send the exact LTE-M location update sample packet. |
| Dynamic packets | send hb | Build and send a dynamic heartbeat from the current simulator state. |
| Dynamic packets | send loc | Build and send a dynamic LTE-M location update from the current simulator state. |
| Raw testing | send raw <HEX_NO_SPACES> | Send arbitrary custom hex without reflashing firmware. |
| Presets | preset normal | Medium battery, not charging, beacon opcode. |
| Presets | preset low | Low battery, not charging. |
| Presets | preset charging | Medium battery, charging. |
| Presets | preset outside | GPS + LTE opcode with outside-location test values. |
| Presets | preset beacon | Beacon opcode. |
| Presets | preset trusted | Trusted-device opcode. |
| Fields | set battery low¦medium¦high | Set battery byte. |
| Fields | set charging on¦off | Set charging byte. |
| Fields | set opcode beacon¦trusted¦gps_lte¦gps_safezone | Set heartbeat location/status opcode. |
| Fields | set pos <lat> <lon> <accuracy_m> <speed_mps> | Set simulated GPS/location fields. |
| Fields | set imei <15_DIGITS> | Set 15-digit ASCII IMEI. |
| Fields | set lastupdate <minutes> | Set lastUpdate value used by config-update logic. |
| Fields | set versions <sw> <fw> | Set software and firmware version bytes. |
| Fields | set beacon <12_HEX_CHARS> | Set 6-byte fixed beacon MAC. |
| Fields | set trusted <12_HEX_CHARS> | Set 6-byte trusted-device address/identity. |

# 6. Test sequences

## 6.1 Basic bring-up

connect 137.184.163.176 5000
mode ascii_hex
send sample_hb_beacon
send sample_hb_gps
send sample_location
status

## 6.2 Persistent socket test

send hb
send hb
send loc
send sample_hb_beacon

The server should receive all packets without requiring another connect command. If the connection closes while idle, increase the server socket timeout.

## 6.3 Preset behavior tests

preset normal
send hb

preset low
send hb

preset charging
send hb

preset outside
send hb
send loc

## 6.4 Custom field test

set battery low
set charging on
set opcode gps_lte
set pos 45.4215 -75.6972 25.5 0.0
send hb
send loc
status

## 6.5 Binary parser test

mode binary
send sample_hb_beacon
send hb
send loc
mode ascii_hex

Switch back to ascii_hex afterward unless the server team is actively testing binary parsing.

# 7. Expected observations

| Command | Use / expected result |
| --- | --- |
| Board prints TCP connected | The LTE modem and socket path reached the server. |
| journalctl shows accepted connection | The server process accepted the board TCP connection. |
| Packet log contains AB10... | ASCII hex mode is reaching the server. |
| Board prints OK | The server sent the default OK response and the board read it. |
| Repeated sends work without reconnect | Persistent socket behavior is working. |

# 8. Troubleshooting quick map

| Command | Use / expected result |
| --- | --- |
| No new journal output | Use journalctl -u micro-tcp-server -f -n 0, then send a packet again. Confirm the service is running. |
| Connection refused | Check systemctl status, ss -ltnp, and whether the service is listening on port 5000. |
| Connection timeout | Check UFW, DigitalOcean firewall rules, public IPv4, and whether LTE is attached. |
| Packets stop after idle time | Increase conn.settimeout to 3600 or remove the short timeout during testing. |
| Old warnings keep appearing | Use -n 0 with journalctl to follow only new logs. |
| Server edit breaks service | Restore the latest backup, run python3 -m py_compile, then restart the service. |
| Board says TCP send failed | Reconnect from the serial CLI, then check server logs and firewall. |

# 9. Notes for future backend work

ASCII hex mode is the default for bring-up because packet contents are readable in logs.

Binary mode should be tested once the backend parser is ready for raw packet bytes.

The current firmware connects to one selected endpoint. Final product behavior should support primary, secondary, and fallback TCP endpoints.

Server-to-device config update handling can start with CONFIG_NONE and CONFIG_UPDATE_ASCII_HEX responses before implementing final binary update packets.