# Onomondo SoftSIM Provisioning Guide

Nordic nRF9151 DK using Windows, VS Code, nRF Connect, and the Onomondo SoftSIM CLI

Version 1.0 - June 24, 2026

Outcome At the end of this procedure, the nRF9151 DK has an Onomondo SoftSIM profile provisioned into device storage and can register on LTE using the SoftSIM.

# Contents

1. Assumptions and security rules

2. What the setup does

3. Open VS Code and prepare nRF Connect

4. Create the Onomondo SoftSIM workspace

5. Build and flash the external-profile sample

6. Download and prepare the Onomondo SoftSIM CLI

7. Generate the RSA key pair and create the SoftSIM API key

8. Fetch and format one encrypted SoftSIM profile

9. Provision the SoftSIM profile through the nRF serial terminal

10. Confirm success and interpret the logs

11. Troubleshooting

Appendix A. Clean command checklist

Appendix B. References

# 1. Assumptions and security rules

## Assumptions

Host computer: Windows with PowerShell.

IDE: Visual Studio Code with the nRF Connect extension installed.

Target board: Nordic nRF9151 DK.

Board target used in the build: nrf9151dk/nrf9151/ns.

You already have an Onomondo account and at least one allocated SoftSIM profile available.

- Thomas Sloan currently has an account with 5 active profiles available and possesses login credentials.

- Commands in this guide are PowerShell commands unless stated otherwise.

The guide uses C:\ss as the SoftSIM CLI/provisioning folder and C:\b\softsim as the short build folder.

## Security rules

Do not paste the Onomondo SoftSIM API key into a chat.

Do not share the private key file named softsim_key.

Do not share the generated HEX SoftSIM profile string.

Use one generated SoftSIM profile for one physical device. Do not provision the same profile onto multiple boards.

Keep the C:\ss folder out of Git. It contains key material and profile material.

If you screen-share while typing the API key, assume the key is exposed and rotate it later.

Terminology warning The Onomondo SoftSIM API key, the RSA private key, and the SoftSIM HEX profile are different things. The API key is used on the PC to fetch encrypted profiles. The private key decrypts one local encrypted profile. The HEX profile is the string pasted into the board serial terminal.

# 2. What the setup does

High-level flow:

Onomondo account
    -> SoftSIM API key created with your public RSA key
        -> softsim.exe fetch downloads encrypted profile(s)
            -> softsim.exe next decrypts and formats one profile as HEX
                -> paste HEX profile into nRF serial terminal
                    -> nRF9151 DK stores profile and reboots
                        -> modem registers on LTE using the SoftSIM

# 3. Open VS Code and prepare nRF Connect

## 3.1 Launch VS Code and open the nRF Connect extension

Launch Visual Studio Code.

Open the Extensions view and install the nRF Connect for VS Code extension if it is not already installed.

Open the nRF Connect panel from the VS Code activity bar.

Install or select a compatible nRF Connect SDK and matching toolchain from the nRF Connect extension. Keep the SDK and toolchain versions matched. Do not mix a random SDK folder with a different toolchain.

Open a terminal that has the Nordic/west environment available. The nRF Connect extension can provide this environment. If west is not recognized, the terminal is not using the correct SDK/toolchain environment.

## 3.2 Plug in the nRF9151 DK

Connect the nRF9151 DK to the computer using USB.

Power the board on.

In the nRF Connect extension, look under Connected Devices.

The board should appear as a connected device. Expand it to view board details and serial ports.

Use the listed serial ports later for the nRF serial terminal. One of these ports will show the SoftSIM sample logs and accept the pasted HEX profile.

Practical check If the board does not appear under Connected Devices, fix USB/power/driver detection before continuing. Do not start debugging SoftSIM until the computer reliably sees the DK. You may have to install/update SEGGER. This is mentioned on the NRF website.

# 4. Create the Onomondo SoftSIM workspace

Create a clean workspace that is separate from the default Nordic SDK workspace. This prevents west from mixing the default Nordic manifest with the Onomondo SoftSIM manifest.

Submodule requirement

These commands can be input into the NRC connect terminal. This terminal can be opened under welcome -> Open terminal.

If it does not show up, create an application, then create a temporary build with the NRF9151 board selected as the device, then refresh the nrf connection

The onomondo-uicc library is a git submodule under modules/lib/onomondo-softsim. If you skip the git submodule update command, the CMake configure step can fail because lib/onomondo-uicc is empty.

## 4.1 Create and enter the workspace folder

mkdir C:\Users\%USERNAME%\onomondo-softsim-test
cd C:\Users\%USERNAME%\onomondo-softsim-test

## 4.2 Correction: clear ZEPHYR_BASE before west init if needed

If west reports an error like this:

FATAL ERROR: already initialized in C:\ncs\v3.3.1, aborting.

Note:
    In your environment, ZEPHYR_BASE is set to:
    C:\ncs\v3.3.1\zephyr

    This forces west to search for a workspace there.
    Try unsetting ZEPHYR_BASE and re-running this command.

Run this command in the same PowerShell session:

Remove-Item Env:ZEPHYR_BASE

Scope of this correction Remove-Item Env:ZEPHYR_BASE clears the variable only for the current PowerShell session. That is usually what you want. It avoids breaking other Nordic SDK workflows permanently.

## 4.3 Initialize the Onomondo SoftSIM workspace

west init -m https://github.com/onomondo/nrf-softsim.git
git -C modules/lib/onomondo-softsim submodule update --init
west update

# 5. Build and flash the external-profile sample

The external-profile sample is the correct first test because it waits for a SoftSIM profile over the serial port, provisions it, reboots, and then tries to attach to LTE.

## 5.1 Navigate to the sample

cd modules/lib/onomondo-softsim/samples/softsim_external_profile

## 5.2 Correction: use a short Windows build directory

On Windows, the normal build folder can produce paths that are too long. Use -d C:\b\softsim for the build output.

New-Item -ItemType Directory -Force C:\b | Out-Null
west build --sysbuild -b nrf9151dk/nrf9151/ns -d C:\b\softsim

## 5.3 Flash using the same build directory

west flash -d C:\b\softsim

Important If you build with -d C:\b\softsim, also flash with -d C:\b\softsim. Otherwise west flash may look in the default build folder and fail.

## 5.4 Expected first-boot serial prompt

After flashing, open the serial terminal. On first boot of the external-profile sample, the board should print something similar to:

SoftSIM sample started.
Transfer SoftSIM profile using serial COM port, terminate by newline character (return key)

# 6. Download and prepare the Onomondo SoftSIM CLI

The Onomondo SoftSIM CLI is the PC-side tool used to fetch encrypted profiles from Onomondo and convert one profile into the HEX format accepted by the nRF91 SoftSIM sample.

## 6.1 Create the local provisioning folder

mkdir C:\ss
cd C:\ss

## 6.2 Download the pre-built binary from GitHub Releases

Open the Onomondo SoftSIM CLI GitHub page: https://github.com/onomondo/onomondo-softsim-cli

On the right side of the GitHub page, find Releases.

Open the latest release.

Download the release asset for your operating system. On Windows, download the Windows binary or archive.

Extract the downloaded archive.

Find softsim.exe inside the extracted files.

Move softsim.exe into C:\ss, so the final path is C:\ss\softsim.exe.

## 6.3 Verify softsim.exe runs

cd C:\ss
.\softsim.exe --help

If Windows blocks the downloaded executable, unblock it and run the help command again:

Unblock-File .\softsim.exe
.\softsim.exe --help

PowerShell syntax Use .\softsim.exe, not softsim.exe by itself, when running from the current folder. PowerShell does not automatically execute programs from the current directory unless you include .\.

# 7. Generate the RSA key pair and create the SoftSIM API key

## 7.1 Generate the RSA key pair

Generate a 4096-bit RSA key pair. When ssh-keygen asks for a passphrase, press Enter twice to leave it empty. The SoftSIM CLI does not support password-protected keys.

cd C:\ss
ssh-keygen -t rsa -m PEM -b 4096 -f .\softsim_key

Files created This creates softsim_key and softsim_key.pub. The file softsim_key is the private key. Keep it secret.

## 7.2 Convert the public key to PEM format

Use this PowerShell-safe command. It avoids older PowerShell redirection behavior that can write UTF-16 text files.

ssh-keygen -e -m PKCS8 -f .\softsim_key.pub | Set-Content -Encoding ascii .\softsim_key_public.pem

Check that the file looks like a PEM public key:

Get-Content .\softsim_key_public.pem

Expected shape:

-----BEGIN PUBLIC KEY-----
...
-----END PUBLIC KEY-----

## 7.3 Copy the public key and create the SoftSIM API key

Get-Content .\softsim_key_public.pem -Raw | Set-Clipboard

Log into the Onomondo platform.

Go to the SoftSIM API key creation page. The CLI guide points to https://app.onomondo.com/api-keys/softsim/new.

Paste the public key from your clipboard.

Create the SoftSIM API key.

Copy the generated API key temporarily. You will use it in the next step.

Key matching rule Profiles fetched with a SoftSIM API key can only be decrypted by the private key that matches the public key used to create that API key. If decryption fails later, this mismatch is the first thing to check.

# 8. Fetch and format one encrypted SoftSIM profile

Run line-by-line Run each PowerShell command one at a time. Do not paste the full block as one single line. Wait for each command to finish before moving to the next.

## 8.1 Go to C:\ss and verify files

cd C:\ss
dir

You should see these files at minimum:

softsim.exe
softsim_key
softsim_key.pub
softsim_key_public.pem

## 8.2 Enter the API key into a temporary PowerShell variable

Run this command by itself. Do not edit it:

$apiKey = Read-Host "Paste your Onomondo SoftSIM API key"

PowerShell will show this prompt:

Paste your Onomondo SoftSIM API key:

At that prompt, paste your Onomondo SoftSIM API key and press Enter. Do not add quotes. This command only stores the key in a temporary variable named $apiKey. It does not fetch the profile yet.

Visibility warning Read-Host without -AsSecureString may visibly echo the pasted API key in some terminals. Do not screen-share or record this step. Remove the variable after the fetch command.

## 8.3 Fetch one encrypted profile

Now run the actual fetch command:

.\softsim.exe fetch --api-key $apiKey -n 1 -o .\profiles

Then clear the temporary API key variable:

Remove-Variable apiKey

Check that the profiles folder was created:

dir .\profiles

Expected folder timing The profiles folder is created by softsim.exe fetch. It is normal if C:\ss\profiles does not exist before the fetch command succeeds.

## 8.4 Convert one encrypted profile into the HEX profile

Run this after dir .\profiles shows a downloaded encrypted profile file:

.\softsim.exe next --key .\softsim_key -i .\profiles --format=hex | Set-Content -Encoding ascii .\profile_001.hex

The next command decrypts and formats one profile, then marks the local encrypted profile as used. Do not run it repeatedly unless you intentionally need additional profiles for additional boards.

## 8.5 Validate and copy the HEX profile to the clipboard

dir .\profile_001.hex

$profile = (Get-Content .\profile_001.hex -Raw).Trim()

if ($profile -notmatch '^[0-9A-Fa-f]+$') {
    throw "profile_001.hex contains non-hex characters. Do not provision this."
}

if (($profile.Length % 2) -ne 0) {
    throw "profile_001.hex has an odd number of hex characters. Do not provision this."
}

$profile.Length
Set-Clipboard -Value $profile

Do not edit the profile Use the entire HEX profile string exactly as generated. Do not remove characters, add spaces, split it manually, or reformat it.

# 9. Provision the SoftSIM profile through the nRF serial terminal

## 9.1 Open the serial terminal

In VS Code, open the nRF Connect extension.

Under Connected Devices, expand the nRF9151 DK.

Find the listed serial ports.

Open the nRF serial terminal for the port that shows the board logs.

Reset the board if needed.

Wait for the SoftSIM sample prompt.

## 9.2 Wait for the profile prompt

SoftSIM sample started.
Transfer SoftSIM profile using serial COM port, terminate by newline character (return key)

## 9.3 Paste the HEX profile

Make sure the HEX profile was copied to the clipboard with Set-Clipboard -Value $profile.

Click inside the nRF serial terminal.

Paste the HEX profile.

Press Enter.

Important serial-terminal behavior When you paste the SoftSIM HEX profile into the nRF serial terminal, the terminal may not visibly show the pasted string. That is normal. Paste the profile once and press Enter. Do not paste it repeatedly just because it did not echo.

## 9.4 Expected provisioning behavior

A successful profile transfer may show a line similar to:

received: 258 characters in total

The board should reboot. After reboot, it should stop asking for the profile and instead begin LTE registration.

# 10. Confirm success and interpret the logs

Your observed successful log looked like this:

[00:00:00.405,578] <inf> softsim_sample: SoftSIM sample started.
[00:00:00.407,562] <inf> softsim_sample: Transfer SoftSIM profile using serial COM port, terminate by newline character (return key)
received: 258 characters in total
*** Booting nRF Connect SDK v3.1.1-e2a97fe2578a ***
*** Using Zephyr OS v4.1.99-ff8f0c579eeb ***
[00:00:00.265,747] <inf> softsim_sample: SoftSIM sample started.
[00:00:00.640,136] <inf> softsim_sample: Waiting for LTE connect event.
+CEREG: 2,"71AC","008D1608",7
[00:00:35.094,787] <inf> softsim_sample: RRC mode: Connected
+CEREG: 5,"71AC","008D1608",7,,,"11100000","11100000"
[00:00:38.821,380] <inf> softsim_sample: Network registration status: Connected - roaming
[00:00:38.821,472] <inf> softsim_sample: LTE connected!
[00:00:38.823,852] <inf> softsim_sample: PSM parameter update: TAU: 3240, Active time: -1
+CSCON: 0
[00:00:51.664,367] <inf> softsim_sample: RRC mode: Idle

## 10.1 What this means

| Item | Detail |
| --- | --- |
| received: 258 characters in total | The sample received the HEX profile over serial. |
| Booting nRF Connect SDK | The board rebooted after provisioning, which is expected. |
| Waiting for LTE connect event | The sample moved past provisioning and started cellular registration. |
| +CEREG: 2 | The modem was searching or trying to register. |
| +CEREG: 5 | The modem registered while roaming. For a global IoT SIM, roaming registration is normal. |
| Network registration status: Connected - roaming | The network registration succeeded. |
| LTE connected! | The SoftSIM is provisioned and the board is attached to LTE. |
| RRC mode: Idle / +CSCON: 0 | Not a failure. The radio connection moved idle after activity, which is normal power-saving behavior. |

Status This developer kit now has an Onomondo SoftSIM profile provisioned. The SoftSIM provisioning and LTE registration stages are done. The next stage is application logic: sending your required messages in response to button presses.

## 10.2 Check the Onomondo platform

Log into the Onomondo platform.

Find the SoftSIM/SIM associated with this device.

Check connection status and recent network events.

Use Network Logs, Traffic Monitor, and Signaling Logs to confirm attach and traffic behavior.

# 11. Troubleshooting

| Problem | Likely cause | Action |
| --- | --- | --- |
| west says already initialized in C:\ncs\... | ZEPHYR_BASE points to an existing Nordic SDK Zephyr folder. | Run Remove-Item Env:ZEPHYR_BASE in the current PowerShell session, then rerun west init. |
| CMake fails with missing Onomondo UICC files | The onomondo-uicc submodule was not initialized. | Run git -C modules/lib/onomondo-softsim submodule update --init, then west update. |
| Windows path length error during build | Build output is under a long user/workspace path. | Build with -d C:\b\softsim and flash with west flash -d C:\b\softsim. |
| softsim.exe is not recognized | PowerShell is not running from C:\ss, or the .\ prefix is missing. | Run cd C:\ss, then .\softsim.exe --help. |
| Windows blocks softsim.exe | Downloaded executable is marked blocked. | Run Unblock-File .\softsim.exe, then .\softsim.exe --help. |
| profiles folder does not exist | The fetch command has not completed successfully yet. | Run .\softsim.exe fetch --api-key $apiKey -n 1 -o .\profiles, then dir .\profiles. |
| fetch fails | Wrong API key, no internet, wrong key type, account permission issue, or API issue. | Confirm it is a SoftSIM API key, not a generic key. Retry with a valid key and internet connection. |
| next/decryption fails | Private key does not match the public key used when the SoftSIM API key was created. | Use the matching private key or create a new SoftSIM API key using your current public key. |
| Board does not ask for a profile | Profile may already be provisioned, wrong sample was flashed, or wrong serial port is open. | Confirm you flashed softsim_external_profile, reset the board, and check the other serial port. |
| Pasted profile does not visibly appear | The serial terminal or sample does not echo the input. | Paste once and press Enter. Look for received: ... characters in total and reboot logs. |
| LTE never connects | Antenna, coverage, SIM/account activation, modem firmware, or network selection issue. | Check antenna, location, Onomondo platform status, modem firmware, and network logs. |
| +CEREG: 5 appears | The device is registered while roaming. | This is expected for global IoT SIMs. Treat it as success when followed by LTE connected! |

# Appendix A. Clean command checklist

Use with care This appendix is a compact checklist. For first-time execution, prefer the detailed sections above. Commands that request secrets should still be run carefully and line-by-line.

## A.1 Workspace, build, and flash

cd C:\Users\%USERNAME%
mkdir onomondo-softsim-test
cd onomondo-softsim-test

Remove-Item Env:ZEPHYR_BASE

west init -m https://github.com/onomondo/nrf-softsim.git
git -C modules/lib/onomondo-softsim submodule update --init
west update

cd modules/lib/onomondo-softsim/samples/softsim_external_profile

New-Item -ItemType Directory -Force C:\b | Out-Null
west build --sysbuild -b nrf9151dk/nrf9151/ns -d C:\b\softsim
west flash -d C:\b\softsim

## A.2 SoftSIM CLI, key pair, fetch, and HEX profile

mkdir C:\ss
cd C:\ss

# Manually download the latest Windows pre-built release from:
# https://github.com/onomondo/onomondo-softsim-cli/releases
# Extract it, then move softsim.exe into C:\ss.

.\softsim.exe --help

ssh-keygen -t rsa -m PEM -b 4096 -f .\softsim_key

ssh-keygen -e -m PKCS8 -f .\softsim_key.pub | Set-Content -Encoding ascii .\softsim_key_public.pem

Get-Content .\softsim_key_public.pem -Raw | Set-Clipboard

# Create the SoftSIM API key in the Onomondo platform using the copied public key.

$apiKey = Read-Host "Paste your Onomondo SoftSIM API key"

.\softsim.exe fetch --api-key $apiKey -n 1 -o .\profiles

Remove-Variable apiKey

dir .\profiles

.\softsim.exe next --key .\softsim_key -i .\profiles --format=hex | Set-Content -Encoding ascii .\profile_001.hex

$profile = (Get-Content .\profile_001.hex -Raw).Trim()

if ($profile -notmatch '^[0-9A-Fa-f]+$') {
    throw "profile_001.hex contains non-hex characters. Do not provision this."
}

if (($profile.Length % 2) -ne 0) {
    throw "profile_001.hex has an odd number of hex characters. Do not provision this."
}

$profile.Length
Set-Clipboard -Value $profile

## A.3 Serial provisioning

1. Open VS Code.
2. Open the nRF Connect extension.
3. Expand Connected Devices.
4. Open the nRF serial terminal for the nRF9151 DK serial port.
5. Wait for:

   SoftSIM sample started.
   Transfer SoftSIM profile using serial COM port, terminate by newline character (return key)

6. Paste the HEX profile from the clipboard.
7. Press Enter.
8. Confirm that the board reboots and prints:

   Network registration status: Connected - roaming
   LTE connected!

# Appendix B. References

Onomondo Help Center: How to integrate the Onomondo SoftSIM with the nRF91 Series from Nordic Semiconductor

https://help.onomondo.com/en/articles/208415-how-to-integrate-the-onomondo-softsim-with-the-nrf91-series-from-nordic-semiconductor

Onomondo GitHub: onomondo/nrf-softsim - Onomondo SoftSIM integration for Nordic Semiconductor nRF91 Series

https://github.com/onomondo/nrf-softsim

Onomondo GitHub: onomondo/onomondo-softsim-cli - Onomondo SoftSIM profile provisioning CLI tool

https://github.com/onomondo/onomondo-softsim-cli

Onomondo Help Center: SoftSIM CLI tool for provisioning SoftSIM

https://help.onomondo.com/en/articles/209636-softsim-cli-tool-for-provisioning-softsim

# Appendix C. What is done vs. not done

| Item | Detail |
| --- | --- |
| SoftSIM profile generated on PC | Done after softsim.exe next creates profile_001.hex. |
| SoftSIM profile provisioned onto DK | Done after the board receives the HEX string, reboots, and no longer asks for the profile. |
| LTE registration | Done after Network registration status: Connected - roaming and LTE connected!. |
| Button-message application | Not done yet. This is the next firmware step. |
| Server endpoint validation | Not done yet unless you have already changed the sample destination and confirmed received data. |