DOCUMENT 3 OF 4

# Simulator Protocol Decisions and Usage

Dual-transport serial CLI, packet generation, Windows relay framing, and response handling

Version 4 - Dual transport release

Project standard: Use nRF Connect SDK v3.4 and the matching v3.4 toolchain installed through the nRF Connect for VS Code extension. Do not mix SDK and toolchain versions.

# 1. Scope

This guide documents the behavior of the current dual-transport simulator firmware. It covers packet-generation decisions, transport selection, serial commands, test presets, and server-response handling. It does not replace the source protocol document; unresolved protocol fields are explicitly identified. The exact protocol is still subject to change.

# 2. Operating model

| Transport | Path | SoftSIM required? | What it validates |
| --- | --- | --- | --- |
| LTE | nRF9151 DK -> Onomondo SoftSIM/LTE-M -> TCP server | Yes | Packet generation, modem socket path, LTE attachment, server parsing, and server response. |
| Relay | nRF9151 DK -> USB serial -> Windows laptop Wi-Fi -> TCP server | No | Packet generation, server parsing, server response logic, and return delivery to the board. It does not validate cellular behavior. |

A provisioned board defaults to LTE transport. An unprovisioned board defaults to relay transport. Either board can change transport using the serial CLI.

# 3. Current simulator decisions

| Topic | Current firmware behavior | Reason / caution |
| --- | --- | --- |
| IMEI | Exactly 15 decimal digits. The set command rejects other lengths and non-numeric characters. | Matches the sample IMEI and transmitted payload. |
| Coordinates | Signed int32 big-endian, decimal degrees x 1,000,000. | The provided sample bytes decode to valid -90/-180 coordinates at this scale. |
| Accuracy and speed | Unsigned 16-bit big-endian values x 10. | 25.5 is transmitted as 255 / 0x00FF. |
| Heartbeat timestamp | 8 fixed sample bytes in dynamic heartbeat packets. | Deterministic bring-up behavior. |
| Dynamic location timestamp | 9 fixed sample bytes, matching the current location sample and existing firmware. | The source protocol also labels the field uint64, so final timestamp length remains a protocol item to resolve. |
| Dynamic CRC | Placeholder bytes D5 00. | The final LifeAssist CRC algorithm/divider is not finalized. Do not treat dynamic CRC as production-valid. |
| Dynamic length field | Little-endian count of all bytes after the length field: placeholder CRC, sequence, command, and payload. | This is the behavior implemented by finish_packet_length(). |
| Socket behavior | Persistent connection until disconnect, failure, server close, or transport change. | Supports repeated interactive testing. |
| Default wire mode | ASCII HEX. | Easiest to inspect in server logs. Binary remains available. |

Use exact sample packets first: The sample_hb_beacon, sample_hb_gps, and sample_location commands transmit the exact current sample strings. Use them before dynamic packets when bringing up a new server parser.

\\

# 4. Startup behavior

| Condition at boot | Firmware action |
| --- | --- |
| SoftSIM provisioned | Select LTE transport, prepare the modem, start asynchronous LTE registration, and immediately expose the CLI. |
| SoftSIM not provisioned | Select relay transport, skip modem startup, immediately expose the CLI, and allow softsim provision later. |
| LTE is unavailable | The CLI remains usable. Switch to transport relay and use the Windows relay. |

Recommended first command (to connect to the DigitalOcean Server) : connect 137.184.163.176 5000

# 5. Command reference

| Category | Command | Behavior |
| --- | --- | --- |
| General | help | Print supported commands. |
| General | status | Show transport, SoftSIM/LTE state, endpoint, socket/relay state, wire mode, and simulated fields. |
| Transport | transport lte | Select direct modem TCP. Requires provisioned SoftSIM and LTE registration. |
| Transport | transport relay | Select Windows serial/Wi-Fi relay. Direct LTE socket is closed. |
| SoftSIM | softsim status | Show whether a profile is provisioned. |
| SoftSIM | softsim provision | Read and provision a complete SoftSIM HEX profile from serial, then reboot. |
| LTE | lte status | Show modem/LTE state. |
| LTE | lte connect | Prepare modem if needed and start asynchronous LTE registration. |
| Connection | connect [ip] [port] | LTE mode opens a modem TCP socket. Relay mode sends a connection request to the Windows relay. |
| Connection | disconnect | Close the active LTE or relay TCP connection. |
| Connection | recv | Poll for a response in LTE mode. Relay responses return automatically. |
| Wire | mode ascii_hex | Send packet HEX as printable ASCII characters. |
| Wire | mode binary | Convert packet HEX to raw bytes before TCP transmission. |
| Samples | send sample_hb_beacon | Send exact beacon heartbeat sample. |
| Samples | send sample_hb_gps | Send exact GPS/LTE heartbeat sample. |
| Samples | send sample_location | Send exact LTE location sample. |
| Dynamic | send hb | Build heartbeat from current state and send it. |
| Dynamic | send loc | Build location packet from current state and send it. |
| Raw | send raw <HEX> | Send arbitrary HEX without reflashing. |

# 6. State-editing and preset commands

## Default Values:

Default server IP:       137.184.163.176
Default server port:     5000
Default mode:            ascii_hex
IMEI:                    861352064050787
Sequence ID:             1
Battery:                 medium
Charging:                not charging
lastUpdate:              2047 minutes
Software version:        1
Firmware version:        1
Heartbeat opcode:        beacon
Latitude:                45.421500
Longitude:               -75.697200
Accuracy:                25.5 m
Speed:                   0.0 m/s
Beacon MAC:              0FAC91003B91
Trusted address:         AABBCCDDEE01

HOW TO  EDIT SIMULATE DDEVICE STATE.

| Command | Effect |
| --- | --- |
| preset normal | Medium battery, not charging, beacon opcode. |
| preset low | Low battery, not charging. |
| preset charging | Medium battery, charging. |
| preset outside | GPS+LTE opcode with 45.450000, -75.750000, 30.0 m accuracy, and 1.2 m/s speed. |
| preset beacon | Beacon opcode. |
| preset trusted | Trusted-device opcode. |
| set battery low¦medium¦high | Set battery byte. |
| set charging on¦off | Set charging byte. |
| set opcode beacon¦trusted¦gps_lte¦gps_safezone | Set heartbeat location/status source. |
| set pos <lat> <lon> <accuracy_m> <speed_mps> | Set simulated location data. |
| set imei <15_digits> | Set the transmitted 15-digit IMEI. |
| set lastupdate <minutes> | Set the configuration-age field. |
| set versions <sw> <fw> | Set software and firmware version bytes. |
| set beacon <12_hex_chars> | Set the 6-byte fixed beacon MAC. |
| set trusted <12_hex_chars> | Set the 6-byte trusted-device identity/address. |

| Normal profile preset normal Sets: Battery: medium Charging: off Heartbeat opcode: beacon Typical use: preset normal send hb | Low battery profile preset low Sets: Battery: low Charging: off Typical use: preset low send hb This simulates a low-battery heartbeat. | Charging profile preset charging Sets: Battery: medium Charging: on Typical use: preset charging send hb This simulates the device being charged. | Outside safe-zone profile preset outside Sets: Heartbeat opcode: GPS + LTE Latitude: 45.450000 Longitude: -75.750000 Accuracy: 30.0 m Speed: 1.2 m/s Typical use: preset outside send hb send loc This simulates a device outside the default safe-area/beacon context. | Beacon profile preset beacon Sets: Heartbeat opcode: beacon Typical use: preset beacon send hb | Trusted-device profile preset trusted Sets: Heartbeat opcode: trusted device Typical use: preset trusted send hb The protocol separates trusted devices and fixed beacons. Beacons are fixed environmental BLE devices. Trusted devices are user-associated BLE devices such as phones. |
| --- | --- | --- | --- | --- | --- |

# 7. Windows relay framing and bidirectional responses

The firmware and Windows relay exchange control lines over USB serial. These markers are internal to the relay path and are not sent as Micro protocol payloads to the TCP server.

| Direction | Serial marker | Meaning |
| --- | --- | --- |
| Board -> Windows | MICRO_RELAY_CONNECT:<host>:<port> | Open or change the persistent server connection. |
| Board -> Windows | MICRO_RELAY_DISCONNECT | Close the relay TCP connection. |
| Board -> Windows | MICRO_RELAY_TX_ASCII_HEX:<HEX> | Send the printable HEX string to the server. |
| Board -> Windows | MICRO_RELAY_TX_BINARY_HEX:<HEX> | Convert HEX to bytes and send binary data. |
| Windows -> Board | MICRO_RELAY_CONNECTED:<host>:<port> | Confirm TCP connection. |
| Windows -> Board | MICRO_RELAY_RX_HEX:<HEX> | Return any server response as serial-safe HEX. |
| Windows -> Board | MICRO_RELAY_RX_EMPTY | No response arrived before the relay timeout. |
| Windows -> Board | MICRO_RELAY_RX_ERROR:<text> | Report TCP or parsing failure. |

Example: a server response of OK followed by newline is returned as 4F4B0A. The firmware decodes the HEX and prints the readable response on the board console.

# 8. Exact sample packets

send sample_hb_beacon AB102500D5E6230138363133353230363430353037383719EDB8266D50000000001007FF0101010FAC91003B91

send sample_hb_gps AB101E00D5E6230138363133353230363430353037383719EDB8266D50000000001007FF010110FAA2B580F5456B0000FF00FF

send sample_location AB10100026A1B2B50F1038363133353230363430353037383700506D26B8ED1900007FFFFAA2B580F5456B000FFF0FFF

## Header structure for dynamic packets

Byte(s)  Field        Encoding
0        Header       0xAB
1        Property     0x10
2-3      Length       uint16 little-endian, Command + Payload length
4-5      CRC          uint16 little-endian, CRC-16/CCITT-FALSE
6-7      Sequence ID  uint16 little-endian
8        Command      0x01 heartbeat or 0x10 LTE-M location
9+       Payload      packet-specific payload

## Dynamic heartbeat packet

IMEI                    15 ASCII bytes, exactly 15 decimal digits
Timestamp               8 bytes, fixed simulator timestamp
Battery Level           1 byte: 00 low, 01 medium, 10 high
isCharging              1 byte: 01 not charging, 10 charging
lastUpdate              2 bytes big-endian
Software version         1 byte
Firmware version         1 byte
OPCODE                  1 byte
locationData            6 bytes for beacon/trusted, 12 bytes for GPS-based opcodes

| Opcode | Meaning | locationData |
| --- | --- | --- |
| 0x01 | Beacon detected | 6-byte fixed beacon MAC |
| 0x0A | GPS safe zone | Latitude, longitude, accuracy, speed |
| 0x10 | GPS + LTE | Latitude, longitude, accuracy, speed |
| 0xA0 | Trusted device detected | 6-byte trusted-device address/identity |

## Dynamic LTE-M location packet

IMEI                    15 ASCII bytes, exactly 15 decimal digits
GNSS Fix Timestamp       8 bytes, fixed simulator timestamp
Battery Level           1 byte: 00 low, 01 medium, 10 high
isCharging              1 byte: 01 not charging, 10 charging
lastUpdate              2 bytes big-endian
Latitude                4 bytes signed int32 big-endian, degrees x 1,000,000
Longitude               4 bytes signed int32 big-endian, degrees x 1,000,000
Accuracy                2 bytes signed int16 big-endian, meters x 10
Speed                   2 bytes signed int16 big-endian, meters/second x 10

# 9. Copy-paste test sequences

## 9.1 Relay mode - no SoftSIM or no LTE-M coverage

Terminal: Python relay console

transport relay connect 137.184.163.176 5000 mode ascii_hex send sample_hb_beacon send sample_hb_gps send sample_location status

## 9.2 Direct LTE mode

Terminal: nRF Terminal or Python relay console

softsim status transport lte lte connect lte status connect 137.184.163.176 5000 mode ascii_hex send sample_hb_beacon

## 9.3 State simulations

preset normal send hb preset low send hb preset charging send hb preset outside send hb send loc

## 9.4 Custom state

set imei 861352064050787 set battery low set charging on set opcode gps_lte set pos 45.4215 -75.6972 25.5 0.0 send hb send loc status

## 9.5 Binary parser test

mode binary send sample_hb_beacon send hb send loc mode ascii_hex

# 10. Server responses

After each send, the direct LTE firmware polls for an immediate server response. In relay mode, the Windows relay waits for a response and returns it automatically to the board.

| Response example | Use |
| --- | --- |
| OK\n | Basic acknowledgement and bidirectional-path test. |
| CONFIG_NONE\n | Indicate that the device configuration is current. |
| CONFIG_UPDATE_ASCII_HEX:<HEX>\n | Temporary readable configuration-update development format. |
| Raw binary packet | Future production-style update response after the packet format is finalized. |

FOR MORE INFORMATION SEE 3_Micro_Simulator_TCP_Server_and_Command_Guide.DOCX

After each send, the firmware attempts to read a response from the TCP server.

Future server responses may include:

CONFIG_NONE
CONFIG_UPDATE_ASCII_HEX:<hex packet>

# 11. Known limitations and protocol items to finalize

Dynamic CRC is a placeholder and is not production-valid.

The dynamic location timestamp remains 9 bytes in the current firmware, while the protocol also calls it uint64. Finalize the field length before production.

Exact sample header/length placement is preserved by sample commands; dynamic packets use the internally implemented length convention.

Relay mode validates packets and application responses but not cellular coverage, latency, PSM/RRC behavior, or LTE power consumption.

The current firmware uses one selected endpoint. Final product firmware may require primary, secondary, and fallback servers.