MICRO SIMULATOR

# Corrected Packet Protocol
and Validation Guide

Version 7 - Single big-endian format, timestamp simulation, CRC, and TCP stream framing

Applies to nRF9151 DK + Onomondo SoftSIM simulator

Status: This document replaces the packet-format sections and sample vectors in Version 4. The transport and CLI concepts remain applicable, but the old hard-coded packets must not be used.

Companion files

main.c | micro_settings.c/.h | micro_protocol.py | micro_packet_decoder.py | micro_tcp_server.py | micro_update_tool.py | automated tests

Contents

1. Purpose and outcome	4

1.1 Deliverables in this correction	4

1.2 Decisions used in this revision	4

2. Packet basics: HEX, bytes, offsets, and endianness	6

2.1 Bit, byte, and hexadecimal	6

2.2 Byte offsets	6

2.3 Endianness	6

2.4 ASCII inside a binary packet	7

3. Canonical packet envelope	8

3.1 Length calculation	8

4. Startup behavior	9

5. Command reference	9

6. Windows relay framing and bidirectional responses	11

7. Copy-paste test sequences	12

7.1 Relay mode - no SoftSIM or no LTE-M coverage	12

7.2 Direct LTE mode	12

7.3 State simulations	12

7.4 Custom state	12

7.5 Binary parser test	12

8. CRC definition and implementation	13

8.1 Algorithm identity	13

8.2 Coverage and byte order	13

8.3 Supplied CRC routine	13

8.4 Correct simulator sequence	13

9. Heartbeat packet: command 0x01	14

10. LTE-M location packet: command 0x10	15

11. Latitude, longitude, accuracy, and speed	16

11.1 Coordinate conversion	16

11.2 Accuracy and speed	16

12. Corrected known-good test vectors	17

12.1 Heartbeat - beacon	17

12.2 Heartbeat - GPS + LTE	17

12.3 LTE-M location	17

13. Discrepancies found in Version 4	18

13.1 Old heartbeat beacon sample diagnostic	18

9.2 Old location sample diagnostic	18

14. Simulator functionality review	19

14.1 What is structurally sound	19

14.2 Corrections made in main.c	19

10.3 Remaining simulator limitations	19

15. Using the packet decoder	20

15.1 Basic use	20

15.2 CRC self-test	20

15.3 What the decoder validates	20

15.4 Automated tests	20

16. Timestamp specification and simulation	21

16.1 Default deterministic timestamp	21

16.2 Simulator time modes	21

16.3 Timestamp CLI commands	21

16.4 Converting human-readable time	22

16.5 Timestamp test coverage	22

17. Why Version 7 uses one byte order	23

18. TCP server framing and the leading AB	24

18.3 Recommended integration mode	24

18.4 Running the updated server	24

18.5 Server response modes	24

18.6 Server automated tests	24

19. Extended heartbeat trusted-device registry	25

Known-good extended beacon heartbeat (one configured trusted device):	25

20. Settings-update packet: command 0x20	25

Example heartbeat-interval update (IMEI 861352064050787, update ID 1, value 60 seconds):	26

Compatibility note	26

21. Server responses and settings application	26

22. Decoder and test additions	26

# 1. Purpose and outcome

The simulator is intended to generate deterministic heartbeat and LTE-M location packets so the receiving team can build and verify its parser before production firmware is complete. Version 4 contained conflicting packet definitions, malformed fixed samples, a placeholder CRC, and a length calculation that did not match the documented packet envelope.

Outcome: Version 7 defines one canonical all-big-endian packet structure, freezes the timestamp format, implements timestamp simulation, adds real TCP stream framing, and includes updated known-good vectors and automated tests.

## 1.1 Deliverables in this correction

A corrected Zephyr simulator source file with real CRC generation and corrected length calculation.

A standalone Python decoder that accepts pasted HEX and prints every field, byte offset, decoded value, length validation, and CRC validation.

Automated tests covering CRC, all-big-endian header fields, corrected packets, malformed legacy packets, Ottawa latitude/longitude, Unix-millisecond conversion, zero/unavailable timestamps, stale GNSS-fix timestamps, and TCP stream fragmentation.

This corrected protocol guide, including HEX, bytes, one consistent big-endian convention, signed values, scaling, UTC Unix-millisecond timestamps, timestamp simulation, binary versus ASCII-HEX transport, and server-side stream framing.

## 1.2 Decisions used in this revision

| Decision | Canonical rule | Basis / status |
| --- | --- | --- |
| Length | uint16 big-endian; counts Command + Payload | Matches the stated total-body rule and fixed byte positions. |
| CRC algorithm | CRC-16/XMODEM, initial value 0x0000 | Exact result of the supplied C# routine. |
| CRC coverage | Payload only; bytes after Command | Meeting statement: CRC is calculated on the payload. |
| CRC byte order | Big-endian in bytes 4-5 | Version 7 protocol-wide byte-order rule. The CRC algorithm defines the numeric result; the protocol defines its byte serialization. |
| Sequence ID | uint16 big-endian | Version 7 protocol-wide byte-order rule. |
| Timestamp | uint64 big-endian; Unix milliseconds since 1970-01-01T00:00:00Z; UTC; 0 = unavailable | Version 7 canonical rule; heartbeat = state-sample time; location = GNSS-fix time. |
| Coordinates | signed int32 big-endian, degrees x 1,000,000 | Old coordinate bytes decode to valid -90 and -180 only at x1,000,000. |
| Accuracy / speed | uint16 big-endian, value x10 | Both quantities are non-negative; unsigned encoding is appropriate. |

Breaking change: Length, CRC, and Sequence ID are now big-endian. Update the receiving parser and replace every Version 6 fixed vector.

# 2. Packet basics: HEX, bytes, offsets, and endianness

You do not need to work directly in binary to understand this protocol. The packet is shown in hexadecimal because HEX is a compact way to display bytes.

## 2.1 Bit, byte, and hexadecimal

| Concept | Meaning | Example |
| --- | --- | --- |
| Bit | One binary value: 0 or 1 | 1 |
| Byte | Eight bits | 10101011 |
| HEX digit | Represents four bits; values 0-9 and A-F | A = decimal 10 |
| HEX byte | Two HEX digits represent one byte | AB = binary 10101011 = decimal 171 |

Counting rule: A packet string containing 90 HEX characters contains 45 bytes, because each byte is represented by two HEX characters.

## 2.2 Byte offsets

Offsets in this document are zero-based. Byte 0 is the first byte. Therefore, the ninth byte is byte offset 8. The command must be at byte 8.

## 2.3 Endianness

Endianness is the order used to transmit a multi-byte number. It does not change the number; it changes which byte is sent first. Version 7 uses big-endian, also called network byte order, for every multi-byte integer in the Micro packet. This removes the previous mixed convention and makes the receiver apply one rule consistently.

| Number | Little-endian bytes | Big-endian bytes | Version 7 use |
| --- | --- | --- | --- |
| 0x0025 = decimal 37 | 25 00 | 00 25 | Length uses big-endian: 00 25 |
| 0x0001 = decimal 1 | 01 00 | 00 01 | Sequence ID uses big-endian: 00 01 |
| 0x07FF = decimal 2047 | FF 07 | 07 FF | lastUpdate uses big-endian: 07 FF |

## 2.4 ASCII inside a binary packet

The IMEI is not transmitted as one large integer. Each digit is transmitted as its ASCII byte. For example, the first digit "8" is byte 0x38, digit "6" is 0x36, and digit "1" is 0x31.

IMEI text: 861352064050787
HEX bytes: 38 36 31 33 35 32 30 36 34 30 35 30 37 38 37

Why the old packet showed command 0x38: 0x38 is the ASCII byte for the digit 8. In the old heartbeat sample, the IMEI began at the command position, proving that the command/header fields were shifted.

Every packet has:  Header + Property + Length + CRC + Sequence ID + Command + Payload

| Byte offset | Size | Field | Encoding |
| --- | --- | --- | --- |
| 0 | 1 | Header | Fixed 0xAB |
| 1 | 1 | Property | Fixed 0x10 |
| 2–3 | 2 | Length | Unsigned 16-bit, big-endian |
| 4–5 | 2 | CRC | CRC-16/XMODEM, big-endian |
| 6–7 | 2 | Sequence ID | Unsigned 16-bit, big-endian |
| 8 | 1 | Command | 0x01 heartbeat or 0x10 location |
| 9 onward | Variable | Payload | Command-specific data |

# 3. Canonical packet envelope

AB | 10 | LENGTH_BE | CRC_BE | SEQUENCE_BE | COMMAND | PAYLOAD
 1    1       2          2          2           1      variable bytes

| Absolute byte(s) | Field | Size | Encoding / rule |
| --- | --- | --- | --- |
| 0 | Header | 1 | Constant 0xAB |
| 1 | Property | 1 | Constant 0x10 |
| 2-3 | Length | 2 | uint16 big-endian; number of bytes from Command through end of Payload |
| 4-5 | CRC | 2 | CRC-16/XMODEM over Payload only; stored big-endian |
| 6-7 | Sequence ID | 2 | uint16 big-endian |
| 8 | Command | 1 | 0x01 heartbeat; 0x10 LTE-M location; 0x20 settings update |
| 9+ | Payload | variable | Command-specific fields |

## 3.1 Length calculation

The length field begins at byte 2, but it does not count the fixed bytes before Command. It is calculated after the packet is built:

length = total_packet_bytes - 8
       = 1 command byte + payload bytes

| Packet | Total bytes | Command + Payload | Length bytes |
| --- | --- | --- | --- |
| LTE-M location | 48 | 40 decimal = 0x0028 | 00 28 |
| Extended heartbeat with beacon/trusted address | 70 | 62 decimal = 0x003E | 00 3E |
| Extended heartbeat with GPS fields | 76 | 68 decimal = 0x0044 | 00 44 |

Version 4 code defect: finish_packet_length() used len - 4. That incorrectly counted CRC and Sequence ID. The correct expression is len - 8.

# 4. Startup behavior

| Condition at boot | Firmware action |
| --- | --- |
| SoftSIM provisioned | Select LTE transport, prepare the modem, start asynchronous LTE registration, and immediately expose the CLI. |
| SoftSIM not provisioned | Select relay transport, skip modem startup, immediately expose the CLI, and allow softsim provision later. |
| LTE is unavailable | The CLI remains usable. Switch to transport relay and use the Windows relay. |

Recommended first command (to connect to the DigitalOcean Server) : connect 137.184.163.176 5000

# 5. Command reference

| Category | Command | Behavior |
| --- | --- | --- |
| General | help | Print supported commands. |
| General | status | Show transport, SoftSIM/LTE state, endpoint, socket/relay state, wire mode, and simulated fields. |
| Transport | transport lte | Select direct modem TCP. Requires provisioned SoftSIM and LTE registration. |
| Transport | transport relay | Select Windows serial/Wi-Fi relay. Direct LTE socket is closed. |
| SoftSIM | softsim status | Show whether a profile is provisioned. |
| SoftSIM | softsim provision | Read and provision a complete SoftSIM HEX profile from serial, then reboot. |
| LTE | lte status | Show modem/LTE state. |
| LTE | lte connect | Prepare modem if needed and start asynchronous LTE registration. |
| Connection | connect [ip] [port] | LTE mode opens a modem TCP socket. Relay mode sends a connection request to the Windows relay. |
| Connection | disconnect | Close the active LTE or relay TCP connection. |
| Connection | recv | Poll for a response in LTE mode. Relay responses return automatically. |
| Wire | mode ascii_hex | Send packet HEX as printable ASCII characters. |
| Wire | mode binary | Convert packet HEX to raw bytes before TCP transmission. |
| Samples | send sample_hb_beacon | Send exact beacon heartbeat sample. |
| Samples | send sample_hb_gps | Send exact GPS/LTE heartbeat sample. |
| Samples | send sample_location | Send exact LTE location sample. |
| Dynamic | send hb | Build heartbeat from current state and send it. |
| Dynamic | send loc | Build location packet from current state and send it. |
| Raw | send raw <HEX> | Send arbitrary HEX without reflashing. |
| Config | config show | Show persistent settings, ranges, and defaults. |
| Config | config generate <setting_name> <value> | Generate and print a complete command-0x20 packet without applying it. |
| Config | config apply <complete_packet_hex> | Validate, atomically apply, and persist a complete settings packet. |
| Config | config set <setting_name> <value> | Generate a packet and route it through the same validator/application path. |
| Config | config reset confirm | Restore documented persistent defaults. |
| Config | config last | Show the last update ID, source, result, changed settings, and validation failure. |

HOW TO  EDIT/SIMULATE DEVICE STATE.

| Command | Effect |
| --- | --- |
| preset normal | Medium battery, not charging, beacon opcode. |
| preset low | Low battery, not charging. |
| preset charging | Medium battery, charging. |
| preset outside | GPS+LTE opcode with 45.450000, -75.750000, 30.0 m accuracy, and 1.2 m/s speed. |
| preset beacon | Beacon opcode. |
| preset trusted | Trusted-device opcode. |
| set battery low¦medium¦high | Set battery byte. |
| set charging on¦off | Set charging byte. |
| set opcode beacon¦trusted¦gps_lte¦gps_safezone | Set heartbeat location/status source. |
| set pos <lat> <lon> <accuracy_m> <speed_mps> | Set simulated location data. |
| set imei <15_digits> | Set the transmitted 15-digit IMEI. |
| set lastupdate <minutes> | Set the configuration-age field. |
| set versions <sw> <fw> | Set software and firmware version bytes. |
| set beacon <12_hex_chars> | Set the 6-byte fixed beacon MAC. |
| set trusted <12_hex_chars> | Set the 6-byte trusted-device identity/address. |

| Normal profile preset normal Sets: Battery: medium Charging: off Heartbeat opcode: beacon Typical use: preset normal send hb | Low battery profile preset low Sets: Battery: low Charging: off Typical use: preset low send hb This simulates a low-battery heartbeat. | Charging profile preset charging Sets: Battery: medium Charging: on Typical use: preset charging send hb This simulates the device being charged. | Outside safe-zone profile preset outside Sets: Heartbeat opcode: GPS + LTE Latitude: 45.450000 Longitude: -75.750000 Accuracy: 30.0 m Speed: 1.2 m/s Typical use: preset outside send hb send loc This simulates a device outside the default safe-area/beacon context. | Beacon profile preset beacon Sets: Heartbeat opcode: beacon Typical use: preset beacon send hb | Trusted-device profile preset trusted Sets: Heartbeat opcode: trusted device Typical use: preset trusted send hb The protocol separates trusted devices and fixed beacons. Beacons are fixed environmental BLE devices. Trusted devices are user-associated BLE devices such as phones. |
| --- | --- | --- | --- | --- | --- |

# 6. Windows relay framing and bidirectional responses

The firmware and Windows relay exchange control lines over USB serial. These markers are internal to the relay path and are not sent as Micro protocol payloads to the TCP server.

| Direction | Serial marker | Meaning |
| --- | --- | --- |
| Board -> Windows | MICRO_RELAY_CONNECT:<host>:<port> | Open or change the persistent server connection. |
| Board -> Windows | MICRO_RELAY_DISCONNECT | Close the relay TCP connection. |
| Board -> Windows | MICRO_RELAY_TX_ASCII_HEX:<HEX> | Send the printable HEX string to the server. |
| Board -> Windows | MICRO_RELAY_TX_BINARY_HEX:<HEX> | Convert HEX to bytes and send binary data. |
| Windows -> Board | MICRO_RELAY_CONNECTED:<host>:<port> | Confirm TCP connection. |
| Windows -> Board | MICRO_RELAY_RX_HEX:<HEX> | Return any server response as serial-safe HEX. |
| Windows -> Board | MICRO_RELAY_RX_EMPTY | No response arrived before the relay timeout. |
| Windows -> Board | MICRO_RELAY_RX_ERROR:<text> | Report TCP or parsing failure. |

Example: a server response of OK followed by newline is returned as 4F4B0A. The firmware decodes the HEX and prints the readable response on the board console.

# 7. Copy-paste test sequences

## 7.1 Relay mode - no SoftSIM or no LTE-M coverage

Terminal: Python relay console

transport relay connect 137.184.163.176 5000 mode ascii_hex send sample_hb_beacon send sample_hb_gps send sample_location status

## 7.2 Direct LTE mode

Terminal: nRF Terminal or Python relay console

softsim status transport lte lte connect lte status connect 137.184.163.176 5000 mode ascii_hex send sample_hb_beacon

## 7.3 State simulations

preset normal send hb preset low send hb preset charging send hb preset outside send hb send loc

## 7.4 Custom state

set imei 861352064050787 set battery low set charging on set opcode gps_lte set pos 45.4215 -75.6972 25.5 0.0 send hb send loc status

## 7.5 Binary parser test

mode binary send sample_hb_beacon send hb send loc mode ascii_hex

# 8. CRC definition and implementation

CRC is an error-detection value. The sender calculates it from the payload and inserts the result into bytes 4-5. The receiver performs the same calculation and compares the result. A mismatch means the packet content, packet boundaries, coverage range, algorithm, initial value, or byte order differs.

## 8.1 Algorithm identity

| Parameter | Value |
| --- | --- |
| Common name | CRC-16/XMODEM |
| Polynomial | 0x1021 |
| Initial CRC | 0x0000 |
| Reflect input/output | false / false |
| Final XOR | 0x0000 |
| Check vector | ASCII "123456789" -> 0x31C3 |

## 8.2 Coverage and byte order

CRC input = Payload only = packet bytes 9 through final byte
CRC does NOT include Header, Property, Length, CRC field, Sequence ID, or Command.
CRC field bytes = high byte first, then low byte (big-endian).

Example: If the numeric CRC result is 0x69FD, the packet contains bytes 69 FD because Version 7 stores the field big-endian.

## 8.3 Supplied CRC routine

ushort CalculateCRC16(byte[] data, ushort? initialCrc = null)
{
    ushort crc = initialCrc ?? 0x0000;
    for (int i = 0; i < data.Length; i++)
    {
        crc = (ushort)(((byte)(crc >> 8)) | (crc << 8));
        crc ^= data[i];
        crc ^= (ushort)(((byte)(crc & 0xFF)) >> 4);
        crc ^= (ushort)((crc << 8) << 4);
        crc ^= (ushort)((((crc & 0xFF) << 4)) << 1);
    }
    return crc;
}

## 8.4 Correct simulator sequence

Write Header, Property, zero placeholders for Length and CRC, Sequence ID, and Command.

Append the full command-specific payload.

Calculate Length as total bytes minus 8 and write it big-endian.

Calculate CRC over bytes beginning at payload offset 9.

Write the CRC high byte into byte 4 and low byte into byte 5.

# 9. Heartbeat packet: command 0x01

The heartbeat communicates device identity, a simulated state-sample timestamp, power/configuration state, and one location-status source.

| Absolute byte(s) | Payload field | Size | Encoding |
| --- | --- | --- | --- |
| 9-23 | IMEI | 15 | Exactly 15 ASCII decimal digits |
| 24-31 | Timestamp | 8 | uint64 big-endian Unix milliseconds; UTC; 0 means unavailable; heartbeat state-sample time |
| 32 | Battery level | 1 | 00 low, 01 medium, 10 high |
| 33 | isCharging | 1 | 01 not charging, 10 charging |
| 34-35 | lastUpdate | 2 | uint16 big-endian, minutes |
| 36 | Software version | 1 | uint8 |
| 37 | Firmware version | 1 | uint8 |
| 38 | Opcode | 1 | Determines the final locationData format |
| 39+ | locationData | 6 or 12 | Address or GPS fields |

| Opcode | Meaning | locationData |
| --- | --- | --- |
| 0x01 | Beacon detected | 6-byte fixed beacon MAC/address |
| 0x0A | GPS safe zone | Latitude + longitude + accuracy + speed (12 bytes) |
| 0x10 | GPS + LTE | Latitude + longitude + accuracy + speed (12 bytes) |
| 0xA0 | Trusted device detected | 6-byte trusted-device identity/address |

Length result: Beacon/trusted heartbeat is 45 total bytes and uses Length 00 25. GPS heartbeat is 51 total bytes and uses Length 00 2B.

# 10. LTE-M location packet: command 0x10

The standalone location packet contains the same identity and power/configuration fields, followed directly by latitude, longitude, accuracy, and speed. It does not include heartbeat software version, firmware version, or opcode fields.

| Absolute byte(s) | Payload field | Size | Encoding |
| --- | --- | --- | --- |
| 9-23 | IMEI | 15 | Exactly 15 ASCII decimal digits |
| 24-31 | GNSS fix timestamp | 8 | uint64 big-endian Unix milliseconds; UTC; 0 means unavailable; coordinate measurement time |
| 32 | Battery level | 1 | 00 low, 01 medium, 10 high |
| 33 | isCharging | 1 | 01 not charging, 10 charging |
| 34-35 | lastUpdate | 2 | uint16 big-endian, minutes |
| 36-39 | Latitude | 4 | signed int32 big-endian, degrees x 1,000,000 |
| 40-43 | Longitude | 4 | signed int32 big-endian, degrees x 1,000,000 |
| 44-45 | Accuracy | 2 | uint16 big-endian, metres x 10 |
| 46-47 | Speed | 2 | uint16 big-endian, metres/second x 10 |

Length result: The corrected location packet is 48 total bytes. Command + Payload is 40 bytes, so Length is 00 28.

Version 4 used a 9-byte timestamp. Version 7 uses exactly 8 bytes for the uint64 Unix-millisecond timestamp.

# 11. Latitude, longitude, accuracy, and speed

## 11.1 Coordinate conversion

Coordinates are converted from decimal degrees to signed integers before transmission:

encoded_coordinate = round_or_parse(decimal_degrees x 1,000,000)
decoded_coordinate = signed_int32 / 1,000,000

| Value | Scaled integer | Big-endian bytes |
| --- | --- | --- |
| Latitude 45.421500 | 45,421,500 | 02 B5 13 BC |
| Longitude -75.697200 | -75,697,200 | FB 7C F3 D0 |
| Latitude -90.000000 | -90,000,000 | FA A2 B5 80 |
| Longitude -180.000000 | -180,000,000 | F5 45 6B 00 |

Negative values use two's-complement signed integer encoding. The bytes do not visibly contain a minus sign; the decoder must read them as signed int32.

Validation: The corrected simulator rejects latitude outside -90..90 and longitude outside -180..180. Version 4 parsed values but did not enforce geographic limits.

## 11.2 Accuracy and speed

accuracy_bytes = uint16_big_endian(accuracy_metres x 10)
speed_bytes    = uint16_big_endian(speed_metres_per_second x 10)

| Human value | Stored integer | Bytes |
| --- | --- | --- |
| 25.5 m accuracy | 255 | 00 FF |
| 0.0 m/s speed | 0 | 00 00 |
| 1.2 m/s speed | 12 | 00 0C |

Documentation correction: Version 4 described these as unsigned in one section and signed in another. They are non-negative physical quantities and are now consistently defined as uint16.

# 12. Corrected known-good test vectors

These three packets are generated from the default simulator state with Sequence ID 1, fixed UTC timestamp 2026-07-21T13:30:00.000Z (1784640600000 Unix ms; bytes 00 00 01 9F 84 DE 77 C0), and big-endian serialization for every multi-byte field.

## 12.1 Heartbeat - beacon

AB10002569FD0001013836313335323036343035303738370000019F84DE77C0010107FF0101010FAC91003B91

| Check | Expected |
| --- | --- |
| Total bytes | 45 |
| Length | 0x0025 -> 00 25 |
| CRC numeric | 0x69FD |
| CRC bytes | 69 FD |
| Command | 01 |
| Opcode | 01 beacon |

## 12.2 Heartbeat - GPS + LTE

AB10002BFA890001013836313335323036343035303738370000019F84DE77C0010107FF01011002B513BCFB7CF3D000FF0000

| Check | Expected |
| --- | --- |
| Total bytes | 51 |
| Length | 0x002B -> 00 2B |
| CRC numeric | 0xFA89 |
| CRC bytes | FA 89 |
| Command | 01 |
| Opcode | 10 GPS + LTE |

## 12.3 LTE-M location

AB10002867C80001103836313335323036343035303738370000019F84DE77C0010107FF02B513BCFB7CF3D000FF0000

| Check | Expected |
| --- | --- |
| Total bytes | 48 |
| Length | 0x0028 -> 00 28 |
| CRC numeric | 0x67C8 |
| CRC bytes | 67 C8 |
| Command | 10 |
| Latitude / longitude | 45.421500 / -75.697200 |

# 13. Discrepancies found in Version 4

| Issue | What was wrong | Correction |
| --- | --- | --- |
| Dynamic length calculation | Code used total length minus 4, which included CRC and Sequence ID. | Use total length minus 8; count Command + Payload only. |
| CRC | Dynamic packets inserted placeholder bytes D5 00; fixed samples used unrelated D5 E6 / 26 A1 values. | Calculate CRC-16/XMODEM over Payload and write it big-endian. |
| Heartbeat sample command | Byte 8 was 0x38, the ASCII digit 8 and first IMEI byte. A 0x01 appeared at byte 7. | Two-byte Sequence ID at bytes 6-7; Command 0x01 at byte 8; IMEI starts byte 9. |
| GPS heartbeat length | Old field was 1E 00 = 30, but actual Command + Payload was 43 bytes. | Version 7 uses 00 2B. |
| Location length | Old field was 10 00 = 16, but actual Command + Payload occupied 40 bytes. | Version 7 uses 00 28. |
| Location command placement | Byte 8 was 0x0F; valid command 0x10 appeared at byte 9, and IMEI began at byte 10. | Command must be at byte 8 and IMEI at byte 9. |
| Location timestamp | Dynamic builder appended 9 bytes while the field was described as uint64. | Use exactly 8 bytes. |
| Coordinate documentation | Text mentioned x100,000 while sample boundary coordinates only made sense at x1,000,000. | Use x1,000,000 consistently. |
| Accuracy/speed signedness | One section said unsigned and another said signed. | Use uint16 because values cannot be negative. |
| Fixed sample strategy | Documentation recommended old fixed samples even though they conflicted with the dynamic layout. | Replace fixed samples with vectors generated from the canonical builder. |

## 13.1 Old heartbeat beacon sample diagnostic

AB102500D5E6230138363133353230363430353037383719EDB8266D50000000001007FF0101010FAC91003B91

Length 25 00 happens to equal the actual 37 bytes after byte 7.

CRC bytes D5 E6 do not match the supplied algorithm over the payload.

Sequence bytes decode as 23 01, and byte 8 is 38 rather than a valid command.

The 15-digit IMEI begins at byte 8 instead of byte 9.

## 9.2 Old location sample diagnostic

AB10100026A1B2B50F1038363133353230363430353037383700506D26B8ED1900007FFFFAA2B580F5456B000FFF0FFF

Length 10 00 declares 16 bytes, while the packet actually contains 40 Command + Payload bytes.

CRC bytes 26 A1 do not match the supplied algorithm over the payload.

Byte 8 is 0F, not a defined command. Command 10 appears one byte late at byte 9.

The IMEI begins at byte 10, demonstrating an inconsistent header/sequence width.

# 14. Simulator functionality review

## 14.1 What is structurally sound

The simulator cleanly separates direct LTE TCP and Windows serial/Wi-Fi relay transports.

SoftSIM provisioning, asynchronous LTE registration, persistent TCP connection, and serial CLI are reasonable for integration testing.

The state-editing commands make it possible to test battery, charging, beacon, trusted-device, GPS, IMEI, versions, and position changes without reflashing.

The coordinate input parser correctly handles negative decimal values and stores six decimal places.

Binary mode transmits actual packet bytes; ASCII-HEX mode remains useful for human-readable server logs.

## 14.2 Corrections made in main.c

Implemented the supplied CRC routine as CRC-16/XMODEM and retained the startup self-test using 123456789 -> 0x31C3.

Changed packet finalization to calculate Length as len - 8 and CRC over bytes 9 through end.

Changed Length, CRC, and Sequence ID serialization to big-endian so every multi-byte field uses one byte order.

Replaced all three fixed samples with Version 7 all-big-endian known-good vectors.

Changed LTE-M location timestamp from 9 bytes to the canonical 8-byte uint64 big-endian Unix-millisecond field.

Added latitude and longitude range validation.

Changed transport switching so selecting LTE explicitly closes an active relay connection.

Added error handling if packet finalization fails.

## 10.3 Remaining simulator limitations

| Limitation | Impact |
| --- | --- |
| No real UTC/GNSS time source | The simulator generates controlled timestamps; it does not prove modem/GNSS clock acquisition or RTC retention. |

The default wire mode is now binary so integration servers receive the actual header byte 0xAB. ASCII-HEX is still available as an explicit diagnostic mode.

# 15. Using the packet decoder

micro_packet_decoder.py is a dependency-free Python command-line tool. It accepts corrected packets and malformed legacy packets. It reports the exact byte that failed rather than silently shifting fields

## 15.1 Basic use

python micro_packet_decoder.py

Paste the HEX string when prompted and press Enter.

python micro_packet_decoder.py AB10002569FD0001013836...

python micro_packet_decoder.py --json AB10002569FD0001013836...

## 15.2 CRC self-test

python micro_packet_decoder.py --crc-self-test
CRC-16/XMODEM('123456789') = 0x31C3

## 15.3 What the decoder validates

Header and Property constants.

Length field as uint16 big-endian against actual packet size.

CRC over Payload using the supplied algorithm.

Sequence ID as uint16 big-endian and Command placement.

IMEI as exactly 15 ASCII digits.

Timestamp as exactly 8 bytes, uint64 big-endian Unix milliseconds, with UTC conversion and zero-as-unavailable handling.

Heartbeat opcode-dependent locationData length.

Latitude/longitude signed decoding and geographic bounds.

Unexpected trailing bytes or truncated fields.

Likely shifted legacy packets by searching nearby for known commands and 15-digit IMEI runs.

## 15.4 Automated tests

python test_micro_packet_decoder.py

The included decoder tests verify all Version 7 vectors, reject the old heartbeat and location vectors, check CRC-16/XMODEM, verify big-endian Length/CRC/Sequence ID, detect corrupted length, and verify 45.421500 / -75.697200 decoding.

# 16. Timestamp specification and simulation

Canonical rule: Every heartbeat and LTE-M location timestamp is an unsigned 64-bit integer in big-endian byte order containing Unix time in milliseconds since 1970-01-01T00:00:00Z. All times are UTC. The value 0 means that no valid timestamp is available. This follows the same big-endian rule as every other multi-byte Version 7 field.

| Property | Version 7 rule | Reason |
| --- | --- | --- |
| Length | 8 bytes / uint64 | Prevents the previous 8-versus-9-byte field shift. |
| Byte order | Big-endian | Matches the other multi-byte payload numeric fields. |
| Epoch | 1970-01-01T00:00:00Z | Standard Unix epoch. |
| Unit | Milliseconds | Adequate ordering precision and broad backend compatibility. |
| Timezone | UTC | Avoids local-time and daylight-saving ambiguity. |
| Zero value | Unavailable / invalid time | Provides an explicit no-time sentinel. |
| Heartbeat meaning | Time the represented device state was sampled | Not merely the TCP send time. |
| Location meaning | Time the transmitted GNSS coordinates were fixed | May be older than the packet send time. |

## 16.1 Default deterministic timestamp

The fixed known-good samples use:

| Representation | Value |
| --- | --- |
| ISO-8601 UTC | 2026-07-21T13:30:00.000Z |
| Unix milliseconds | 1784640600000 |
| uint64 big-endian bytes | 00 00 01 9F 84 DE 77 C0 |

## 16.2 Simulator time modes

| Mode | Behaviour | Primary use |
| --- | --- | --- |
| fixed | Every dynamically generated packet uses the configured timestamp. | Repeatable packet/CRC and parser tests. |
| running | Configured Unix time advances using Zephyr board uptime. | Realistic increasing timestamps without requiring a real RTC. |
| step | Timestamp advances by a configured number of milliseconds after each generated packet. | Fast deterministic ordering and gap tests. |

## 16.3 Timestamp CLI commands

| Command | Effect | Example |
| --- | --- | --- |
| set timestamp <unix_ms> | Set the base UTC timestamp and reset the uptime anchor. | set timestamp 1784640600000 |
| set time_mode fixed¦running¦step | Select fixed, uptime-running, or per-packet step behaviour. | set time_mode running |
| set time_step <milliseconds> | Set the increment used by step mode; must be greater than zero. | set time_step 5000 |
| set fixage <seconds> | Make location packet GNSS-fix time older than simulated current time. Heartbeat time is unaffected. | set fixage 120 |
| status | Show current simulated Unix ms, mode, step, and GNSS fix age. | status |

Example - fixed time:
set timestamp 1784640600000
set time_mode fixed
send hb
send loc

Example - running time with a two-minute-old GNSS fix:
set timestamp 1784640600000
set time_mode running
set fixage 120
send hb
send loc

Example - deterministic five-second steps:
set timestamp 1784640600000
set time_mode step
set time_step 5000
send hb
send hb
send loc

## 16.4 Converting human-readable time

The firmware accepts Unix milliseconds to avoid timezone and text-parsing ambiguity. The decoder provides conversion commands:

python micro_packet_decoder.py --timestamp-to-unix-ms 2026-07-21T13:30:00Z

python micro_packet_decoder.py --unix-ms-to-timestamp 1784640600000

The first command prints the Unix-millisecond value, the eight big-endian packet bytes, and the exact simulator command to enter.

## 16.5 Timestamp test coverage

| Automated test | What it proves |
| --- | --- |
| ISO-to-Unix and Unix-to-ISO conversion | The default timestamp round-trips to 2026-07-21T13:30:00.000Z. |
| Big-endian byte vector | 1784640600000 encodes as 00 00 01 9F 84 DE 77 C0. |
| Known-good packet timestamps | All three fixed vectors decode to the same canonical UTC timestamp. |
| Zero timestamp | Value 0 decodes as unavailable while retaining a valid packet CRC. |
| GNSS fix-age vector | A 120-second-old fix decodes to 2026-07-21T13:28:00.000Z. |
| CRC, length, and protocol endianness regression tests | Known vectors require big-endian Length, CRC, Sequence ID, timestamp, and payload numeric fields; changing the timestamp still requires a matching CRC while packet length remains unchanged. |

# 17. Why Version 7 uses one byte order

The prior Version 6 format mixed little-endian header/control fields with big-endian payload fields. That provided no protocol benefit and made parser mistakes more likely. Version 7 standardizes every multi-byte integer as big-endian/network byte order. This is a breaking change: Version 6 and Version 7 packets must not be parsed with the same byte-order assumptions.

| Field group | Version 7 byte order | Reason |
| --- | --- | --- |
| Length, CRC, Sequence ID | Big-endian | One consistent network-order rule; removes header-specific exceptions. |
| Timestamp, lastUpdate, coordinates, accuracy, speed | Big-endian | Network byte order for every multi-byte payload value. |
| Single-byte values | Not applicable | A one-byte value has no endianness. |
| Version 7 decision: every multi-byte integer uses big-endian/network byte order. Do not silently deploy this against a Version 6 parser. Coordinate simulator and server updates, replace fixed vectors, and run both test suites before integration. |  |  |

# 18. TCP server framing and the leading AB

| Simulator mode | First bytes on TCP | What a raw HEX logger prints | Meaning |
| --- | --- | --- | --- |
| binary | AB 10 00 25 ... | AB100025... | The first network byte is the actual header byte 0xAB. |
| ascii_hex | 41 42 31 30 30 30 32 35 ... | 4142313030303235... | The network contains text characters "AB100025...". 0x41 is A and 0x42 is B; there is no binary 0xAB byte. |

ASCII-HEX wire bytes: 41 42 31 30 ...  -> text "AB10..."
Binary wire bytes:    AB 10 00 25 ...  -> actual packet header 0xAB

## 18.3 Recommended integration mode

Use binary mode when testing with another team's parser:

mode binary
send sample_hb_beacon
send hb
send loc

Use ascii_hex only when a person needs readable packet text in a simple terminal or log. A production-style receiver should normally consume actual binary packet bytes.

## 18.4 Running the updated server

sudo cp micro_tcp_server.py /root/
sudo cp micro_packet_decoder.py /root/
sudo chmod +x /root/micro_tcp_server.py
sudo python3 /root/micro_tcp_server.py

Keep both Python files in the same directory. The server can frame packets without the decoder, but placing the decoder beside it enables detailed validation in the log.

## 18.5 Server response modes

| File content | Response |
| --- | --- |
| AUTO | Valid/no update -> OK; invalid -> ERROR; pending update -> SUP plus binary packet |
| ERROR | Packet rejected; settings remain unchanged |
| SUP | Settings update follows as one Version 7-framed binary packet |
| FWUP | Recognized and logged; firmware-image handling deferred |

## 18.6 Server automated tests

python test_micro_tcp_server.py

The tests cover binary and ASCII-HEX packets split across recv calls, two packets combined in one recv call, mixed modes on one connection, leading noise, big-endian Length framing, and the difference between ASCII bytes 41 42 and binary header byte AB.

# 19. Extended heartbeat trusted-device registry

Every dynamically generated heartbeat retains the existing opcode-selected locationData and then appends the complete configured trusted-device registry. The registry describes which trusted devices are configured; it does not indicate which device is currently nearby. Static beacon identifiers remain separate. Original Version 7 heartbeats without this block remain decodable.

| Field | Size | Rule |
| --- | --- | --- |
| trustedDeviceCount | 1 byte | 0 to 4 |
| trustedDeviceSlot1..4 | 4 x 6 bytes | Configured entries first in deterministic order; unused slots are six zero bytes |
| Total extension | 25 bytes | Included in the payload-only CRC |
| Heartbeat | Total bytes | Length |
| Extended beacon/trusted | 70 | 00 3E |
| Extended GPS | 76 | 00 44 |

## Known-good extended beacon heartbeat (one configured trusted device):

AB10003E621D0001013836313335323036343035303738370000019F84DE77C0010107FF0101010FAC91003B9101AABBCCDDEE01000000000000000000000000000000000000

A heartbeat can identify one trusted Bluetooth device that is currently detected through the existing fields:

Opcode:       0xA0

locationData: 6-byte trusted-device identity/address

The new trusted-device extension is separate:

trustedDeviceCount

trustedDeviceSlot1

trustedDeviceSlot2

trustedDeviceSlot3

trustedDeviceSlot4

Those four slots report which trusted devices are configured on the tracker.

# 20. Settings-update packet: command 0x20

The settings update uses the unchanged Version 7 envelope. The payload is typed TLV data so settings can be added without redefining the complete packet. The simulator validates the full packet before changing active state and persists one complete validated configuration record through Zephyr settings/NVS.

The implemented command-0x20 settings packet supports six persistent settings. Each packet may contain one setting or several settings. Settings not included in the packet remain unchanged.

| Payload field |  | Size |  | Encoding / validation |  |
| --- | --- | --- | --- | --- | --- |
| Target IMEI |  | 15 bytes |  | Exactly 15 ASCII decimal digits |  |
| Schema version |  | 1 byte |  | Initial value 0x01 |  |
| Update ID |  | 2 bytes |  | uint16 big-endian |  |
| Entry count |  | 1 byte |  | Number of TLV entries |  |
| Setting ID |  | 1 byte |  | Must exist in the shared registry; duplicates rejected |  |
| Value type |  | 1 byte |  | Must match the registry definition |  |
| Value length |  | 2 bytes |  | uint16 big-endian |  |
| Value |  | Variable |  | Validated type, length, and range |  |
| Setting ID | Name |  | Type |  | Development range/default |
| 0x01 | heartbeat_interval_seconds |  | uint16 |  | 1..65535; default 60 |
| 0x02 | lte_update_interval_seconds |  | uint16 |  | 1..65535; default 480 |
| 0x03 | sleep_interval_seconds |  | uint16 |  | 1..65535; default 480 |
| 0x10 | safe_zones |  | raw |  | 0..4 records of int32 latitude_e7, int32 longitude_e7, uint16 radius_m |
| 0x11 | beacon_list |  | raw |  | 0..4 six-byte static beacon identifiers |
| 0x12 | trusted_device_list |  | raw |  | 0..4 six-byte trusted-device identifiers |

## Example heartbeat-interval update (IMEI 861352064050787, update ID 1, value 60 seconds):

AB10001A63A80001203836313335323036343035303738370100010101020002003C

## Compatibility note

The supplied under-development firmware function parses a positional payload containing zones, beacons, trusted devices, LTE interval, sleep interval, and a final sending-update flag. The simulator implementation requested for this revision uses command 0x20 and TLV entries. The setting meanings were used as reference, but the two payloads are not binary-compatible until the production firmware adopts the same command and schema.

## Device serial commands

### Inspect the current configuration

config show

Display the result of the most recent attempted update:

config last

### Change the heartbeat interval

Generate a packet without applying it:

config generate heartbeat_interval_seconds 60

Generate, validate, apply, and persist it:

config set heartbeat_interval_seconds 60

Other interval examples:

config set heartbeat_interval_seconds 120

config set lte_update_interval_seconds 480

config set sleep_interval_seconds 300

### Update static beacons

One beacon:

config set beacon_list 142525252525

Two beacons:

config set beacon_list 1425252525250FAC91003B92

Four beacons:

config set beacon_list 1425252525250FAC91003B92AB12C5D6E9A5D0A556B54856

Each beacon is exactly six bytes, represented by 12 HEX characters. The values are concatenated without separators:

142525252525 | 0FAC91003B92

The beacon list normally originates from the server and is stored on the device.

### Update trusted-device identifiers

One trusted device:

config set trusted_device_list 123456789123

Two trusted devices:

config set trusted_device_list 123456789123AABBCCDDEE02

Four trusted devices:

config set trusted_device_list 123456789123AABBCCDDEE02AB12C5D6E9A5C354DE15A6B5

The current code permits trusted devices to be changed by a server update. Under your intended ownership model, this should normally be used only for deliberate restoration or administrative synchronization. In regular operation, the device will more commonly report its configured trusted-device registry to the server through its extended heartbeat.

The implementation treats these as six-byte non-secret development identifiers. It does not transmit IRKs, bonding keys, or other Bluetooth secrets.

### Update safe zones

Each safe-zone record is:

Latitude:  4-byte signed int32, big-endian, degrees × 10,000,000

Longitude: 4-byte signed int32, big-endian, degrees × 10,000,000

Radius:    2-byte uint16, big-endian, metres

Example for:

Latitude:  45.421500

Longitude: -75.697200

Radius:    150 metres

Encoded record:

1B12C558 D2E18620 0096

Command:

config set safe_zones 1B12C558D2E186200096

Multiple safe zones are concatenated. No count byte is entered manually; the number of complete 10-byte records determines the count.

A settings safe-zone coordinate uses degrees × 10,000,000. This differs from the existing heartbeat and location coordinate representation, which uses degrees × 1,000,000.

### Apply an already generated packet

config apply AB10001A63A80001203836313335323036343035303738370100010101020002003C

That example targets IMEI 861352064050787 and sets the heartbeat interval to 60 seconds.

Restore defaults:

config reset confirm

This resets all six persistent setting groups through the same packet-generation and validation path.

# 21. Server responses and settings application

| Response | Simulator behavior |
| --- | --- |
| OK | Log success and continue normal operation. |
| ERROR or ERROR:<code> | Preserve and log the response; do not alter settings. |
| SUP | Buffer the following binary settings packet across arbitrary TCP reads; validate, apply atomically, and persist if valid. |
| FWUP | Log as deferred; do not request or install firmware content. |

The direct LTE receive path and the Windows relay return path feed the same response parser. A token may be split across reads, SUP and the packet may arrive together, or the packet may be fragmented. Invalid, truncated, wrong-IMEI, wrong-schema, duplicate, unknown, out-of-range, or CRC-failed updates leave the previous configuration unchanged.

## 22. Decoder and test additions

The decoder and tests now cover original and extended heartbeats, command 0x20 settings, fragmented responses, AUTO mode, IMEI isolation, and C/Python constant agreement.