Micro Data Packet Canonical Version 8

State machine, independent timers, heartbeat, LTE-M location, and server-to-device configuration

# 1. Status and scope

This document is the cross-team source of truth. The simulator and TCP-server guides are implementation and operating guides; they must follow this document. It replaces the former command-0x20 typed-TLV settings format. Command 0x02 is positional and full replacement.

# 2. Common packet envelope

AB | 10 | LENGTH_BE | CRC_BE | SEQUENCE_BE | COMMAND | PAYLOAD

| Offset | Field | Rule |
| --- | --- | --- |
| 0 | Header | Fixed 0xAB |
| 1 | Property | Fixed 0x10 |
| 2-3 | Length | uint16 big-endian; counts Command + Payload |
| 4-5 | CRC | CRC-16/XMODEM, initial 0x0000, payload only, big-endian |
| 6-7 | Sequence ID | uint16 big-endian |
| 8 | Command | 0x01 heartbeat; 0x02 configuration update; 0x10 LTE-M location |
| 9+ | Payload | Command-specific |

Length is calculated as total packet bytes minus 8. CRC begins at payload offset 9 and does not include the command. All multi-byte numeric fields are big-endian.

# 3. Heartbeat: command 0x01

The corrected Version 7 heartbeat fields remain, followed by a mandatory complete configured trusted-device registry. This registry is separate from a currently detected beacon or trusted device.

| Payload field | Size | Encoding |
| --- | --- | --- |
| IMEI | 15 | ASCII decimal digits |
| Timestamp | 8 | uint64 UTC Unix milliseconds |
| Battery / isCharging | 1 / 1 | uint8 |
| lastUpdate | 2 | uint16 BE minutes |
| Software / firmware / opcode | 1 / 1 / 1 | uint8 |
| locationData | 6 or 12 | 6-byte beacon/trusted ID; or 12-byte GPS fields |
| trustedDeviceCount | 1 | 0..4 |
| trustedDeviceSlot1..4 | 24 | four fixed six-byte slots; unused slots are zero |

| Opcode | Meaning | locationData |
| --- | --- | --- |
| 0x01 | Fixed beacon detected | 6-byte fixed beacon MAC/address |
| 0x0A | GPS safe zone | Latitude + longitude + accuracy + speed; 12 bytes |
| 0x10 | GPS + LTE | Latitude + longitude + accuracy + speed; 12 bytes |
| 0xA0 | Trusted device detected | 6-byte trusted-device identity/address |

Canonical beacon/trusted heartbeats are 70 bytes with Length 00 3E. Canonical GPS heartbeats are 76 bytes with Length 00 44.

# 4. Device state machine and independent timers

Runtime simulation evaluates local context in this fixed order: configured detected beacon, then configured detected trusted device, then GNSS safe-zone evaluation. A beacon produces opcode 0x01, a trusted device produces 0xA0, GNSS inside a safe zone produces 0x0A, and GNSS outside all safe zones produces 0x10.

| Timer | Responsibility | Packet behavior |
| --- | --- | --- |
| bleCheckIntervalSeconds | Reevaluate local BLE context; escalate to GNSS only if neither configured BLE identity qualifies. | Does not itself send a heartbeat or LTE location. |
| heartbeatIntervalSeconds | Contact the backend in every state using the latest known state. | Sends command 0x01; server returns OK\n if no update is pending, otherwise a matching binary command-0x02 packet. |
| LTEupdateIntervalSeconds | Report fresh location while state is GPS_LTE_OUTSIDE. | Sends standalone command 0x10 on outside entry and periodically while still outside. |

The simulator starts in configuration-only mode. Runtime simulation is explicitly enabled with `simulation on`; `simulation off` stops all three automatic schedules without changing stored settings. Heartbeat expiry never forces BLE or GNSS reevaluation.

# 5. Configuration update: command 0x02

This is a positional full-replacement update. Validate the entire packet before changing persistent configuration. A zero count clears its corresponding list.

| Order | Field | Encoding |
| --- | --- | --- |
| 1 | Target IMEI | 15 ASCII decimal digits; must match the receiving device |
| 2 | Update ID | uint16 big-endian; stored after successful application |
| 3 | gpsSafeZoneCount | uint8 0..4 |
| 4 | safeZone[i] | int32 latitude e6 + int32 longitude e6 + uint16 radius metres |
| 5 | beaconCount / beacon[i] | uint8 0..4 followed by six-byte fixed beacon identifiers |
| 6 | trustedDeviceCount / trustedDevice[i] | uint8 0..4 followed by six-byte trusted identifiers |
| 7 | heartbeatIntervalSeconds | uint16 BE, 1..65535; one base heartbeat timer |
| 8 | LTEupdateIntervalSeconds | uint16 BE, 1..65535; active only while outside |
| 9 | bleCheckIntervalSeconds | uint16 BE, 1..65535; same two-byte position formerly named sleepIntervalSeconds |
| 10 | SendingUpdate | 1 byte: 00 = no firmware update; FF = firmware update indicated |

Payload = 27 + (10 x safe zones) + (6 x beacons) + (6 x trusted devices)
Length  = 1 + Payload
Total   = 8 + Length

The byte position, width, Length, CRC, and existing regression packet values are unchanged by the BLE-check semantic rename. `sleepIntervalSeconds` is a legacy local CLI alias only; canonical decoding and documentation use `bleCheckIntervalSeconds`.

# 6. Application and transport rules

Configuration is atomic. Bad CRC, Length, IMEI, count, field value, truncated data, invalid flag, or trailing bytes leave the previous persistent configuration unchanged.

The server queues updates by target IMEI. After a valid command-0x01 heartbeat, the server returns exactly one normal response: OK followed by newline (ASCII bytes 4F 4B 0A) when no matching configuration update is pending, or one complete binary command-0x02 configuration packet when an update is pending. No SUP token is used.

TCP is a byte stream. Implementations must buffer split tokens and split packets, retain buffered bytes after a complete packet, and report closure or timeout while a packet is incomplete.

Use binary packet transmission for integration. ASCII-HEX is diagnostic only and must not be confused with binary 0xAB framing.

# 7. Test settings

### Sample 1

AB100032EF0E00010238363133353230363430353037383700010102B513BCFB7CF3D00096010FAC91003B9101AABBCCDDEE01003C03FF01E000

| Setting | Value |
| --- | --- |
| Target IMEI / Update ID | 861352064050787 / 1 |
| GPS safe zones | 1: 45.421500, -75.697200, 150 m |
| Configured beacons | 1: 0FAC91003B91 |
| Configured trusted devices | 1: AABBCCDDEE01 |
| heartbeatIntervalSeconds | 60 seconds |
| LTEupdateIntervalSeconds | 1023 seconds |
| bleCheckIntervalSeconds | 480 seconds |
| SendingUpdate | 1 byte: 00 = no firmware update; FF = firmware update indicated |

### Sample 2

AB100048289A00020238363133353230363430353037383700020202B513BCFB7CF3D0009602B05747FB7D6B16012C020FAC91003B9100112233445502AABBCCDDEE01CCDDEEFFAABB007803FF0096FF

| Setting | Value |
| --- | --- |
| Target IMEI / Update ID | 861352064050787 / 2 |
| GPS safe zones | 2: 45.421500, -75.697200, 150 m; 45.111111, -75.666666, 300 m |
| Configured beacons | 2: 0FAC91003B91, 001122334455 |
| Configured trusted devices | 2: AABBCCDDEE01, CCDDEEFFAABB |
| heartbeatIntervalSeconds | 120 seconds |
| LTEupdateIntervalSeconds | 1023 seconds |
| bleCheckIntervalSeconds | 150 seconds |
| SendingUpdate | 1 byte: 00 = no firmware update; FF = firmware update indicated |

### Sample 3

AB100064869400030238363133353230363430353037383700030302B513BCFB7CF3D0009602B05747FB7D6B16012C02B2094EFB7BB90F012C030FAC91003B9100112233445511223344556604AABBCCDDEE01CCDDEEFFAABBAABBCCDDEE03BBCCDDEEFF0000B403FF0096FF

| Setting | Value |
| --- | --- |
| Target IMEI / Update ID | 861352064050787 / 3 |
| GPS safe zones | 3: 45.421500, -75.697200, 150 m; 45.111111, -75.666666, 300 m; 45.222222, -75.777777, 300 m |
| Configured beacons | 3: 0FAC91003B91, 001122334455, 112233445566 |
| Configured trusted devices | 4: AABBCCDDEE01, CCDDEEFFAABB, AABBCCDDEE03, BBCCDDEEFF00 |
| heartbeatIntervalSeconds | 180 seconds |
| LTEupdateIntervalSeconds | 1023 seconds |
| bleCheckIntervalSeconds | 150 seconds |
| SendingUpdate | 1 byte: 00 = no firmware update; FF = firmware update indicated |

### Sample 4

AB100074060A00040238363133353230363430353037383700040402B513BCFB7CF3D0009602B05747FB7D6B16012C02B2094EFB7BB90F012C02B58310FB7C73B000C8040FAC91003B910011223344551122334455660FAC91003B9404AABBCCDDEE01CCDDEEFFAABBAABBCCDDEE03BBCCDDEEFF00012C007803B6FF

| Setting | Value |
| --- | --- |
| Target IMEI / Update ID | 861352064050787 / 4 |
| GPS safe zones | 4: 45.421500, -75.697200, 150 m; 45.111111, -75.666666, 300 m; 45.222222, -75.777777, 300 m; 45.450000, -75.730000, 200 m |
| Configured beacons | 4: 0FAC91003B91, 001122334455, 112233445566, 0FAC91003B94 |
| Configured trusted devices | 4: AABBCCDDEE01, CCDDEEFFAABB, AABBCCDDEE03, BBCCDDEEFF00 |
| heartbeatIntervalSeconds | 300 seconds |
| LTEupdateIntervalSeconds | 120 seconds |
| bleCheckIntervalSeconds | 950 seconds |
| SendingUpdate | 1 byte: 00 = no firmware update; FF = firmware update indicated |

# 8. Plain-English operating description

The Micro tracker uses Bluetooth, GPS/GNSS, LTE-M, and periodic heartbeats together to decide whether the person appears to be in a known safe context and to keep the backend informed. The device has four main reported states inside its heartbeat. 01 means a configured fixed Bluetooth beacon is currently detected.

A0 means a configured trusted Bluetooth device is detected.

0A means Bluetooth did not establish the safe context, GPS was used, and the tracker is inside a configured GPS safe zone.

10 means the tracker used GPS and is outside its configured safe zones, so it is in the GPS/LTE tracking state.

These values are heartbeat opcodes, while the heartbeat packet itself uses command 0x01.

The local state machine starts with Bluetooth because it can establish the person's context without immediately needing GNSS. At each BLE check, the device first looks for one of its configured fixed beacons. If a recognized beacon is present, the current state becomes Beacon and the heartbeat will report opcode 01. If there is no recognized beacon, it looks for a configured trusted Bluetooth device. If one is present, the state becomes Trusted Device and the heartbeat reports A0. If neither Bluetooth condition succeeds, the device escalates to GNSS. It obtains a position and checks that position against the configured GPS safe zones. Inside a safe zone produces 0A; outside every safe zone produces 10.

The BLE check interval controls how quickly the tracker itself notices that the physical situation has changed. For example, with a 30-second BLE interval, the tracker can determine approximately every 30 seconds whether its home beacon or trusted phone is still nearby. That check is local; it does not inherently require a cellular transmission.

The heartbeat interval controls something different: how frequently the tracker contacts the backend even when the tracker itself believes everything is normal. A heartbeat reports the current state, device health and the complete configured trusted-device list. It also gives the server an opportunity to return a configuration update. If nothing needs changing, the server replies OK followed by newline. If an update is waiting for that IMEI, the server sends the complete binary command 0x02 configuration packet directly. The command byte already identifies the response as a configuration update, so no separate SUP token is used.

This heartbeat is particularly important when the local tracker cannot know that its definition of “safe” has changed. For example, suppose the person's phone is a trusted device and the person leaves while carrying that phone. The tracker continues detecting the trusted phone and therefore continues to believe the situation is safe. If someone reports the person missing through the backend, the server can queue a new configuration that removes the phone from the trusted-device list. The tracker receives that configuration at its next heartbeat. Therefore, a five-minute heartbeat interval means approximately a five-minute normal maximum wait for the tracker to ask the server for that pending change, assuming cellular connectivity is available. Actual tracking may take somewhat longer because the device still has to process the update, reevaluate its surroundings and obtain GNSS.

Configuration changes use packet command 0x02. This is a full-replacement configuration packet rather than a small individual-setting message. It contains the target IMEI, Update ID, GPS safe zones, fixed beacons, trusted devices and timing settings. A list count of zero clears that list, and the entire packet must validate before any persistent configuration changes.

That gives the backend a way to implement a lost-person workflow without reflashing the device. For example, it could send a new 0x02 configuration with no trusted devices and no GPS safe zones. If the intention is to remove all conditions that can suppress tracking, the fixed beacon list must also be cleared because the normal state machine checks recognized beacons first. After the update is applied, the device should immediately reevaluate its state rather than waiting another complete BLE interval.

Once the tracker determines that it is outside the safe context, the LTE update interval becomes important. This controls how frequently it obtains and reports a fresh position while actively tracking. For example, an LTE interval of 60 seconds could result in approximately one fresh location update every minute while the tracker remains outside. The standalone location packet uses packet command 0x10. This must not be confused with heartbeat opcode 0x10, which simply tells the backend that the heartbeat's current state is GPS + LTE. The canonical V7 protocol defines both command uses separately.

In short, the three timers answer three different questions: BLE interval: “How quickly should the tracker notice its environment changed?” Heartbeat interval: “How frequently should the tracker check in with the backend and give the backend a chance to intervene?” LTE interval: “Once active tracking is required, how frequently should a new location be reported?” Keeping those three responsibilities separate allows fast local detection, reliable remote intervention, and aggressive location tracking when necessary without requiring constant cellular communication during normal safe operation.

# 9. TCP server response workflow

Canonical heartbeat response rule. Every valid command-0x01 heartbeat receives exactly one normal server response. If no configuration change is pending for the heartbeat IMEI, the server sends the three ASCII bytes 4F 4B 0A, which are the text OK followed by newline (OK\n). If a configuration change is pending, the server sends one complete binary Micro packet whose Command byte at offset 8 is 0x02. The binary configuration packet starts with the normal AB 10 envelope and is sent directly; there is no SUP prefix or other announcement token.

Why the two responses are sufficient. OK\n confirms that the backend received and processed the heartbeat and that there is no new configuration to apply. A command-0x02 packet is self-identifying: its AB/10 envelope, Length field, CRC, Sequence ID and Command byte already tell the device that a complete configuration update is being delivered. A separate SUP token would duplicate information already present in the 0x02 packet and is therefore not part of the canonical Version 8 flow.

| Condition | Server bytes | Meaning | Device action |
| --- | --- | --- | --- |
| Valid heartbeat; no configuration pending | 4F 4B 0A (OK\n) | Heartbeat accepted; no configuration change | Continue normal operation |
| Valid heartbeat; configuration pending | AB 10 ... with Command byte 0x02 at offset 8 | Complete Version 8 full-replacement configuration | Buffer by Length, validate CRC/IMEI/fields, atomically persist/apply, reschedule affected timers, reevaluate state |
| Invalid/rejected inbound packet | ERROR\n or ERROR:<code>\n | Packet was not accepted | Log error; do not change persistent configuration |

TCP stream requirement. TCP receive boundaries are not packet boundaries. The receiver must preserve partial OK/error tokens and partial binary packets across reads. For a binary Micro packet, use the big-endian Length field to determine the complete frame size: total bytes = 8 + Length. A command-0x02 packet may arrive in one read or many reads; it must not be applied until the complete packet has been received and validated.

SendingUpdate field. The final SendingUpdate byte inside command 0x02 is not a settings-delivery flag. 0x00 means no firmware update is indicated. 0xFF means a firmware update is indicated. Either value is valid for a settings packet and does not change whether the configuration itself is validated and applied.

Firmware update boundary. The firmware-update transfer protocol still needs to be designed. Version 8 currently defines only the 0x00/0xFF indication in SendingUpdate. It does not yet define the firmware image source, image version/manifest, authentication or signature verification, integrity checks, chunking, transfer retries/resume, installation, rollback, reboot behavior, or final confirmation. Those items must be specified separately before firmware delivery is implemented.

Server implementation summary. A TCP server should validate each received heartbeat, identify the device by IMEI, check whether a complete configuration is pending for that IMEI, then send either OK\n or the complete binary command-0x02 packet. The server should not send both for one heartbeat. The configuration packet must use the exact Version 8 positional payload and binary framing defined above. After sending an update, server-side delivery state may record that the packet was transmitted, but transmission alone does not prove that the device successfully persisted it because Version 8 does not currently define a device-to-server configuration-application acknowledgement.