MICRO SIMULATOR

# Corrected Packet Protocol
and Validation Guide

Version 8 - Canonical state machine, independent timers, mandatory trusted-device heartbeat registry, and full-replacement configuration

Applies to nRF9151 DK + Onomondo SoftSIM simulator

Status: Version 8 compatibility revision. The Version 8 canonical state-machine/settings document is the source of truth. This guide retains the Version 7 big-endian envelope, CRC, timestamps, and TCP framing, but replaces command-0x20 TLV settings with positional command 0x02 and makes the configured trusted-device registry mandatory in every heartbeat.

Companion files

main.c | micro_settings.c/.h | micro_protocol.py | micro_packet_decoder.py | micro_tcp_server.py | micro_update_tool.py | automated tests

Contents

1. Purpose and outcome	4

1.1 Deliverables in this correction	4

1.2 Decisions used in this revision	4

2. Packet basics: HEX, bytes, offsets, and endianness	6

2.1 Bit, byte, and hexadecimal	6

2.2 Byte offsets	6

2.3 Endianness	6

2.4 ASCII inside a binary packet	7

3. Canonical packet envelope	8

3.1 Length calculation	8

4. Startup behavior	9

5. Command reference	9

6. Windows relay framing and bidirectional responses	11

7. Copy-paste test sequences	13

7.1 Relay mode - no SoftSIM or no LTE-M coverage	13

7.2 Direct LTE mode	13

7.3 State simulations	13

7.4 Custom state	13

7.5 Binary parser test	13

8. CRC definition and implementation	14

8.1 Algorithm identity	14

8.2 Coverage and byte order	14

8.3 Supplied CRC routine	14

8.4 Correct simulator sequence	14

9. Heartbeat packet: command 0x01	15

10. LTE-M location packet: command 0x10	16

11. Latitude, longitude, accuracy, and speed	17

11.1 Coordinate conversion	17

11.2 Accuracy and speed	17

12. Corrected known-good test vectors	18

12.1 Heartbeat - beacon	18

12.2 Heartbeat - GPS + LTE	18

12.3 LTE-M location	18

13. Discrepancies found in Version 4	19

13.1 Old heartbeat beacon sample diagnostic	19

9.2 Old location sample diagnostic	20

14. Simulator functionality review	21

14.1 What is structurally sound	21

14.2 Corrections made in main.c	21

14.3 Remaining simulator limitations	21

15. Using the packet decoder	22

15.1 Basic use	22

15.2 CRC self-test	22

15.3 What the decoder validates	22

15.4 Automated tests	22

16. Timestamp specification and simulation	23

16.1 Default deterministic timestamp	23

16.2 Simulator time modes	23

16.3 Timestamp CLI commands	23

16.4 Converting human-readable time	24

16.5 Timestamp test coverage	24

17. Why Version 8 retains one byte order	25

18. TCP server framing and the leading AB	26

18.3 Recommended integration mode	26

18.4 Running the updated server	26

18.5 Server response modes	26

18.6 Server automated tests	26

19. Extended heartbeat trusted-device registry	27

Known-good extended beacon heartbeat (one configured trusted device):	27

19.1 Device state machine and independent timers	27

20. Configuration update: command 0x02	28

Canonical full-replacement example (Sample 1)	29

Compatibility note	29

Device serial commands	29

Inspect the current configuration	29

Change one value through the Version 8 full-replacement convenience path	29

Update static beacons (local convenience; generated wire packet is still a full replacement)	30

Update trusted-device identifiers (local convenience; generated wire packet is still a full replacement)	30

Update safe zones	31

Apply an already generated Version 8 packet	31

21. Server responses and settings application	32

22. Decoder and test additions	32

# 1. Purpose and outcome

The simulator is intended to generate deterministic heartbeat and LTE-M location packets so the receiving team can build and verify its parser before production firmware is complete. Version 4 contained conflicting packet definitions, malformed fixed samples, a placeholder CRC, and a length calculation that did not match the documented packet envelope.

Outcome: Version 8 retains the Version 7 all-big-endian packet envelope while adding the canonical four-state evaluation order, three independent schedules, mandatory configured trusted-device heartbeat registry, and atomic positional full-replacement command-0x02 configuration.

## 1.1 Deliverables in this correction

- A Zephyr simulator implementation that retains the Version 7 big-endian envelope/CRC rules and adds the Version 8 state machine, independent timers, mandatory trusted-device heartbeat registry, and full-replacement configuration flow.

- A standalone Python decoder that accepts pasted HEX and prints every field, byte offset, decoded value, length validation, and CRC validation.

- Automated tests covering CRC and big-endian framing, Version 8 heartbeat lengths and trusted-device slots, command-0x02 full-replacement configuration, wrong-IMEI/length/CRC rejection, state-machine priority, independent timers, timestamps, coordinates, and TCP stream fragmentation.

- This Version 8-compatible protocol and usage guide, including HEX/byte interpretation, the retained big-endian envelope, state-machine behavior, independent BLE/heartbeat/LTE timers, mandatory trusted-device registry, positional configuration, UTC Unix-millisecond timestamps, binary versus ASCII-HEX transport, and server-side stream framing.

## 1.2 Decisions used in this revision

| Decision | Canonical rule | Basis / status |
| --- | --- | --- |
| Length | uint16 big-endian; counts Command + Payload | Retained unchanged in Version 8. |
| CRC algorithm | CRC-16/XMODEM, initial value 0x0000 | Exact result of the supplied C# routine. |
| CRC coverage | Payload only; bytes after Command | Meeting statement: CRC is calculated on the payload. |
| CRC byte order | Big-endian in bytes 4-5 | Version 7 protocol-wide byte-order rule. The CRC algorithm defines the numeric result; the protocol defines its byte serialization. |
| Sequence ID | uint16 big-endian | Version 7 protocol-wide byte-order rule. |
| Timestamp | uint64 big-endian; Unix milliseconds since 1970-01-01T00:00:00Z; UTC; 0 = unavailable | Version 7 canonical rule; heartbeat = state-sample time; location = GNSS-fix time. |
| Coordinates | signed int32 big-endian, degrees x 1,000,000 | Old coordinate bytes decode to valid -90 and -180 only at x1,000,000. |
| Accuracy / speed | uint16 big-endian, value x10 | Both quantities are non-negative; unsigned encoding is appropriate. |

Breaking change: Length, CRC, and Sequence ID are now big-endian. Update the receiving parser and replace every Version 6 fixed vector.

# 2. Packet basics: HEX, bytes, offsets, and endianness

You do not need to work directly in binary to understand this protocol. The packet is shown in hexadecimal because HEX is a compact way to display bytes.

## 2.1 Bit, byte, and hexadecimal

| Concept | Meaning | Example |
| --- | --- | --- |
| Bit | One binary value: 0 or 1 | 1 |
| Byte | Eight bits | 10101011 |
| HEX digit | Represents four bits; values 0-9 and A-F | A = decimal 10 |
| HEX byte | Two HEX digits represent one byte | AB = binary 10101011 = decimal 171 |

Counting rule: A packet string containing 90 HEX characters contains 45 bytes, because each byte is represented by two HEX characters.

## 2.2 Byte offsets

Offsets in this document are zero-based. Byte 0 is the first byte. Therefore, the ninth byte is byte offset 8. The command must be at byte 8.

## 2.3 Endianness

Endianness is the order used to transmit a multi-byte number. It does not change the number; it changes which byte is sent first. Version 8 retains the Version 7 rule: every multi-byte integer in the Micro packet uses big-endian/network byte order. This removes mixed byte-order assumptions and keeps the receiver rule consistent.

| Number | Little-endian bytes | Big-endian bytes | Version 8 retained use |
| --- | --- | --- | --- |
| 0x0025 = decimal 37 | 25 00 | 00 25 | Length uses big-endian: 00 25 |
| 0x0001 = decimal 1 | 01 00 | 00 01 | Sequence ID uses big-endian: 00 01 |
| 0x07FF = decimal 2047 | FF 07 | 07 FF | lastUpdate uses big-endian: 07 FF |

## 2.4 ASCII inside a binary packet

The IMEI is not transmitted as one large integer. Each digit is transmitted as its ASCII byte. For example, the first digit "8" is byte 0x38, digit "6" is 0x36, and digit "1" is 0x31.

IMEI text: 861352064050787
HEX bytes: 38 36 31 33 35 32 30 36 34 30 35 30 37 38 37

Why the old packet showed command 0x38: 0x38 is the ASCII byte for the digit 8. In the old heartbeat sample, the IMEI began at the command position, proving that the command/header fields were shifted.

Every packet has:  Header + Property + Length + CRC + Sequence ID + Command + Payload

| Byte offset | Size | Field | Encoding |
| --- | --- | --- | --- |
| 0 | 1 | Header | Fixed 0xAB |
| 1 | 1 | Property | Fixed 0x10 |
| 2–3 | 2 | Length | Unsigned 16-bit, big-endian |
| 4–5 | 2 | CRC | CRC-16/XMODEM, big-endian |
| 6–7 | 2 | Sequence ID | Unsigned 16-bit, big-endian |
| 8 | 1 | Command | 0x01 heartbeat; 0x02 configuration update; 0x10 location |
| 9 onward | Variable | Payload | Command-specific data |

# 3. Canonical packet envelope

AB | 10 | LENGTH_BE | CRC_BE | SEQUENCE_BE | COMMAND | PAYLOAD
 1    1       2          2          2           1      variable bytes

| Absolute byte(s) | Field | Size | Encoding / rule |
| --- | --- | --- | --- |
| 0 | Header | 1 | Constant 0xAB |
| 1 | Property | 1 | Constant 0x10 |
| 2-3 | Length | 2 | uint16 big-endian; number of bytes from Command through end of Payload |
| 4-5 | CRC | 2 | CRC-16/XMODEM over Payload only; stored big-endian |
| 6-7 | Sequence ID | 2 | uint16 big-endian |
| 8 | Command | 1 | 0x01 heartbeat; 0x02 configuration update; 0x10 LTE-M location |
| 9+ | Payload | variable | Command-specific fields |

## 3.1 Length calculation

The length field begins at byte 2, but it does not count the fixed bytes before Command. It is calculated after the packet is built:

length = total_packet_bytes - 8
       = 1 command byte + payload bytes

| Packet | Total bytes | Command + Payload | Length bytes |
| --- | --- | --- | --- |
| LTE-M location | 48 | 40 decimal = 0x0028 | 00 28 |
| Extended heartbeat with beacon/trusted address | 70 | 62 decimal = 0x003E | 00 3E |
| Extended heartbeat with GPS fields | 76 | 68 decimal = 0x0044 | 00 44 |

Version 4 code defect: finish_packet_length() used len - 4. That incorrectly counted CRC and Sequence ID. The correct expression is len - 8.

# 4. Startup behavior

| Condition at boot | Firmware action |
| --- | --- |
| SoftSIM provisioned | Select LTE transport, prepare the modem, start asynchronous LTE registration, and immediately expose the CLI. Runtime simulation still remains off until `simulation on`. |
| SoftSIM not provisioned | Select relay transport, skip modem startup, immediately expose the CLI, and allow softsim provision later. Runtime simulation still remains off until `simulation on`. |
| LTE is unavailable | The CLI remains usable. Switch to transport relay and use the Windows relay. Runtime simulation can be enabled independently with `simulation on`. |
| All boots | Load persistent configuration and expose the CLI in configuration-only mode. Automatic BLE, heartbeat, and LTE-location schedules remain stopped until `simulation on`. |

Recommended first command (to connect to the DigitalOcean Server) : connect 137.184.163.176 5000

# 5. Command reference

| Category | Command | Behavior |
| --- | --- | --- |
| General | help | Print supported commands. |
| General | status | Show transport, SoftSIM/LTE state, endpoint, socket/relay state, wire mode, operation mode, current tracking state, detected BLE context, configured lists, and all three timer states. |
| Transport | transport lte | Select direct modem TCP. Requires provisioned SoftSIM and LTE registration. |
| Transport | transport relay | Select Windows serial/Wi-Fi relay. Direct LTE socket is closed. |
| SoftSIM | softsim status | Show whether a profile is provisioned. |
| SoftSIM | softsim provision | Read and provision a complete SoftSIM HEX profile from serial, then reboot. |
| LTE | lte status | Show modem/LTE state. |
| LTE | lte connect | Prepare modem if needed and start asynchronous LTE registration. |
| Connection | connect [ip] [port] | LTE mode opens a modem TCP socket. Relay mode sends a connection request to the Windows relay. |
| Connection | disconnect | Close the active LTE or relay TCP connection. |
| Connection | recv | Poll for a response in LTE mode. Relay responses return automatically. |
| Wire | mode ascii_hex | Send packet HEX as printable ASCII characters. |
| Wire | mode binary | Convert packet HEX to raw bytes before TCP transmission. |
| Samples | send sample_hb_beacon | Send exact beacon heartbeat sample. |
| Samples | send sample_hb_gps | Send exact GPS/LTE heartbeat sample. |
| Samples | send sample_location | Send exact LTE location sample. |
| Dynamic | send hb | Build heartbeat from current state and send it. |
| Dynamic | send loc | Build location packet from current state and send it. |
| Raw | send raw <HEX> | Send arbitrary HEX without reflashing. |
| Config | config show | Show persistent settings, ranges, and defaults. |
| Config | config generate <setting_name> <value> | Generate and print a complete Version 8 command-0x02 full-replacement packet using the active configuration plus the requested local override; do not apply it. |
| Config | config apply <complete_packet_hex> | Validate, atomically apply, persist, reschedule affected timers, and immediately request state reevaluation for a complete command-0x02 packet. |
| Config | config set <setting_name> <value> | Local convenience: build a complete command-0x02 packet from the active configuration plus one override, then route it through the normal validator/application path. |
| Config | config reset confirm | Restore documented persistent defaults. |
| Config | config last | Show the last update ID, source, result, changed settings, and validation failure. |
| Simulation | simulation status¦on¦off | Inspect or control runtime simulation. `on` schedules BLE checks and heartbeats and reevaluates state immediately; `off` stops all three automatic schedules without changing stored settings. |

HOW TO EDIT / SIMULATE DEVICE STATE (VERSION 8).

| Command | Effect |
| --- | --- |
| preset normal | Medium battery, not charging; detected default beacon and trusted device are both present. In runtime mode the configured-beacon priority resolves to Beacon when the default beacon is configured. |
| preset low | Low battery, not charging. BLE/GNSS context is otherwise unchanged; runtime state is still derived by the state machine. |
| preset charging | Medium battery, charging. BLE/GNSS context is otherwise unchanged; runtime state is still derived by the state machine. |
| preset outside | Clear detected beacon/trusted device and set 45.450000, -75.750000, 30.0 m, 1.2 m/s. Runtime reevaluation uses GNSS safe-zone evaluation; the default scenario resolves to GPS_LTE_OUTSIDE. |
| preset beacon | Mark the default beacon detected and trusted device absent; request runtime reevaluation. |
| preset trusted | Mark the default beacon absent and trusted device detected; request runtime reevaluation. |
| set battery low¦medium¦high | Set battery byte. |
| set charging on¦off | Set charging byte. |
| set opcode beacon¦trusted¦gps_lte¦gps_safezone | Diagnostic-only direct opcode/state override. Runtime simulation derives the state again on the next BLE-context reevaluation. |
| set pos <lat> <lon> <accuracy_m> <speed_mps> | Set simulated location data. |
| set imei <15_digits> | Set the transmitted 15-digit IMEI. |
| set lastupdate <minutes> | Set the configuration-age field. |
| set versions <sw> <fw> | Set software and firmware version bytes. |
| set beacon <12_hex_chars>¦off | Set or clear the currently detected six-byte beacon identity; request runtime state reevaluation. |
| set trusted <12_hex_chars>¦off | Set or clear the currently detected six-byte trusted-device identity; request runtime state reevaluation. |

| Normal profile preset normal Sets medium battery, not charging, detected default beacon + trusted device. V8 runtime result: configured beacon wins priority -> opcode 0x01. Typical use: simulation on preset normal send hb | Low battery profile preset low Sets low battery, not charging; detection context is unchanged. Typical use: simulation on preset normal preset low send hb | Charging profile preset charging Sets medium battery, charging; detection context is unchanged. Typical use: simulation on preset normal preset charging send hb | Outside profile preset outside Clears BLE detections and sets 45.450000, -75.750000, 30.0 m, 1.2 m/s. V8 runtime result: GNSS safe-zone check; default scenario -> GPS_LTE_OUTSIDE / opcode 0x10 and outside LTE-location schedule. | Beacon profile preset beacon Detected configured beacon present; trusted device absent. V8 runtime result: Beacon / opcode 0x01. Typical use: simulation on preset beacon send hb | Trusted-device profile preset trusted Detected beacon absent; trusted device present. V8 runtime result: Trusted Device / opcode 0xA0 when the detected ID is configured. Typical use: simulation on preset trusted send hb |
| --- | --- | --- | --- | --- | --- |

# 6. Windows relay framing and bidirectional responses

The firmware and Windows relay exchange control lines over USB serial. These markers are internal to the relay path and are not sent as Micro protocol payloads to the TCP server.

| Direction | Serial marker | Meaning |
| --- | --- | --- |
| Board -> Windows | MICRO_RELAY_CONNECT:<host>:<port> | Open or change the persistent server connection. |
| Board -> Windows | MICRO_RELAY_DISCONNECT | Close the relay TCP connection. |
| Board -> Windows | MICRO_RELAY_TX_ASCII_HEX:<HEX> | Send the printable HEX string to the server. |
| Board -> Windows | MICRO_RELAY_TX_BINARY_HEX:<HEX> | Convert HEX to bytes and send binary data. |
| Windows -> Board | MICRO_RELAY_CONNECTED:<host>:<port> | Confirm TCP connection. |
| Windows -> Board | MICRO_RELAY_RX_HEX:<HEX> | Return any server response as serial-safe HEX. |
| Windows -> Board | MICRO_RELAY_RX_EMPTY | No response arrived before the relay timeout. |
| Windows -> Board | MICRO_RELAY_RX_ERROR:<text> | Report TCP or parsing failure. |

Example: a server response of OK followed by newline is returned as 4F4B0A. The firmware decodes the HEX and prints the readable response on the board console.

# 7. Copy-paste test sequences

## 7.1 Relay mode - no SoftSIM or no LTE-M coverage

Terminal: Python relay console

transport relay connect 137.184.163.176 5000 mode binary send sample_hb_beacon send sample_hb_gps send sample_location status

## 7.2 Direct LTE mode

Terminal: nRF Terminal or Python relay console

softsim status transport lte lte connect lte status connect 137.184.163.176 5000 mode binary send sample_hb_beacon

## 7.3 State simulations

simulation on preset normal send hb preset low send hb preset charging send hb preset beacon send hb preset trusted send hb preset outside send hb status

## 7.4 Custom state

simulation on set imei 861352064050787 set battery low set charging on set beacon off set trusted off set pos 45.4215 -75.6972 25.5 0.0 status send hb send loc

## 7.5 Binary parser test

mode binary send sample_hb_beacon simulation on send hb send loc mode ascii_hex

# 8. CRC definition and implementation

CRC is an error-detection value. The sender calculates it from the payload and inserts the result into bytes 4-5. The receiver performs the same calculation and compares the result. A mismatch means the packet content, packet boundaries, coverage range, algorithm, initial value, or byte order differs.

## 8.1 Algorithm identity

| Parameter | Value |
| --- | --- |
| Common name | CRC-16/XMODEM |
| Polynomial | 0x1021 |
| Initial CRC | 0x0000 |
| Reflect input/output | false / false |
| Final XOR | 0x0000 |
| Check vector | ASCII "123456789" -> 0x31C3 |

## 8.2 Coverage and byte order

CRC input = Payload only = packet bytes 9 through final byte
CRC does NOT include Header, Property, Length, CRC field, Sequence ID, or Command.
CRC field bytes = high byte first, then low byte (big-endian).

Example: If the numeric CRC result is 0x69FD, the packet contains bytes 69 FD because Version 7 stores the field big-endian.

## 8.3 Supplied CRC routine

ushort CalculateCRC16(byte[] data, ushort? initialCrc = null)
{
    ushort crc = initialCrc ?? 0x0000;
    for (int i = 0; i < data.Length; i++)
    {
        crc = (ushort)(((byte)(crc >> 8)) | (crc << 8));
        crc ^= data[i];
        crc ^= (ushort)(((byte)(crc & 0xFF)) >> 4);
        crc ^= (ushort)((crc << 8) << 4);
        crc ^= (ushort)((((crc & 0xFF) << 4)) << 1);
    }
    return crc;
}

## 8.4 Correct simulator sequence

- Write Header, Property, zero placeholders for Length and CRC, Sequence ID, and Command.

- Append the full command-specific payload.

- Calculate Length as total bytes minus 8 and write it big-endian.

- Calculate CRC over bytes beginning at payload offset 9.

- Write the CRC high byte into byte 4 and low byte into byte 5.

# 9. Heartbeat packet: command 0x01

The Version 8 heartbeat communicates device identity, state-sample timestamp, power/configuration state, one opcode-selected location/status source, and the complete configured trusted-device registry. The configured registry is mandatory and is separate from a trusted device that is currently detected.

| Absolute byte(s) | Payload field | Size | Encoding |
| --- | --- | --- | --- |
| 9-23 | IMEI | 15 | Exactly 15 ASCII decimal digits |
| 24-31 | Timestamp | 8 | uint64 big-endian Unix milliseconds; UTC; 0 means unavailable; heartbeat state-sample time |
| 32 | Battery level | 1 | 00 low, 01 medium, 10 high |
| 33 | isCharging | 1 | 01 not charging, 10 charging |
| 34-35 | lastUpdate | 2 | uint16 big-endian, minutes |
| 36 | Software version | 1 | uint8 |
| 37 | Firmware version | 1 | uint8 |
| 38 | Opcode | 1 | Determines the final locationData format |
| 39+ | locationData | 6 or 12 | Address or GPS fields |
| after locationData | trustedDeviceCount | 1 | uint8 0..4; number of configured trusted devices |
| following count | trustedDeviceSlot1..4 | 24 | Four fixed six-byte slots; configured entries first, unused slots all zero |

| Opcode | Meaning | locationData |
| --- | --- | --- |
| 0x01 | Beacon detected | 6-byte fixed beacon MAC/address |
| 0x0A | GPS safe zone | Latitude + longitude + accuracy + speed (12 bytes) |
| 0x10 | GPS + LTE | Latitude + longitude + accuracy + speed (12 bytes) |
| 0xA0 | Trusted device detected | 6-byte trusted-device identity/address |

Length result: Version 8 beacon/trusted heartbeat is 70 total bytes and uses Length 00 3E. Version 8 GPS heartbeat is 76 total bytes and uses Length 00 44. The extra 25 bytes are part of the payload-only CRC.

# 10. LTE-M location packet: command 0x10

The standalone location packet contains the same identity and power/configuration fields, followed directly by latitude, longitude, accuracy, and speed. It does not include heartbeat software version, firmware version, or opcode fields.

| Absolute byte(s) | Payload field | Size | Encoding |
| --- | --- | --- | --- |
| 9-23 | IMEI | 15 | Exactly 15 ASCII decimal digits |
| 24-31 | GNSS fix timestamp | 8 | uint64 big-endian Unix milliseconds; UTC; 0 means unavailable; coordinate measurement time |
| 32 | Battery level | 1 | 00 low, 01 medium, 10 high |
| 33 | isCharging | 1 | 01 not charging, 10 charging |
| 34-35 | lastUpdate | 2 | uint16 big-endian, minutes |
| 36-39 | Latitude | 4 | signed int32 big-endian, degrees x 1,000,000 |
| 40-43 | Longitude | 4 | signed int32 big-endian, degrees x 1,000,000 |
| 44-45 | Accuracy | 2 | uint16 big-endian, metres x 10 |
| 46-47 | Speed | 2 | uint16 big-endian, metres/second x 10 |

Length result: The corrected location packet is 48 total bytes. Command + Payload is 40 bytes, so Length is 00 28.

Version 4 used a 9-byte timestamp. Version 7 uses exactly 8 bytes for the uint64 Unix-millisecond timestamp.

# 11. Latitude, longitude, accuracy, and speed

## 11.1 Coordinate conversion

Coordinates are converted from decimal degrees to signed integers before transmission:

encoded_coordinate = round_or_parse(decimal_degrees x 1,000,000)
decoded_coordinate = signed_int32 / 1,000,000

| Value | Scaled integer | Big-endian bytes |
| --- | --- | --- |
| Latitude 45.421500 | 45,421,500 | 02 B5 13 BC |
| Longitude -75.697200 | -75,697,200 | FB 7C F3 D0 |
| Latitude -90.000000 | -90,000,000 | FA A2 B5 80 |
| Longitude -180.000000 | -180,000,000 | F5 45 6B 00 |

Negative values use two's-complement signed integer encoding. The bytes do not visibly contain a minus sign; the decoder must read them as signed int32.

Validation: The corrected simulator rejects latitude outside -90..90 and longitude outside -180..180. Version 4 parsed values but did not enforce geographic limits.

## 11.2 Accuracy and speed

accuracy_bytes = uint16_big_endian(accuracy_metres x 10)
speed_bytes    = uint16_big_endian(speed_metres_per_second x 10)

| Human value | Stored integer | Bytes |
| --- | --- | --- |
| 25.5 m accuracy | 255 | 00 FF |
| 0.0 m/s speed | 0 | 00 00 |
| 1.2 m/s speed | 12 | 00 0C |

Documentation correction: Version 4 described these as unsigned in one section and signed in another. They are non-negative physical quantities and are now consistently defined as uint16.

# 12. Corrected known-good test vectors

The Version 8 canonical heartbeat vectors below use Sequence ID 1, fixed UTC timestamp 2026-07-21T13:30:00.000Z (1784640600000 Unix ms; bytes 00 00 01 9F 84 DE 77 C0), one configured trusted device (AABBCCDDEE01), and big-endian serialization for every multi-byte field. The LTE-M location vector retains the same Version 7 envelope and payload layout.

## 12.1 Heartbeat - beacon

AB10003E621D0001013836313335323036343035303738370000019F84DE77C0010107FF0101010FAC91003B9101AABBCCDDEE01000000000000000000000000000000000000

| Check | Expected |
| --- | --- |
| Total bytes | 70 |
| Length | 0x003E -> 00 3E |
| CRC numeric | 0x621D |
| CRC bytes | 62 1D |
| Command | 01 |
| Opcode | 01 beacon |
| trustedDeviceCount | 1; slot1=AABBCCDDEE01; slots2..4=zero |

## 12.2 Heartbeat - GPS + LTE

AB10004464B10001013836313335323036343035303738370000019F84DE77C0010107FF01011002B513BCFB7CF3D000FF000001AABBCCDDEE01000000000000000000000000000000000000

| Check | Expected |
| --- | --- |
| Total bytes | 76 |
| Length | 0x0044 -> 00 44 |
| CRC numeric | 0x64B1 |
| CRC bytes | 64 B1 |
| Command | 01 |
| Opcode | 10 GPS + LTE |
| trustedDeviceCount | 1; slot1=AABBCCDDEE01; slots2..4=zero |

## 12.3 LTE-M location

AB10002867C80001103836313335323036343035303738370000019F84DE77C0010107FF02B513BCFB7CF3D000FF0000

| Check | Expected |
| --- | --- |
| Total bytes | 48 |
| Length | 0x0028 -> 00 28 |
| CRC numeric | 0x67C8 |
| CRC bytes | 67 C8 |
| Command | 10 |
| Latitude / longitude | 45.421500 / -75.697200 |

# 13. Discrepancies found in Version 4

| Issue | What was wrong | Correction |
| --- | --- | --- |
| Dynamic length calculation | Code used total length minus 4, which included CRC and Sequence ID. | Use total length minus 8; count Command + Payload only. |
| CRC | Dynamic packets inserted placeholder bytes D5 00; fixed samples used unrelated D5 E6 / 26 A1 values. | Calculate CRC-16/XMODEM over Payload and write it big-endian. |
| Heartbeat sample command | Byte 8 was 0x38, the ASCII digit 8 and first IMEI byte. A 0x01 appeared at byte 7. | Two-byte Sequence ID at bytes 6-7; Command 0x01 at byte 8; IMEI starts byte 9. |
| GPS heartbeat length | Old field was 1E 00 = 30, but actual Command + Payload was 43 bytes. | Version 7 base heartbeat used 00 2B. Version 8 adds the mandatory 25-byte trusted-device registry, so the canonical GPS heartbeat uses 00 44. |
| Location length | Old field was 10 00 = 16, but actual Command + Payload occupied 40 bytes. | Version 7 uses 00 28. |
| Location command placement | Byte 8 was 0x0F; valid command 0x10 appeared at byte 9, and IMEI began at byte 10. | Command must be at byte 8 and IMEI at byte 9. |
| Location timestamp | Dynamic builder appended 9 bytes while the field was described as uint64. | Use exactly 8 bytes. |
| Coordinate documentation | Text mentioned x100,000 while sample boundary coordinates only made sense at x1,000,000. | Use x1,000,000 consistently. |
| Accuracy/speed signedness | One section said unsigned and another said signed. | Use uint16 because values cannot be negative. |
| Fixed sample strategy | Documentation recommended old fixed samples even though they conflicted with the dynamic layout. | Use vectors generated from the canonical builder. Version 8 canonical heartbeat vectors include the mandatory registry; legacy short Version 7 heartbeat vectors are non-canonical for V8 integration. |

## 13.1 Old heartbeat beacon sample diagnostic

AB102500D5E6230138363133353230363430353037383719EDB8266D50000000001007FF0101010FAC91003B91

- Length 25 00 happens to equal the actual 37 bytes after byte 7.

- CRC bytes D5 E6 do not match the supplied algorithm over the payload.

- Sequence bytes decode as 23 01, and byte 8 is 38 rather than a valid command.

- The 15-digit IMEI begins at byte 8 instead of byte 9.

## 9.2 Old location sample diagnostic

AB10100026A1B2B50F1038363133353230363430353037383700506D26B8ED1900007FFFFAA2B580F5456B000FFF0FFF

- Length 10 00 declares 16 bytes, while the packet actually contains 40 Command + Payload bytes.

- CRC bytes 26 A1 do not match the supplied algorithm over the payload.

- Byte 8 is 0F, not a defined command. Command 10 appears one byte late at byte 9.

- The IMEI begins at byte 10, demonstrating an inconsistent header/sequence width.

# 14. Simulator functionality review

## 14.1 What is structurally sound

- The simulator cleanly separates direct LTE TCP and Windows serial/Wi-Fi relay transports.

- SoftSIM provisioning, asynchronous LTE registration, persistent TCP connection, and serial CLI are reasonable for integration testing.

- The state-editing commands make it possible to test battery, charging, beacon, trusted-device, GPS, IMEI, versions, and position changes without reflashing.

- The coordinate input parser correctly handles negative decimal values and stores six decimal places.

- Binary mode transmits actual packet bytes; ASCII-HEX mode remains useful for human-readable server logs.

## 14.2 Corrections made in main.c

- Implemented the supplied CRC routine as CRC-16/XMODEM and retained the startup self-test using 123456789 -> 0x31C3.

- Changed packet finalization to calculate Length as len - 8 and CRC over bytes 9 through end.

- Changed Length, CRC, and Sequence ID serialization to big-endian so every multi-byte field uses one byte order.

- Replaced the heartbeat fixed samples with the Version 8 canonical mandatory-registry vectors (70-byte beacon/trusted and 76-byte GPS heartbeats); the 48-byte LTE-M location vector remains unchanged.

- Changed LTE-M location timestamp from 9 bytes to the canonical 8-byte uint64 big-endian Unix-millisecond field.

- Added latitude and longitude range validation.

- Added Version 8 configuration-only startup plus explicit simulation status/on/off control and independent heartbeat, BLE-context, and outside-LTE location schedules.

- Added Version 8 positional full-replacement configuration application, timer rescheduling, and immediate state reevaluation after a successful configuration change.

## 14.3 Remaining simulator limitations

| Limitation | Impact |
| --- | --- |
| No real UTC/GNSS time source | The simulator generates controlled timestamps; it does not prove modem/GNSS clock acquisition or RTC retention. |

- The default wire mode is now binary so integration servers receive the actual header byte 0xAB. ASCII-HEX is still available as an explicit diagnostic mode.

# 15. Using the packet decoder

micro_packet_decoder.py is a dependency-free Python command-line tool. For Version 8 integration it must validate canonical mandatory-registry heartbeats, LTE-M location packets, and positional command-0x02 configuration packets; legacy packets may be diagnosed separately but are not Version 8 canonical traffic.

## 15.1 Basic use

python micro_packet_decoder.py

Paste the HEX string when prompted and press Enter.

python micro_packet_decoder.py AB10003E621D0001013836313335323036343035...

python micro_packet_decoder.py --json AB10003E621D0001013836313335323036343035...

## 15.2 CRC self-test

python micro_packet_decoder.py --crc-self-test
CRC-16/XMODEM('123456789') = 0x31C3

## 15.3 What the decoder validates

- Header and Property constants.

- Length field as uint16 big-endian against actual packet size.

- CRC over Payload using the supplied algorithm.

- Sequence ID as uint16 big-endian and Command placement.

- IMEI as exactly 15 ASCII digits.

- Timestamp as exactly 8 bytes, uint64 big-endian Unix milliseconds, with UTC conversion and zero-as-unavailable handling.

- Heartbeat opcode-dependent locationData length plus the mandatory trustedDeviceCount and four fixed six-byte trusted-device slots.

- Latitude/longitude signed decoding and geographic bounds.

- Unexpected trailing bytes or truncated fields.

- Likely shifted legacy packets by searching nearby for known commands and 15-digit IMEI runs.

## 15.4 Automated tests

python test_micro_packet_decoder.py

The Version 8 decoder tests should verify the 70-byte beacon/trusted and 76-byte GPS heartbeat vectors, the 48-byte location vector, command-0x02 positional configuration samples, CRC-16/XMODEM, big-endian Length/CRC/Sequence ID, mandatory trusted-device slot padding, count/value/trailing-byte rejection, and 45.421500 / -75.697200 coordinate decoding.

# 16. Timestamp specification and simulation

Canonical rule: Every heartbeat and LTE-M location timestamp is an unsigned 64-bit integer in big-endian byte order containing Unix time in milliseconds since 1970-01-01T00:00:00Z. All times are UTC. The value 0 means that no valid timestamp is available. Version 8 retains the same big-endian timestamp rule as Version 7.

| Property | Version 7 rule | Reason |
| --- | --- | --- |
| Length | 8 bytes / uint64 | Prevents the previous 8-versus-9-byte field shift. |
| Byte order | Big-endian | Matches the other multi-byte payload numeric fields. |
| Epoch | 1970-01-01T00:00:00Z | Standard Unix epoch. |
| Unit | Milliseconds | Adequate ordering precision and broad backend compatibility. |
| Timezone | UTC | Avoids local-time and daylight-saving ambiguity. |
| Zero value | Unavailable / invalid time | Provides an explicit no-time sentinel. |
| Heartbeat meaning | Time the represented device state was sampled | Not merely the TCP send time. |
| Location meaning | Time the transmitted GNSS coordinates were fixed | May be older than the packet send time. |

## 16.1 Default deterministic timestamp

The fixed known-good samples use:

| Representation | Value |
| --- | --- |
| ISO-8601 UTC | 2026-07-21T13:30:00.000Z |
| Unix milliseconds | 1784640600000 |
| uint64 big-endian bytes | 00 00 01 9F 84 DE 77 C0 |

## 16.2 Simulator time modes

| Mode | Behaviour | Primary use |
| --- | --- | --- |
| fixed | Every dynamically generated packet uses the configured timestamp. | Repeatable packet/CRC and parser tests. |
| running | Configured Unix time advances using Zephyr board uptime. | Realistic increasing timestamps without requiring a real RTC. |
| step | Timestamp advances by a configured number of milliseconds after each generated packet. | Fast deterministic ordering and gap tests. |

## 16.3 Timestamp CLI commands

| Command | Effect | Example |
| --- | --- | --- |
| set timestamp <unix_ms> | Set the base UTC timestamp and reset the uptime anchor. | set timestamp 1784640600000 |
| set time_mode fixed¦running¦step | Select fixed, uptime-running, or per-packet step behaviour. | set time_mode running |
| set time_step <milliseconds> | Set the increment used by step mode; must be greater than zero. | set time_step 5000 |
| set fixage <seconds> | Make location packet GNSS-fix time older than simulated current time. Heartbeat time is unaffected. | set fixage 120 |
| status | Show current simulated Unix ms, mode, step, and GNSS fix age. | status |

Example - fixed time:
set timestamp 1784640600000
set time_mode fixed
send hb
send loc

Example - running time with a two-minute-old GNSS fix:
set timestamp 1784640600000
set time_mode running
set fixage 120
send hb
send loc

Example - deterministic five-second steps:
set timestamp 1784640600000
set time_mode step
set time_step 5000
send hb
send hb
send loc

## 16.4 Converting human-readable time

The firmware accepts Unix milliseconds to avoid timezone and text-parsing ambiguity. The decoder provides conversion commands:

python micro_packet_decoder.py --timestamp-to-unix-ms 2026-07-21T13:30:00Z

python micro_packet_decoder.py --unix-ms-to-timestamp 1784640600000

The first command prints the Unix-millisecond value, the eight big-endian packet bytes, and the exact simulator command to enter.

## 16.5 Timestamp test coverage

| Automated test | What it proves |
| --- | --- |
| ISO-to-Unix and Unix-to-ISO conversion | The default timestamp round-trips to 2026-07-21T13:30:00.000Z. |
| Big-endian byte vector | 1784640600000 encodes as 00 00 01 9F 84 DE 77 C0. |
| Known-good packet timestamps | All three fixed vectors decode to the same canonical UTC timestamp. |
| Zero timestamp | Value 0 decodes as unavailable while retaining a valid packet CRC. |
| GNSS fix-age vector | A 120-second-old fix decodes to 2026-07-21T13:28:00.000Z. |
| CRC, length, and protocol endianness regression tests | Known vectors require big-endian Length, CRC, Sequence ID, timestamp, and payload numeric fields; changing the timestamp still requires a matching CRC while packet length remains unchanged. |

# 17. Why Version 8 retains one byte order

The prior Version 6 format mixed little-endian header/control fields with big-endian payload fields. Version 7 removed that ambiguity by standardizing every multi-byte integer as big-endian/network byte order, and Version 8 retains that wire rule. Version 6 traffic must not be parsed using Version 8 assumptions.

| Field group | Version 8 byte order | Reason |
| --- | --- | --- |
| Length, CRC, Sequence ID | Big-endian | One consistent network-order rule; removes header-specific exceptions. |
| Timestamp, lastUpdate, coordinates, accuracy, speed | Big-endian | Network byte order for every multi-byte payload value. |
| Single-byte values | Not applicable | A one-byte value has no endianness. |
| Version 8 retains the Version 7 decision: every multi-byte integer uses big-endian/network byte order. Do not deploy this against a Version 6 parser. Keep simulator, server, decoder, tests, and receiving parser aligned. |  |  |

# 18. TCP server framing and the leading AB

| Simulator mode | First bytes on TCP | What a raw HEX logger prints | Meaning |
| --- | --- | --- | --- |
| binary | AB 10 00 3E ... | AB10003E... | The first network byte is the actual header byte 0xAB. |
| ascii_hex | 41 42 31 30 30 30 33 45 ... | 4142313030303345... | The network contains text characters "AB10003E...". 0x41 is A and 0x42 is B; there is no binary 0xAB byte. |

ASCII-HEX wire bytes: 41 42 31 30 ...  -> text "AB10..."
Binary wire bytes:    AB 10 00 3E ...  -> actual packet header 0xAB

## 18.3 Recommended integration mode

Use binary mode when testing with another team's parser:

mode binary
send sample_hb_beacon
send hb
send loc

Use ascii_hex only when a person needs readable packet text in a simple terminal or log. A production-style receiver should normally consume actual binary packet bytes.

## 18.4 Running the updated server

sudo cp micro_tcp_server.py /root/
sudo cp micro_packet_decoder.py /root/
sudo chmod +x /root/micro_tcp_server.py
sudo python3 /root/micro_tcp_server.py

Keep both Python files in the same directory. The server can frame packets without the decoder, but placing the decoder beside it enables detailed validation in the log.

## 18.5 Server response modes

| File content | Response |
| --- | --- |
| AUTO | Valid/no update -> OK; invalid -> ERROR; pending update -> SUP plus binary packet |
| ERROR | Packet rejected; settings remain unchanged |
| SUP | Settings update follows as one complete Version 8 command-0x02 full-replacement binary packet |
| FWUP | Recognized and logged; firmware-image handling deferred |

## 18.6 Server automated tests

python test_micro_tcp_server.py

The tests cover binary and ASCII-HEX packets split across recv calls, two packets combined in one recv call, mixed modes on one connection, leading noise, big-endian Length framing, and the difference between ASCII bytes 41 42 and binary header byte AB.

# 19. Extended heartbeat trusted-device registry

Every Version 8 canonical heartbeat retains the opcode-selected locationData and then appends the complete configured trusted-device registry. The registry describes which trusted devices are configured; it does not indicate which device is currently nearby. Static beacon identifiers remain separate. Short Version 7 heartbeats without this block are legacy/non-canonical for Version 8 integration.

| Field | Size | Rule |
| --- | --- | --- |
| trustedDeviceCount | 1 byte | 0 to 4 |
| trustedDeviceSlot1..4 | 4 x 6 bytes | Configured entries first in deterministic order; unused slots are six zero bytes |
| Canonical requirement | 25 bytes | Mandatory in every Version 8 heartbeat and included in the payload-only CRC |
| Heartbeat | Total bytes | Length |
| Extended beacon/trusted | 70 | 00 3E |
| Extended GPS | 76 | 00 44 |

## Known-good extended beacon heartbeat (one configured trusted device):

AB10003E621D0001013836313335323036343035303738370000019F84DE77C0010107FF0101010FAC91003B9101AABBCCDDEE01000000000000000000000000000000000000

A heartbeat can identify one trusted Bluetooth device that is currently detected through the existing fields:

Opcode:       0xA0

locationData: 6-byte trusted-device identity/address

The mandatory configured trusted-device block is separate from the currently detected locationData:

trustedDeviceCount

trustedDeviceSlot1

trustedDeviceSlot2

trustedDeviceSlot3

trustedDeviceSlot4

Those four slots report which trusted devices are configured on the tracker.

## 19.1 Device state machine and independent timers

When runtime simulation is enabled, local context is evaluated in a fixed priority order: configured detected beacon -> configured detected trusted device -> GNSS safe-zone evaluation. The resulting heartbeat opcodes are 0x01 Beacon, 0xA0 Trusted Device, 0x0A GPS Safe Zone, and 0x10 GPS + LTE Outside.

| Timer / setting | Responsibility | Packet behavior |
| --- | --- | --- |
| bleCheckIntervalSeconds | Reevaluate local BLE context; escalate to GNSS only when neither configured BLE identity qualifies. | Does not itself send a heartbeat or LTE location. |
| heartbeatIntervalSeconds | Contact the backend in every state using the latest known state. | Sends command 0x01. Backend returns OK or SUP plus matching command 0x02. |
| LTEupdateIntervalSeconds | Report fresh location while state is GPS_LTE_OUTSIDE. | Sends standalone command 0x10 on outside entry and periodically while still outside. |

The simulator starts in configuration-only mode. `simulation on` enables runtime schedules and requests an immediate state evaluation; `simulation off` stops heartbeat, BLE-check, and outside-LTE schedules without changing persistent configuration.

Heartbeat expiry never forces BLE or GNSS reevaluation. Conversely, a successfully applied full-replacement configuration requests immediate state reevaluation so changed safe zones, beacons, or trusted-device lists take effect without waiting for the next BLE interval.

# 20. Configuration update: command 0x02

Version 8 replaces the former command-0x20 typed-TLV settings format. Command 0x02 uses the unchanged big-endian packet envelope but carries one positional full-replacement configuration payload. The entire packet must validate before persistent configuration changes.

A command-0x02 packet always represents the complete configuration: target IMEI, Update ID, GPS safe zones, fixed beacons, trusted devices, heartbeat interval, LTE update interval, BLE check interval, and SendingUpdate. A zero list count clears that list; omitted settings are not a partial-update concept in Version 8.

| Order | Field | Encoding / validation |
| --- | --- | --- |
| 1 | Target IMEI | 15 ASCII decimal digits; must match the receiving device |
| 2 | Update ID | uint16 big-endian; stored after successful application |
| 3 | gpsSafeZoneCount | uint8 0..4; zero clears the safe-zone list |
| 4 | safeZone[i] | int32 latitude e6 + int32 longitude e6 + uint16 radius metres; repeated count times |
| 5 | beaconCount / beacon[i] | uint8 0..4 followed by six-byte fixed beacon identifiers; zero clears the list |
| 6 | trustedDeviceCount / trustedDevice[i] | uint8 0..4 followed by six-byte trusted identifiers; zero clears the list |
| 7 | heartbeatIntervalSeconds | uint16 BE, 1..65535; one base heartbeat timer |
| 8 | LTEupdateIntervalSeconds | uint16 BE, 1..65535; active only while GPS_LTE_OUTSIDE |
| 9 | bleCheckIntervalSeconds | uint16 BE, 1..65535; same two-byte position formerly named sleepIntervalSeconds |
| 10 | SendingUpdate | one byte: 00 or FF; firmware-image transfer is outside this release |

## Canonical full-replacement example (Sample 1)

AB100032EF0E00010238363133353230363430353037383700010102B513BCFB7CF3D00096010FAC91003B9101AABBCCDDEE01003C03FF01E000

## Compatibility note

Command 0x20 / schema / typed-TLV serializers are obsolete for Version 8. A Version 8-compatible local or server-side convenience command may still accept one requested override, but it must construct and validate a complete positional command-0x02 configuration before transmission or application.

Payload bytes = 27 + (10 × gpsSafeZoneCount) + (6 × beaconCount) + (6 × trustedDeviceCount). Length = 1 + Payload; total packet bytes = 8 + Length.

Configuration is atomic: bad CRC, Length, target IMEI, count, field value, truncated payload, invalid SendingUpdate flag, or trailing bytes leave the previous persistent configuration unchanged.

Canonical terminology is bleCheckIntervalSeconds. The local CLI may retain sleep_interval_seconds only as a legacy alias for ble_check_interval_seconds; the wire position and two-byte width are unchanged.

## Device serial commands

- Inspect the current configuration

config show

Display the result of the most recent attempted update:

config last

- Change one value through the Version 8 full-replacement convenience path

The local generate/set syntax takes one override but serializes the complete active configuration plus that override as command 0x02:

config generate heartbeat_interval_seconds 60

Generate, validate, apply, and persist it:

config set heartbeat_interval_seconds 60

Other interval examples:

config set heartbeat_interval_seconds 120

config set lte_update_interval_seconds 480

config set ble_check_interval_seconds 300

- Update static beacons (local convenience; generated wire packet is still a full replacement)

One beacon:

config set beacon_list 142525252525

Two beacons:

config set beacon_list 1425252525250FAC91003B92

Four beacons:

config set beacon_list 1425252525250FAC91003B92AB12C5D6E9A5D0A556B54856

Each beacon is exactly six bytes, represented by 12 HEX characters. The values are concatenated without separators:

142525252525 | 0FAC91003B92

The beacon list normally originates from the server and is stored on the device.

- Update trusted-device identifiers (local convenience; generated wire packet is still a full replacement)

One trusted device:

config set trusted_device_list 123456789123

Two trusted devices:

config set trusted_device_list 123456789123AABBCCDDEE02

Four trusted devices:

config set trusted_device_list 123456789123AABBCCDDEE02AB12C5D6E9A5C354DE15A6B5

Version 8 permits the configured trusted-device list to be replaced by a validated server update. In regular operation every heartbeat reports the complete configured registry, while opcode 0xA0/locationData identifies a trusted device that is currently detected.

The implementation treats these as six-byte non-secret development identifiers. It does not transmit IRKs, bonding keys, or other Bluetooth secrets.

- Update safe zones

Each safe-zone record is:

Latitude:  4-byte signed int32, big-endian, degrees × 1,000,000

Longitude: 4-byte signed int32, big-endian, degrees × 1,000,000

Radius:    2-byte uint16, big-endian, metres

Example for:

Latitude:  45.421500

Longitude: -75.697200

Radius:    150 metres

Encoded record:

02B513BC FB7CF3D0 0096

Command:

config set safe_zones 02B513BCFB7CF3D00096

Multiple safe-zone records are concatenated in the local convenience value. The Version 8 packet builder inserts gpsSafeZoneCount and serializes each complete 10-byte record in order.

Version 8 uses degrees × 1,000,000 for configuration safe-zone coordinates, matching heartbeat and LTE-M location coordinates.

- Apply an already generated Version 8 packet

config apply AB100032EF0E00010238363133353230363430353037383700010102B513BCFB7CF3D00096010FAC91003B9101AABBCCDDEE01003C03FF01E000

This canonical sample targets IMEI 861352064050787 with Update ID 1 and fully replaces the configuration with one safe zone, one fixed beacon, one trusted device, heartbeatIntervalSeconds=60, LTEupdateIntervalSeconds=1023, bleCheckIntervalSeconds=480, and SendingUpdate=0x00.

Restore defaults:

config reset confirm

This restores the documented persistent defaults through the same complete command-0x02 generation, validation, and atomic application path.

# 21. Server responses and settings application

| Response | Simulator behavior |
| --- | --- |
| OK | Log success and continue normal operation. |
| ERROR or ERROR:<code> | Preserve and log the response; do not alter settings. |
| SUP | Buffer the following complete binary Version 8 command-0x02 packet across arbitrary TCP reads; validate the entire positional payload, apply atomically, persist, reschedule affected timers, and request immediate state reevaluation. |
| FWUP | Log as deferred; do not request or install firmware content. |

The direct LTE receive path and the Windows relay return path feed the same response parser. A token may be split across reads, SUP and the packet may arrive together, or the packet may be fragmented. Invalid, truncated, wrong-IMEI, bad-count, out-of-range, invalid-flag, trailing-byte, Length, or CRC-failed updates leave the previous configuration unchanged.

# 22. Decoder and test additions

The Version 8 decoder/tests cover mandatory-registry heartbeats, command-0x02 positional full-replacement configuration, zero-count list clearing, wrong IMEI/count/value/flag/trailing-byte rejection, state-machine priority, independent BLE/heartbeat/LTE timers, fragmented responses, AUTO mode, IMEI isolation, and C/Python constant agreement.