DOCUMENT 3 OF 3

Simulator TCP Server and Command Guide

DigitalOcean deployment, Version 8 canonical state-machine/settings compatibility, retained big-endian stream framing, mandatory trusted-device heartbeat parsing, AUTO full-replacement updates, packet validation, timestamp testing, and server-to-device responses

| Version | 8 — State-machine/settings compatibility release (Version 7 big-endian envelope retained) |  |
| --- | --- | --- |
| Protocol | All multi-byte fields big-endian; CRC-16/XMODEM; Unix-millisecond timestamps; commands 0x01 heartbeat, 0x02 configuration, 0x10 location |  |
| Primary endpoint | 137.184.163.176:5000 |  |
| Project standard | Use nRF Connect SDK v3.4.x with the matching v3.4 toolchain |  |
| Compatibility warning: Version 8 replaces command-0x20 typed-TLV settings and requires the configured trusted-device registry in every heartbeat. The nRF simulator, TCP server, decoder, update tool, tests, documentation, and receiving parser must use compatible Version 8 semantics. |  |

Operational document — do not store passwords, SoftSIM profiles, or private keys in Git or shared documentation.

# Quick-start deployment checklist

|  | Verification item |  |
| --- | --- | --- |
| ☐ | Download or clone the Version 8-compatible files from GitHub. |  |
| ☐ | Upload the server, decoder, and both test files to a staging directory on the DigitalOcean server. |  |
| ☐ | Back up the currently deployed server and decoder. |  |
| ☐ | Run Python syntax checks and both automated test suites. |  |
| ☐ | Stop the existing micro-tcp-server service. |  |
| ☐ | Install micro_tcp_server.py and micro_packet_decoder.py together in /root. |  |
| ☐ | Restart the systemd service and confirm port 5000 is listening. |  |
| ☐ | Send known-good packets in binary mode and confirm validation: VALID in the server log. |  |
| ☐ | Test Version 8 state-machine transitions, independent timers, dynamic heartbeat/location, timestamps, and response behavior. |  |
| ☐ | Set micro_response_mode.txt to AUTO for automatic update testing. |  |
| ☐ | Queue a complete Version 8 command-0x02 full-replacement update for the intended IMEI and verify SUP plus the binary packet. |  |
| ☐ | Confirm successful settings persist after simulator reboot; invalid full-replacement updates do not overwrite them; successful changes immediately reevaluate state and reschedule affected timers. |  |
| ☐ | Review the text packet log and JSON Lines protocol trace. |  |
| Most important log check: An OK response confirms that the server framed a packet and returned a response. It does not by itself prove that the CRC or packet contents were valid. Confirm validation: VALID in the log. |  |  |

# 1. Purpose and current endpoint

This guide explains how to update and operate the TCP test server for Version 8 integration, verify retained Version 7 big-endian framing/CRC rules, validate mandatory-registry heartbeats and LTE-M locations, test direct LTE and Windows relay paths, and confirm server-to-device command-0x02 full-replacement configuration responses.

| Item | Current value |  |
| --- | --- | --- |
| Public IPv4 | 137.184.163.176 |  |
| TCP port | 5000 |  |
| SSH user | root |  |
| Systemd service | micro-tcp-server |  |
| Operational server file | /root/micro_tcp_server.py |  |
| Operational decoder file | /root/micro_packet_decoder.py |  |
| Packet log | /root/micro_tcp_packets.log |  |
| Response mode file | /root/micro_response_mode.txt |  |
| Protocol version | Version 8 — canonical state machine/settings; Version 7 all-big-endian envelope retained |  |
| Preferred wire mode | Binary |  |
| Credentials: Ask Amir for current server access details through a private channel. Credentials and SoftSIM HEX profiles must not be placed in Git, shared documents, email threads, or public messages. |  |

# 2. Version 8 compatibility summary

Version 8 retains the corrected Version 7 big-endian packet envelope, CRC, timestamps, and TCP stream-framing rules, while making the trusted-device heartbeat registry mandatory, adding a canonical local state machine with independent timers, and replacing command-0x20 typed-TLV settings with positional full-replacement command 0x02.

Endianness: Every numeric field larger than one byte is transmitted big-endian. This includes Length, CRC, Sequence ID, Timestamp, lastUpdate, Latitude, Longitude, Accuracy, and Speed.

Length: The Length field counts the one-byte Command plus the Payload. Total packet bytes are calculated as 8 + Length.

CRC: The placeholder CRC was removed. CRC-16/XMODEM is calculated over the Payload only and placed on the wire as a two-byte big-endian value.

Timestamp: Heartbeat and location timestamps are exactly eight bytes and represent unsigned Unix milliseconds in UTC. A zero timestamp means unavailable.

TCP framing: The server buffers the byte stream, waits for the complete length-defined packet, and separates multiple packets received together.

Wire modes: The server accepts raw binary packets and printable ASCII-HEX packets. Binary is the preferred integration format.

Validation: The server invokes micro_packet_decoder.py to validate length, CRC, command, IMEI, timestamp, coordinates, and other payload fields.

Automated tests: Protocol-decoder, TCP-stream, configuration, and firmware-side tests must cover Version 8 heartbeat lengths/registry slots, command-0x02 full replacement, state-machine priority, independent timers, fragmentation, AUTO delivery, and legacy rejection/diagnostics.

## 2.1 Version 8 packet prefix

| Bytes | Field | Encoding | Example |  |
| --- | --- | --- | --- | --- |
| 0 | Header | Fixed byte | AB |  |
| 1 | Property | Fixed byte | 10 |  |
| 2–3 | Length | uint16 big-endian | 00 3E = 62 (canonical beacon/trusted heartbeat) |  |
| 4–5 | CRC | uint16 big-endian | 62 1D (canonical beacon heartbeat example) |  |
| 6–7 | Sequence ID | uint16 big-endian | 00 01 |  |
| 8 | Command | 1 byte | 01 heartbeat / 02 configuration update / 10 location |  |
| 9+ | Payload | Command-specific | Command-specific; V8 heartbeat includes mandatory configured trusted-device registry |  |
| Total packet bytes = 8 + Length CRC input bytes = Payload only, beginning at byte offset 9 |  |  |  |

# 3. Transport paths

The server receives the same Micro application packet through either transport. The Windows relay removes its internal serial control markers before forwarding the packet to the TCP server.

| Mode | Path to server | Return path |  |
| --- | --- | --- | --- |
| Direct LTE | nRF9151 DK → Onomondo SoftSIM/LTE-M → TCP server | Server → LTE TCP socket → board |  |
| Windows relay | nRF9151 DK → USB serial → Windows relay → Wi-Fi → TCP server | Server → Windows TCP socket → USB serial → board |  |
| Scope: Relay mode validates packet generation, TCP parsing, server responses, and return delivery. It does not validate LTE-M registration, cellular latency, PSM/RRC behavior, roaming, or power consumption. |  |  |

# 4. Binary versus ASCII-HEX and the leading AB

The text “AB” can represent two different things on a TCP connection. This distinction explains why a logger may appear to see the header while another parser reports that byte 0 is not 0xAB.

| Mode | What the simulator sends | First raw TCP bytes | Parser sees header byte 0xAB? |
| --- | --- | --- | --- |
| Binary | Actual packet bytes | AB 10 00 3E … | Yes |
| ASCII-HEX | Printable text “AB10003E…” | 41 42 31 30 30 30 33 45 … | No; 41 is ASCII “A” |

The Version 4 server only printed each recv() chunk. In ASCII-HEX mode, safe_ascii() displayed the characters AB even though the actual bytes were 41 42. The Version 7 server logs both raw_hex and ascii, then converts ASCII-HEX into canonical binary bytes before validation.

Integration recommendation: Use mode binary when testing another team’s real packet parser. In that mode, the first actual TCP byte is 0xAB. Use ASCII-HEX only for readable debugging.

# 5. Required server files

| File | Purpose | Operational requirement |
| --- | --- | --- |
| micro_tcp_server.py | Persistent TCP server, stream buffering, packet framing, logging, responses | Required |
| micro_packet_decoder.py | Protocol field decoding, length and CRC validation | Required for validation; keep beside server file |
| test_micro_tcp_server.py | Offline TCP stream-framing regression tests | Run before deployment |
| test_micro_packet_decoder.py | Offline Version 8 protocol/decoder regression tests, including mandatory heartbeat registry and positional configuration | Run before deployment |
| micro_protocol.py | Shared Version 8 constants, heartbeat/configuration encoders, positional full-replacement validation, and test vectors | Required; keep beside server and decoder |
| micro_pending_store.py | Atomic pending-update JSON store by IMEI | Required for AUTO settings delivery |
| micro_update_tool.py | Generate/queue/inspect complete Version 8 command-0x02 configurations by target IMEI; legacy command-0x20/TLV output is not compatible | Administrative development utility |
| micro_response_parser.py | Fragmented response parser model and tests | Test/support utility |
| micro_pending_updates.json | Pending update records and sent_unconfirmed status | Generated; do not edit while the tool/server is writing |
| micro_protocol_trace.jsonl | Structured connection and packet trace | Generated local diagnostic log |

The operational server imports decode_packet from micro_packet_decoder.py. If the decoder is missing or cannot be imported, the server can still frame and log packets, but the log reports validation: decoder unavailable.

# 6. Safe update workflow

## 6.1 Upload from Windows

Windows PowerShell

$Repo = Join-Path $env:USERPROFILE "NRF9151-Simulator-Guides" git -C $Repo pull $ServerFiles = Join-Path $Repo "Micro_Simulator_Dual_Transport_Complete_v6\tools\protocol_test" scp "$ServerFiles\*.py" root@137.184.163.176:/root/micro_server_staging/ scp "$ServerFiles\micro_response_mode.txt.example" root@137.184.163.176:/root/micro_server_staging/

Windows PowerShell

ssh root@137.184.163.176

## 6.2 Create a staging directory

Remote Ubuntu shell

ssh root@137.184.163.176 mkdir -p /root/micro_server_staging cd /root/micro_server_staging ls -la

## 6.3 Back up the deployed files

Remote Ubuntu shell

BACKUP="/root/micro_backup_$(date -u +%Y%m%dT%H%M%SZ)" mkdir -p "$BACKUP" cp -a /root/micro_tcp_server.py "$BACKUP/" 2>/dev/null ¦¦ true cp -a /root/micro_packet_decoder.py "$BACKUP/" 2>/dev/null ¦¦ true cp -a /root/micro_tcp_packets.log "$BACKUP/" 2>/dev/null ¦¦ true cp -a /root/micro_response_mode.txt "$BACKUP/" 2>/dev/null ¦¦ true echo "Backup: $BACKUP"

## 6.4 Run syntax checks and automated tests

Remote Ubuntu shell

| python3 -m py_compile micro_tcp_server.py micro_packet_decoder.py test_micro_tcp_server.py test_micro_packet_decoder.py python3 -m unittest -v test_micro_packet_decoder.py python3 -m unittest -v test_micro_tcp_server.py |  |
| --- | --- |
| Deployment gate: Do not replace the running server if py_compile fails or either test suite reports FAILED or ERROR. |

## 6.5 Stop, install, and restart

Remote Ubuntu shell

systemctl stop micro-tcp-server cp micro_tcp_server.py /root/micro_tcp_server.py cp micro_packet_decoder.py /root/micro_packet_decoder.py python3 -m py_compile /root/micro_tcp_server.py /root/micro_packet_decoder.py systemctl restart micro-tcp-server systemctl status micro-tcp-server --no-pager ss -ltnp ¦ grep ":5000"

## 6.6 Follow new service output

Remote Ubuntu shell

journalctl -u micro-tcp-server -f -n 0

Keep the journal window open while sending packets from the board or a local test client.

# 7. Service, log, network, and firewall commands

## 7.1 Service management

| Command | Use / expected result |
| --- | --- |
| systemctl status micro-tcp-server --no-pager | Show whether the service is running and recent status lines. |
| systemctl start micro-tcp-server | Start the TCP server. |
| systemctl stop micro-tcp-server | Stop the TCP server. |
| systemctl restart micro-tcp-server | Restart after installing or editing Python files. |
| systemctl enable micro-tcp-server | Start automatically after server reboot. |
| systemctl daemon-reload | Reload systemd unit files after changing the service definition. |

## 7.2 Journal and packet logs

| Command | Use / expected result |
| --- | --- |
| journalctl -u micro-tcp-server -f -n 0 | Follow only new service output. Best command during live tests. |
| journalctl -u micro-tcp-server -n 100 --no-pager | Show the last 100 service lines. |
| journalctl -u micro-tcp-server --since "10 minutes ago" --no-pager | Show recent activity. |
| tail -f /root/micro_tcp_packets.log | Follow saved framed-packet records. |
| tail -n 100 /root/micro_tcp_packets.log | Show the last 100 saved records. |
| grep -E "validation:¦wire_mode:¦packet_hex:" /root/micro_tcp_packets.log ¦ tail -n 60 | Show recent framing and validation results. |

## 7.3 Network and firewall checks

| Command | Use / expected result |
| --- | --- |
| ss -ltnp ¦ grep ":5000" | Confirm that a process is listening on TCP port 5000. |
| ufw status verbose | Show firewall rules. |
| ufw allow OpenSSH | Preserve SSH access before enabling UFW. |
| ufw allow 5000/tcp | Allow the Micro test server port. |
| ufw reload | Reload firewall rules. |
| Test-NetConnection 137.184.163.176 -Port 5000 | From Windows, verify that the public endpoint is reachable. |

# 8. Packet decoder and protocol verification

micro_packet_decoder.py is a local inspection tool. It does not connect to the board or server. It accepts a pasted packet HEX string, decodes every field, and reports whether Length and CRC are valid.

## 8.1 Interactive packet decode

Windows or Linux terminal

python3 micro_packet_decoder.py

Paste the complete packet HEX string and press Enter. The decoder prints byte offsets, raw field bytes, interpreted values, errors, warnings, and an Overall result of VALID or INVALID.

## 8.2 Decode a packet supplied on the command line

python3 micro_packet_decoder.py "AB10003E621D0001013836313335323036343035303738370000019F84DE77C0010107FF0101010FAC91003B9101AABBCCDDEE01000000000000000000000000000000000000"

## 8.3 CRC and timestamp utility commands

python3 micro_packet_decoder.py --crc-self-test python3 micro_packet_decoder.py --timestamp-to-unix-ms 2026-07-21T13:30:00Z python3 micro_packet_decoder.py --unix-ms-to-timestamp 1784640600000

The CRC self-test must produce 0x31C3 for the ASCII text 123456789. The timestamp converter displays the Unix-millisecond value and the exact eight-byte big-endian representation used by the simulator.

## 8.4 Automated protocol tests

python3 -m unittest -v test_micro_packet_decoder.py

The protocol tests verify the CRC check vector, known-good packets, big-endian fields, coordinate decoding, timestamp conversion, zero timestamp behavior, GNSS fix age, corrupted length detection, and rejection of malformed legacy packets.

# 9. TCP stream-parser tests

test_micro_tcp_server.py exercises the parser without opening the public port or changing the running service.

python3 -m unittest -v test_micro_tcp_server.py

One binary packet divided across multiple recv() calls.

One ASCII-HEX packet divided across multiple recv() calls.

Two packets received in one TCP chunk.

Binary and ASCII-HEX packets on the same connection.

Leading non-packet noise before a valid header.

Big-endian Length controlling the total framed packet size.

The difference between binary byte AB and ASCII bytes 41 42.

Expected result: The test output must end with OK. These tests validate stream framing logic, not LTE coverage or the deployed systemd service.

## 9.1 Foreground server test

Stop the systemd service before starting a foreground copy, because only one process can bind port 5000.

Remote Ubuntu shell

systemctl stop micro-tcp-server cd /root python3 -u micro_tcp_server.py

Expected startup text should identify the Version 8 compatibility build, the retained all-big-endian envelope, and accepted binary/ASCII-HEX wire modes. Runtime simulation remains configuration-only until `simulation on`.

## 9.2 Local binary packet test

Second remote Ubuntu shell

python3 - <<'PY' import socket packet = bytes.fromhex( "AB10003E621D0001013836313335323036343035303738370000019F84DE77C0010107FF0101010FAC91003B" "9101AABBCCDDEE01000000000000000000000000000000000000" ) with socket.create_connection(("127.0.0.1", 5000), timeout=5) as sock: sock.sendall(packet) print(sock.recv(1024)) PY

Expected response: b'OK\n'. The server log should report wire_mode: binary, header_byte: 0xAB, and validation: VALID.

## 9.3 Local ASCII-HEX packet test

python3 - <<'PY' import socket packet_hex = ( "AB10003E621D0001013836313335323036343035303738370000019F84DE77C0010107FF0101010FAC91003B" "9101AABBCCDDEE01000000000000000000000000000000000000" ) with socket.create_connection(("127.0.0.1", 5000), timeout=5) as sock: sock.sendall(packet_hex.encode("ascii")) print(sock.recv(1024)) PY

The raw receive log begins with 4142 because those are the ASCII characters A and B. The framed packet is converted to binary and should still validate as VALID.

# 10. Systemd deployment

## 10.1 Existing service

systemctl cat micro-tcp-server.service

Confirm that the service runs /root/micro_tcp_server.py and that micro_packet_decoder.py is in the same directory. Unbuffered Python output is recommended so journal messages appear immediately.

## 10.2 Recommended service definition

[Unit] Description=Micro TCP Server Version 8 Compatibility After=network-online.target Wants=network-online.target [Service] Type=simple User=root WorkingDirectory=/root ExecStart=/usr/bin/python3 -u /root/micro_tcp_server.py Restart=on-failure RestartSec=2 [Install] WantedBy=multi-user.target

systemctl daemon-reload systemctl enable micro-tcp-server systemctl restart micro-tcp-server systemctl status micro-tcp-server --no-pager

# 11. Direct LTE and relay tests

## 11.1 Direct LTE test

nRF terminal

softsim status transport lte lte connect lte status connect 137.184.163.176 5000 mode binary send sample_hb_beacon send sample_hb_gps send sample_location

Expected server results: one accepted TCP connection, one response per complete packet, and validation: VALID for the 70-byte beacon heartbeat, 76-byte GPS heartbeat, and 48-byte LTE-M location packet.

| Packet | Total bytes | Command |
| --- | --- | --- |
| Beacon heartbeat | 70 | 0x01 |
| GPS heartbeat | 76 | 0x01 |
| LTE-M location | 48 | 0x10 |

## 11.2 Windows relay test

Windows PowerShell

$relay = Join-Path $env:USERPROFILE "onomondo-softsim-test\tools\micro_relay" powershell -ExecutionPolicy Bypass -File (Join-Path $relay "run_windows_relay.ps1") -Port COM7

Relay console / nRF terminal

transport relay connect 137.184.163.176 5000 mode binary send sample_hb_beacon send sample_hb_gps send sample_location

Expected return path: server response → Windows TCP socket → MICRO_RELAY_RX_HEX over USB serial → board decode and print.

## 11.3 Version 8 dynamic state tests

| Commands | Expected state | Server check |
| --- | --- | --- |
| simulation on preset normal send hb | Beacon state (0x01) with medium battery, not charging; configured beacon wins priority over trusted device | 70-byte heartbeat validates; trusted-device registry is present and padded |
| simulation on preset normal preset low send hb | Beacon state remains; battery becomes low | Heartbeat validates; battery field decodes as low |
| simulation on preset normal preset charging send hb | Beacon state remains; charging becomes on | Heartbeat validates; charging field decodes as charging |
| simulation on preset outside status send hb | BLE detections absent; GNSS safe-zone evaluation -> GPS_LTE_OUTSIDE (0x10) for the default outside position | 76-byte GPS heartbeat validates; standalone command 0x10 is sent on outside entry and then by LTEupdateIntervalSeconds |
| simulation on preset trusted status send hb | Trusted Device state (0xA0) when the detected trusted identity is configured and no configured beacon is detected | 70-byte heartbeat validates; locationData identifies current trusted device while registry reports all configured trusted devices |
| simulation on set beacon off set trusted off set pos 45.4215 -75.6972 25.5 0.0 status send hb | GNSS safe-zone evaluation -> GPS_SAFE_ZONE (0x0A) for the canonical sample safe-zone position | 76-byte GPS heartbeat validates; outside LTE-location timer is stopped |

## 11.4 Version 8 independent timer checks

| Test | Commands / setup | Expected result |
| --- | --- | --- |
| Configuration-only boot | reboot; simulation status; status | Runtime simulation is off; heartbeat, BLE-check, and LTE-location automatic schedules are stopped; persistent settings remain available. |
| Enable runtime | connect server; simulation on; status | BLE reevaluation is requested immediately. BLE and heartbeat schedules start; outside-LTE schedule starts only when state is GPS_LTE_OUTSIDE. |
| BLE timer independence | Set/use a short ble_check_interval_seconds; change detected beacon/trusted context. | BLE expiry only reevaluates local state. It does not itself send heartbeat or location packets. |
| Heartbeat timer independence | Use heartbeat_interval_seconds; remain in any state. | Each expiry sends command 0x01 using the latest known state. Heartbeat expiry does not trigger BLE/GNSS reevaluation. |
| Outside LTE timer | preset outside; observe entry and periodic traffic | Entry to GPS_LTE_OUTSIDE triggers one standalone command 0x10; further locations follow LTEupdateIntervalSeconds while still outside; leaving outside stops that schedule. |
| Configuration application | Apply/receive a valid command-0x02 packet that changes lists or timer values. | Configuration changes atomically; affected schedules are rescheduled and state is reevaluated immediately. Invalid packets leave the previous configuration and schedules intact. |

Timer checks assume runtime simulation is enabled and, for automatic network sends, the selected transport is connected. The three timer responsibilities must remain independent.

# 12. Timestamp simulation tests

Version 8 retains uint64 big-endian Unix-millisecond UTC timestamps. The dynamic heartbeat timestamp represents the state-sample time; the dynamic location timestamp represents the GNSS fix time. Timestamp schedules are independent from the Version 8 BLE/heartbeat/LTE state-machine timers.

| Test | Commands | Expected result |  |
| --- | --- | --- | --- |
| Fixed time | set timestamp 1784640600000 set time_mode fixed send hb send hb | Both packets decode to 2026-07-21T13:30:00.000Z; Sequence ID increases. |  |
| Step time | set timestamp 1784640600000 set time_mode step set time_step 5000 send hb three times | Timestamps increase by exactly 5 seconds per dynamic packet. |  |
| Running time | set timestamp 1784640600000 set time_mode running send hb; wait; send hb | Second timestamp advances approximately by elapsed board uptime. |  |
| Old GNSS fix | set timestamp 1784640600000 set time_mode fixed set fixage 120 send loc | GNSS fix timestamp decodes to 2026-07-21T13:28:00.000Z. |  |
| Fixed sample commands: send sample_hb_beacon, send sample_hb_gps, and send sample_location remain constant known-good vectors. Timestamp state controls affect send hb and send loc. |  |  |

# 13. Server response modes

The server reads the response mode from /root/micro_response_mode.txt for each complete framed packet.

| File value | Response | Use |
| --- | --- | --- |
| OK | OK\n | Basic acknowledgement and return-path test. |
| CONFIG_NONE | CONFIG_NONE\n | Indicate that no configuration update is required. |
| ECHO_HEX | RX_HEX:<canonical packet HEX>\n | Prove exactly which packet the server reconstructed. |
| CONFIG_UPDATE_ASCII_HEX:<HEX> | Same readable configuration-update line plus newline | Temporary development response. |
| BINARY_HEX:<HEX> | Raw bytes represented by the supplied HEX | Binary response test. |
| AUTO | OK, ERROR, or SUP plus one binary settings packet | Normal automatic development mode |
| ERROR | ERROR\n or ERROR:<code>\n | Invalid/unaccepted packet; no settings change |
| SUP | SUP\n followed immediately by complete Version 8 binary command-0x02 packet | Pending full-replacement update for received heartbeat IMEI |
| FWUP | FWUP\n | Recognized by simulator; firmware update deferred |

echo "OK" > /root/micro_response_mode.txt echo "CONFIG_NONE" > /root/micro_response_mode.txt echo "ECHO_HEX" > /root/micro_response_mode.txt

Validation behavior: The current test server responds even when the decoder marks a framed packet INVALID. This is intentional for diagnostics. Check the packet log; do not treat OK as proof of a valid CRC.

# 14. Reading the Version 8 logs

The service journal records raw TCP chunks, while /root/micro_tcp_packets.log records complete framed packets and decoder results.

Representative valid binary record

recv chunk 70 bytes ... raw_hex=AB10003E... ascii=..>... wire_mode: binary packet_length: 70 bytes packet_hex: AB10003E... header_byte: 0xAB validation: VALID sequence_id: 1 command: 0x01 (heartbeat) imei: 861352064050787 trusted_device_count: 1 trusted_device_slot1: AABBCCDDEE01 timestamp_utc: 2026-07-21T13:30:00.000Z

For ASCII-HEX, the raw chunk begins 4142 and the ascii text begins AB. The framed packet_hex still begins AB because the server converts the readable text to binary before decoding it.

An invalid packet includes validation: INVALID followed by one or more validation_error lines, such as a CRC mismatch or Length mismatch.

# 15. Troubleshooting

| Symptom | Check / corrective action |
| --- | --- |
| No new journal output | Confirm service status, port 5000 listener, endpoint, and that a new packet was actually sent. |
| Connection refused | The service is stopped or not listening. Check systemctl status and ss -ltnp. |
| Connection timeout | Check UFW, cloud firewall, address, port, and public reachability. |
| Other server cannot find AB | Confirm wire mode. Binary starts with raw AB; ASCII-HEX starts with raw 41 42. |
| Server logs AB but parser rejects byte 0 | The logger may be displaying ASCII characters. Inspect raw_hex, not only printable ascii. |
| Packet arrives in pieces | Normal TCP behavior. Use a persistent per-connection buffer and Length-based framing. |
| CRC mismatch | Confirm CRC-16/XMODEM, initial 0x0000, payload-only coverage, and big-endian CRC bytes. |
| Length mismatch | Confirm Length is uint16 big-endian and counts Command + Payload. Total bytes are 8 + Length. |
| Timestamp appears unreasonable | Confirm uint64 big-endian Unix milliseconds in UTC, not seconds or nanoseconds. |
| Decoder unavailable | Place micro_packet_decoder.py beside micro_tcp_server.py and restart the service. |
| Address already in use | Stop the existing service or foreground process before starting another server on port 5000. |
| OK received but packet invalid | Expected test-server behavior. Inspect validation: VALID or INVALID in the log. |
| LTE fails but relay works | Likely a cellular coverage, roaming, provisioning, or modem issue. Relay does not validate LTE. |
| SUP returned but no complete packet applies | Confirm the server sent SUP followed by complete binary 0xAB packet bytes, not ASCII HEX. Verify command byte 0x02, Length, CRC, target IMEI, list counts, positional field order/ranges, SendingUpdate flag, and absence of trailing bytes. A command-0x20/TLV packet is legacy and incompatible. |
| Update is sent repeatedly | AUTO sends only records with status pending. Inspect the pending store; sent updates become sent_unconfirmed and require an explicit requeue. |
| Another device receives an update | Verify the pending record key and target IMEI. AUTO matches the decoded heartbeat IMEI before delivery. |
| FWUP appears in the simulator console | Expected. Firmware-image transfer is outside this release and is safely deferred. |

# 16. Rollback and security

## 16.1 Roll back the server files

systemctl stop micro-tcp-server cp /root/micro_backup_<TIMESTAMP>/micro_tcp_server.py /root/micro_tcp_server.py cp /root/micro_backup_<TIMESTAMP>/micro_packet_decoder.py /root/micro_packet_decoder.py python3 -m py_compile /root/micro_tcp_server.py /root/micro_packet_decoder.py systemctl restart micro-tcp-server

A rollback only restores server software. If the board and receiving parser use Version 8 canonical heartbeat/configuration semantics, a legacy Version 7 server/decoder or command-0x20 update tool may remain incompatible. Keep firmware, server, decoder, tests, and administrative tooling aligned.

## 16.2 Security boundaries

Do not commit server passwords, API keys, SoftSIM profiles, private keys, or production device credentials.

# 17. Version 8 acceptance checklist

- Verification item

Both Python files compile without syntax errors.

Protocol decoder tests complete with OK.

TCP stream-parser tests complete with OK.

Systemd service is active and enabled.

Port 5000 is listening and publicly reachable where intended.

Known-good beacon heartbeat validates in binary mode.

Known-good GPS heartbeat validates in binary mode.

Known-good location packet validates in binary mode.

ASCII-HEX mode is recognized and converted correctly for debugging.

Dynamic battery/charging tests and Version 8 state transitions (Beacon -> Trusted Device -> GPS Safe Zone -> GPS/LTE Outside according to configured context) behave as documented.

Fixed, running, step, and GNSS-fix-age timestamp tests behave as documented.

Server responses return through both LTE and Windows relay paths.

The receiving team has updated its parser for the Version 8 canonical command set and mandatory heartbeat registry while retaining the Version 7 big-endian field rules.

Canonical beacon/trusted heartbeats are 70 bytes and canonical GPS heartbeats are 76 bytes; every heartbeat includes trustedDeviceCount plus four fixed six-byte trusted-device slots.

Trusted-device count and all four slots are logged and unused slots are zero-filled.

AUTO returns ERROR for invalid packets, OK when no update is pending, and SUP plus the correct IMEI-specific update.

A pending record becomes sent_unconfirmed after transmission and is not automatically retransmitted.

The update-generation/queueing utility in use is Version 8-compatible and emits complete positional command-0x02 configurations; any utility that emits command 0x20 / typed TLVs is rejected as legacy.

Structured packet/connection logs and Windows relay logs are created without credentials or IRKs.

Python tests and host C settings/state-machine tests complete successfully, including command-0x02 full replacement, zero-count clearing, immediate reevaluation, and independent-timer behavior.

The current TCP test server is for development. It does not provide application-layer authentication or encryption.

Use restricted firewall rules and a production-grade authenticated protocol before deployment to real users.

- •	Rotate and archive logs according to project data-retention requirements.

- Completion criterion: Integration testing is complete only when sender, receiver, decoder, server, update tool, tests, and documentation all agree on Version 8 canonical heartbeat/configuration semantics and the retained big-endian wire envelope.

# 18. AUTO mode and pending updates by IMEI

## 18.0 Version 8 command-0x02 payload contract

AUTO may deliver only a complete positional full-replacement payload in this order. Zero counts clear the corresponding list; all interval fields are uint16 big-endian in seconds.

| Order | Field | Rule |
| --- | --- | --- |
| 1 | Target IMEI | 15 ASCII digits; must match the heartbeat/device |
| 2 | Update ID | uint16 BE |
| 3-4 | gpsSafeZoneCount + safeZone[i] | 0..4; each zone = int32 lat_e6 + int32 lon_e6 + uint16 radius_m |
| 5 | beaconCount + beacon[i] | 0..4; six-byte identifiers |
| 6 | trustedDeviceCount + trustedDevice[i] | 0..4; six-byte identifiers |
| 7 | heartbeatIntervalSeconds | uint16 BE, 1..65535 |
| 8 | LTEupdateIntervalSeconds | uint16 BE, 1..65535; active only outside |
| 9 | bleCheckIntervalSeconds | uint16 BE, 1..65535; legacy local alias may be sleep_interval_seconds |
| 10 | SendingUpdate | 00 or FF |

Payload = 27 + (10 × safe zones) + (6 × beacons) + (6 × trusted devices); Length = 1 + Payload; total packet bytes = 8 + Length.

Set /root/micro_response_mode.txt to AUTO. The server validates each framed packet. Invalid or unknown packets receive ERROR:INVALID_PACKET. A valid heartbeat receives OK unless its IMEI has a pending update; then the server sends SUP followed immediately by one complete binary Version 8 command-0x02 full-replacement configuration packet. After successful transmission, the pending record becomes sent_unconfirmed; transmission alone is not proof that the device applied the update.

| File / status | Meaning |
| --- | --- |
| /root/micro_pending_updates.json | Atomic JSON development store keyed by target IMEI |
| pending | Eligible for one automatic transmission |
| sent_unconfirmed | Transmitted; no device-to-server application acknowledgement is defined |
| cancelled | Retained for troubleshooting but not sent |
| failed | Server recorded a send/storage error |

## 18.1 Update-generation utility

Compatibility requirement before generate/queue: the administrative tool must construct a complete positional command-0x02 configuration. A `--set` option may be retained only as convenience syntax that loads/builds the complete candidate configuration; it must not emit a partial TLV packet.

python3 micro_update_tool.py generate --imei 861352064050787 --set heartbeat_interval_seconds=60

Queue for AUTO delivery after confirming the generated packet uses command byte 0x02 and validates as a complete Version 8 configuration:

python3 micro_update_tool.py queue --imei 861352064050787 --set heartbeat_interval_seconds=60

Canonical packet sanity check: Sample 1 below is a complete Version 8 command-0x02 configuration for the same development IMEI (Update ID 1).

AB100032EF0E00010238363133353230363430353037383700010102B513BCFB7CF3D00096010FAC91003B9101AABBCCDDEE01003C03FF01E000

Administrative examples remain: list, inspect, requeue, cancel/remove. Those pending-store operations are independent of the wire payload format.

## 18.2 SUP sequence

Simulator -> server: Version 8 canonical heartbeat
Server -> simulator: SUP\n
Server -> simulator: one complete binary Version 8 command-0x02 full-replacement packet
Simulator: buffer across reads, validate the entire positional configuration, atomically apply/persist it, reschedule affected timers, immediately reevaluate state, and log the result

The TCP receiver must not assume that the response token or update packet arrives in one recv() call. The Windows relay returns all received bytes through MICRO_RELAY_RX_HEX, and the board feeds the reconstructed bytes into the same parser used by direct LTE.

# 19. Local protocol and relay logs

The server continues to write /root/micro_tcp_packets.log and also writes /root/micro_protocol_trace.jsonl. Each connection has a connection ID and records open/close time, remote endpoint, raw chunks, framed packets, IMEI, command, validation, trusted-device registry, response, pending update ID, update transmission, errors, and duration. The Windows relay writes micro_relay_trace.jsonl beside the relay script, recording the external endpoint, bytes sent and received, serial response markers, timeouts, failures, and disconnects. No traffic is forwarded to a second diagnostic server.

# 20. Updated tests

Run from the protocol_test directory:

python3 -m py_compile *.py
python3 -m unittest -v
./run_c_settings_tests.sh

The Version 8 test suite should cover 70/76-byte mandatory-registry heartbeats, zero/one/four trusted devices with zero-filled unused slots, positional command-0x02 full replacement, zero-count list clearing, wrong IMEI, bad count/value/flag/trailing bytes, CRC/Length errors, TCP fragmentation, AUTO responses, IMEI isolation, pending-state behavior, fragmented OK/ERROR/SUP/FWUP responses, state-machine priority, and independent BLE/heartbeat/LTE timers. Host C tests should exercise atomic candidate application plus immediate post-update reevaluation and timer rescheduling.